I. design basis

Load code for building structures (GB 50009-2012);

Code for seismic design of buildings (GB 50011-2010);

Code for steel structure design (GB 50017-2003);

Technical specification for steel structure of light housing with portal frame (GB 51022-2015);

Computing software information

This engineering calculation software is PKPM steel structure design software (version 10 V3.1.5- September 9, 2016).

The calculation date is 14:13:41 seconds on 17 July 2019.

3. Schematic diagram of structure calculation

FIG. 1-1 schematic diagram of structure

Structure calculation information

Structure type: portal frame light housing steel structure

Design specification: calculated according to technical specification for steel structure of light housing with portal frame (GB 51022-2015)

Structural importance coefficient: 1.00

Total number of nodes: 9

Column count: 4

Liang: 4

Bearing constraint number: 2

Total number of standard sections: 4

Calculation information of live load: unfavorable arrangement of live load is considered

Wind load calculation information: calculate wind load

Steel: Q345

Beam and column deadweight calculation information: beam and column deadweight calculation

Axial deformation of column under constant load: consider

Calculation increase coefficient of beam and column dead weight: 1.10

Beam stiffness increase factor: 1.00

Net section area and gross section area ratio of steel structure: 0.95

The overall stability of portal frame beam in plane is checked according to bending member

The program automatically determines the allowable slenderness ratio

Allowable flexural span ratio of steel beam (constant + live) : l /180

Allowable horizontal displacement of column top/column height: l /60

Seismic impact coefficient value basis: 10 anti-gauge (2010 edition)

Seismic action calculation: calculate horizontal seismic action

Calculate mode number: 3

Earthquake intensity: 7.00

Category: in-situ soil Ⅱ class

Number of additional weight nodes: 0

Designed seismic grouping: group 1

Periodic reduction coefficient :0.80

Seismic force calculation method: mode decomposition method

Structural damping ratio: 0.050

According to gb50011-2010 seismic effect enlargement coefficient :1.000

5. Basic structure information

The node coordinates

| Node number | X | Y | Node number | X | Y |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.00 | 5.00 | 2 | 18.00 | 5.00 |
| 3 | 2.30 | 7.27 | 4 | 20.30 | 7.27 |
| 5 | 0.00 | 7.50 | 6 | 18.00 | 7.50 |
| 7 | 9.00 | 8.40 | 8 | 0.00 | 0.00 |
| 9 | 18.00 | 0.00 |  |  |  |

Column number associated

| Column no. | Node Ι | Node Π | Column no. | Node Ι | Node Π |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 | 1 | 2 | 9 | 2 |
| 3 | 1 | 5 | 4 | 2 | 6 |

Beam correlation,

| Beam is no. | Node Ι | Node Π | Beam is no. | Node Ι | Node Π |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 5 | 2 | 5 | 7 |
| 3 | 6 | 4 | 4 | 7 | 6 |

Eccentric column joints (m)

| Node number | Eccentric column value | Node number | Eccentric column value | Node number | Eccentric column value |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.000 | 2 | 0.000 | 3 | 0.000 |
| 4 | 0.000 | 5 | 0.000 | 6 | 0.000 |
| 7 | 0.000 | 8 | 0.000 | 9 | 0.000 |

Standard section information

| Cross section no. | Cross section information |
| --- | --- |
| 1 | Welding combination H section: Tw B2 B1 H * * * * T1 * T2 = 400 * 250 * 250 * 6 * 10 * 10 |
| 2 | H deformation section: B2 B1 (H1 and H2) * * * Tw * T1 * T2 = (600 ~ 300) * 200 * 200 * 6 * 10 * 10 |
| 3 | Welding combination H section: Tw B2 B1 H * * * * T1 * T2 = 200 * 200 * 200 * 6 * 8 * 8 |
| 4 | H deformation section: B2 B1 (H1 and H2) * * * Tw * T1 * T2 = (300 ~ 600) * 200 * 200 * 6 * 10 * 10 |

Column layout section number, constraint information, section layout Angle

| Column no. | Standard section number | The constraint information | Section arrangement Angle |
| --- | --- | --- | --- |
| 1 | 1 | Just pick up on both ends | 0 |
| 2 | 1 | Just pick up on both ends | 0 |
| 3 | 1 | Just pick up on both ends | 0 |
| 4 | 1 | Just pick up on both ends | 0 |

Beam layout section number, constraint information

| Beam is no. | Standard section number | The constraint information |
| --- | --- | --- |
| 1 | 3 | Just pick up on both ends |
| 2 | 2 | Just pick up on both ends |
| 3 | 3 | Just pick up on both ends |
| 4 | 4 | Just pick up on both ends |

Cross section features

| Cross section no. | Xc (mm) | Yc (mm). | Ix (cm4) | Iy (cm4) | A (cm2) |
| --- | --- | --- | --- | --- | --- |
| 1 | 125.0 | 200.0 | 21760.3 | 2604.9 | 72.8 |
| 2 | 100.0 | 225.0 | 23338.7 | 1334.1 | 65.8 |
| 3 | 100.0 | 100.0 | 3262.3 | 1067.0 | 43.0 |
| 4 | 100.0 | 225.0 | 23338.7 | 1334.1 | 65.8 |

Cross section features

| Cross section no. | Ix (cm) | Iy (cm) | W1x (cm3) | W2x (cm3) | W1y (cm3) | W2y (cm3) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 17.3 | 6.0 | 1088.0 | 1088.0 | 208.4 | 208.4 |
| 2 | 18.8 | 4.5 | 1037.3 | 1037.3 | 133.4 | 133.4 |
| 3 | 8.7 | 5.0 | 326.2 | 326.2 | 106.7 | 106.7 |
| 4 | 18.8 | 4.5 | 1037.3 | 1037.3 | 133.4 | 133.4 |

Combination of load and effect

1. Load table under each single working condition

The node load

| Working condition | Node number | Bending moment | Vertical force | Horizontal force |
| --- | --- | --- | --- | --- |
| -- | -- | -- | -- | -- |

The column load

| Working condition | Column no. | The load type | The load value | Load parameter 1 | Load parameter 2 |
| --- | --- | --- | --- | --- | --- |
| Left the wind 1 | 1 | 1 | 3.30 | 0.00 | 0.00 |
| 左风1 | 2 | 1 | 3.30 | 0.00 | 0.00 |
| 左风1 | 3 | 1 | 3.30 | 0.00 | 0.00 |
| 左风1 | 4 | 1 | 3.30 | 0.00 | 0.00 |
| 1 the right wind | 1 | 1 | 9.90 | 0.00 | 0.00 |
| 右风1 | 2 | 1 | 3.30 | 0.00 | 0.00 |
| 右风1 | 3 | 1 | 3.30 | 0.00 | 0.00 |
| 右风1 | 4 | 1 | 3.30 | 0.00 | 0.00 |
| The left 2 wind | 1 | 1 | 3.30 | 0.00 | 0.00 |
| 左风2 | 2 | 1 | 3.30 | 0.00 | 0.00 |
| 左风2 | 3 | 1 | 3.30 | 0.00 | 0.00 |
| 左风2 | 4 | 1 | 3.30 | 0.00 | 0.00 |
| 2 the right wind | 1 | 1 | 9.90 | 0.00 | 0.00 |
| 右风2 | 2 | 1 | 3.30 | 0.00 | 0.00 |
| 右风2 | 3 | 1 | 3.30 | 0.00 | 0.00 |
| 右风2 | 4 | 1 | 3.30 | 0.00 | 0.00 |

Beam load

| Working condition | For several | The number of load | The load type | The load value of 1 | Load parameter 1 | The load value 2 | Load parameter 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The constant load | 1 | 1 | 1 | 1.20 | 0.00 | 0.00 | 0.00 |
| 恒荷载 | 1 | 1 | 1 | 1.20 | 0.00 | 0.00 | 0.00 |
| 恒荷载 | 1 | 1 | 1 | 1.20 | 0.00 | 0.00 | 0.00 |
| 恒荷载 | 1 | 1 | 1 | 1.20 | 0.00 | 0.00 | 0.00 |
| Live load | 1 | 1 | 1 | 1.80 | 0.00 | 0.00 | 0.00 |
| 活荷载 | 1 | 1 | 1 | 1.80 | 0.00 | 0.00 | 0.00 |
| 活荷载 | 1 | 1 | 1 | 1.80 | 0.00 | 0.00 | 0.00 |
| 活荷载 | 1 | 1 | 1 | 1.80 | 0.00 | 0.00 | 0.00 |
| Left the wind 1 | 1 | 1 | 1 | 7.92 | 0.00 | 0.00 | 0.00 |
| 左风1 | 1 | 1 | 1 | 7.92 | 0.00 | 0.00 | 0.00 |
| 左风1 | 1 | 1 | 1 | 6.60 | 0.00 | 0.00 | 0.00 |
| 左风1 | 1 | 1 | 1 | 6.60 | 0.00 | 0.00 | 0.00 |
| 1 the right wind | 1 | 1 | 1 | 6.60 | 0.00 | 0.00 | 0.00 |
| 右风1 | 1 | 1 | 1 | 6.60 | 0.00 | 0.00 | 0.00 |
| 右风1 | 1 | 1 | 1 | 3.30 | 0.00 | 0.00 | 0.00 |
| 右风1 | 1 | 1 | 1 | 6.60 | 0.00 | 0.00 | 0.00 |
| The left 2 wind | 1 | 1 | 1 | 2.64 | 0.00 | 0.00 | 0.00 |
| 左风2 | 1 | 1 | 1 | 2.64 | 0.00 | 0.00 | 0.00 |
| 左风2 | 1 | 1 | 1 | 7.92 | 0.00 | 0.00 | 0.00 |
| 左风2 | 1 | 1 | 1 | 7.92 | 0.00 | 0.00 | 0.00 |
| 2 the right wind | 1 | 1 | 1 | 7.92 | 0.00 | 0.00 | 0.00 |
| 右风2 | 1 | 1 | 1 | 7.92 | 0.00 | 0.00 | 0.00 |
| 右风2 | 1 | 1 | 1 | 2.64 | 0.00 | 0.00 | 0.00 |
| 右风2 | 1 | 1 | 1 | 2.64 | 0.00 | 0.00 | 0.00 |

Crane 1

| project | node | node |
| --- | --- | --- |
| Crane connection node number | 1 | 2 |
| Vertical load of each node generated by the maximum wheel pressure on the left (Dmax is on the left of the span) | 181.430 | 56.390 |
| Vertical load of each node generated by the maximum wheel pressure on the right (Dmax on the right) | 56.390 | 181.430 |
| Vertical load of each node generated by the maximum wheel pressure of empty car on the left (Wmax is on the left of the span) | 0.000 | 0.000 |
| Vertical load of each node generated by the maximum wheel pressure of empty car on the right side (Wmax is on the right side of the span) | 0.000 | 0.000 |
| Vertical eccentricity between vertical load of crane and joints (m) | 0.500 | 0.500 |
| Maximum horizontal force (Tmax) generated by horizontal brake force of crane at each node | 5.002 | 5.002 |
| Vertical distance between transverse horizontal load of crane and each node (m) | 0.800 | 0.800 |
| The effect adjustment coefficient considering space work and torsion effect | 1.000 |  |
| Seismic shear force and increasing coefficient of bending moment caused by crane bridge | 1.000 |  |
| Crane bridge weight | 141.221 |  |
| Reduction coefficient of combined load of single span crane | 1.000 |  |
| Reduction coefficient of combined load of two span cranes | 1.000 |  |

2. Combination of load effects

(1) basic combination of column internal forces

| The basic combination of column internal forces |  |
| --- | --- |
| (1)1.35 constant +0.98 active 1 | (2)1.35 constant +0.98 active 2 |
| (3)1.35 constant +0.98 active 3 | (4)1.35 constant +0.98 active 4 |
| (5)1.2 constant +1.4 active 1 | (6)1.2 constant +1.4 active 2 |
| (7)1.2 constant +1.4 active 3 | (8)1.2 constant +1.4 live 4 |
| (9)1.0 constant +1.4 live 1 | (10)1.0 constant +1.4 live 2 |
| (11)1.0 constant +1.4 live 3 | (12)1.0 constant +1.4 live 4 |
| 1.2 constant +1.4 left wind 1 | 1.2 constant +1.4 right wind 1 |
| 1.2 constant +1.4 left wind 2 | 1.2 constant +1.4 right wind 2 |
| 1.0 constant +1.4 left wind 1 | 1.0 constant +1.4 right wind 1 |
| 1.0 constant +1.4 left wind 2 | 1.0 constant +1.4 right wind 2 |
| (21)1.2 constant +1.4 live 1+0.84 left wind 1 | 1.2 constant +1.4 live 1+0.84 right wind 1 |
| 1.2 constant +1.4 live 1+0.84 left wind 2 | 1.2 constant +1.4 live 1+0.84 right wind 2 |
| 1.2 constant +1.4 live 2+0.84 left wind 1 | 1.2 constant +1.4 live 2+0.84 right wind 1 |
| 1.2 constant +1.4 live 2+0.84 left wind 2 | 1.2 constant +1.4 live 2+0.84 right wind 2 |
| 1.2 constant +1.4 live 3+0.84 left wind 1 | 1.2 constant +1.4 live 3+0.84 right wind 1 |
| 1.2 constant +1.4 live 3+0.84 left wind 2 | 1.2 constant +1.4 live 3+0.84 right wind 2 |
| (33)1.2 constant +1.4 live 4+0.84 left wind 1 | 1.2 constant +1.4 live 4+0.84 right wind 1 |
| 1.2 constant +1.4 live 4+0.84 left wind 2 | 1.2 constant +1.4 live 4+0.84 right wind 2 |
| 1.0 constant +1.4 live 1+0.84 left wind 1 | 1.0 constant +1.4 live 1+0.84 right wind 1 |
| 1.0 constant +1.4 live 1+0.84 left wind 2 | 1.0 constant +1.4 live 1+0.84 right wind 2 |
| 1.0 constant +1.4 live 2+0.84 left wind 1 | 1.0 constant +1.4 live 2+0.84 right wind 1 |
| 1.0 constant +1.4 live 2+0.84 left wind 2 | 1.0 constant +1.4 live 2+0.84 right wind 2 |
| 1.0 constant +1.4 live 3+0.84 left wind 1 | 1.0 constant +1.4 live 3+0.84 right wind 1 |
| 1.0 constant +1.4 live 3+0.84 left wind 2 | 1.0 constant +1.4 live 3+0.84 right wind 2 |
| 1.0 constant +1.4 live 4+0.84 left wind 1 | 1.0 constant +1.4 live 4+0.84 right wind 1 |
| 1.0 constant +1.4 live 4+0.84 left wind 2 | 1.0 constant +1.4 live 4+0.84 right wind 2 |
| 1.2 constant +0.98 live 1+1.4 left wind 1 | 1.2 constant +0.98 live 1+1.4 right wind 1 |
| 1.2 constant +0.98 live 1+1.4 left wind 2 | 1.2 constant +0.98 live 1+1.4 right wind 2 |
| 1.2 constant +0.98 live 2+1.4 left wind 1 | 1.2 constant +0.98 live 2+1.4 right wind 1 |
| 1.2 constant +0.98 live 2+1.4 left wind 2 | 1.2 constant +0.98 live 2+1.4 right wind 2 |
| 1.2 constant +0.98 live 3+1.4 left wind 1 | 1.2 constant +0.98 live 3+1.4 right wind 1 |
| 1.2 constant +0.98 live 3+1.4 left wind 2 | 1.2 constant +0.98 live 3+1.4 right wind 2 |
| 1.2 constant +0.98 live 4+1.4 left wind 1 | 1.2 constant +0.98 live 4+1.4 right wind 1 |
| 1.2 constant +0.98 live 4+1.4 left wind 2 | 1.2 constant +0.98 live 4+1.4 right wind 2 |
| 1.0 constant +0.98 live 1+1.4 left wind 1 | 1.0 constant +0.98 live 1+1.4 right wind 1 |
| 1.0 constant +0.98 live 1+1.4 left wind 2 | 1.0 constant +0.98 live 1+1.4 right wind 2 |
| 1.0 constant +0.98 live 2+1.4 left wind 1 | 1.0 constant +0.98 live 2+1.4 right wind 1 |
| 1.0 constant +0.98 live 2+1.4 left wind 2 | 1.0 constant +0.98 live 2+1.4 right wind 2 |
| 1.0 constant +0.98 live 3+1.4 left wind 1 | 1.0 constant +0.98 live 3+1.4 right wind 1 |
| 1.0 constant +0.98 live 3+1.4 left wind 2 | 1.0 constant +0.98 live 3+1.4 right wind 2 |
| 1.0 constant +0.98 live 4+1.4 left wind 1 | 1.0 constant +0.98 live 4+1.4 right wind 1 |
| 1.0 constant +0.98 live 4+1.4 left wind 2 | 1.0 constant +0.98 live 4+1.4 right wind 2 |
| 1.2 constant +0.6 live 1+1.3 left earthquake | 1.2 constant +0.6 live 1+1.3 right earthquake |
| 1.2 constant +0.6 live 2+1.3 left earthquake | 1.2 constant +0.6 live 2+1.3 right earthquake |
| 1.2 constant +0.6 live 3+1.3 left earthquake | 1.2 constant +0.6 live 3+1.3 right earthquake |
| 1.2 constant +0.6 live 4+1.3 left earthquake | 1.2 constant +0.6 live 4+1.3 right earthquake |
| 1.0 constant +0.5 live 1+1.3 left earthquake | 1.0 constant +0.5 live 1+1.3 right earthquake |
| 1.0 constant +0.5 live 2+1.3 left earthquake | 1.0 constant +0.5 live 2+1.3 right earthquake |
| 1.0 constant +0.5 live 3+1.3 left earthquake | 1.0 constant +0.5 live 3+1.3 right earthquake |
| 1.0 constant +0.5 live 4+1.3 left earthquake | 1.0 constant +0.5 live 4+1.3 right earthquake |
| 1.2 constant +1.4 hoisting 1 | 1.2 constant +1.4 crane 2 |
| 1.2 constant +1.4 crane 3 | 1.2 constant +1.4 hoisting 4 |
| 1.2 constant +1.4 hoisting 5 | 1.2 constant +1.4 crane 6 |
| 1.2 constant +1.4 crane 7 | 1.2 constant +1.4 crane 8 |
| 1.0 constant +1.4 hoisting 1 | 1.0 constant +1.4 crane 2 |
| 1.0 constant +1.4 hoisting 3 | 1.0 constant +1.4 hoisting 4 |
| 1.0 constant +1.4 crane 5 | 1.0 constant +1.4 hoisting 6 |
| 1.0 constant +1.4 crane 7 | 1.0 constant +1.4 crane 8 |
| 1.2 constant +0.98 live 1+1.4 hanging 1 | 1.2 constant +0.98 live 1+1.4 hanging 7 |
| 1.2 constant +0.98 live 2+1.4 hanging 2 | 1.2 constant +0.98 live 2+1.4 crane 8 |
| 1.2 constant +0.98 live 3+1.4 hanging 3 | 1.2 constant +0.98 live 3+1.4 hanging 4 |
| 1.2 constant +0.98 live 4+1.4 hanging 5 | 1.2 constant +0.98 live 4+1.4 crane 6 |
| 1.0 constant +0.98 live 1+1.4 hanging 1 | 1.0 constant +0.98 live 1+1.4 crane 7 |
| 1.0 constant +0.98 live 2+1.4 hanging 2 | 1.0 constant +0.98 live 2+1.4 crane 8 |
| 1.0 constant +0.98 live 3+1.4 hanging 3 | 1.0 constant +0.98 live 3+1.4 hanging 4 |
| 1.0 constant +0.98 live 4+1.4 hanging 5 | 1.0 constant +0.98 live 4+1.4 hanging 6 |
| 1.2 constant +1.4 live 1+0.98 hanging 1 | 1.2 constant +1.4 live 1+0.98 crane 7 |
| 1.2 constant +1.4 live 2+0.98 crane 2 | 1.2 constant +1.4 live 2+0.98 crane 8 |
| 1.2 constant +1.4 live 3+0.98 crane 3 | 1.2 constant +1.4 live 3+0.98 crane 4 |
| 1.2 constant +1.4 live 4+0.98 crane 5 | 1.2 constant +1.4 live 4+0.98 crane 6 |
| 1.0 constant +1.4 live 1+0.98 hanging 1 | 1.0 constant +1.4 live 1+0.98 crane 7 |
| 1.0 constant +1.4 live 2+0.98 crane 2 | 1.0 constant +1.4 live 2+0.98 crane 8 |
| 1.0 constant +1.4 live 3+0.98 crane 3 | 1.0 constant +1.4 live 3+0.98 crane 4 |
| 1.0 constant +1.4 live 4+0.98 crane 5 | 1.0 constant +1.4 live 4+0.98 crane 6 |
| 1.2 constant +0.98 live 1+0.84 left wind 1+1.4 hanging 1 | 1.2 constant +0.98 live 1+0.84 left wind 1+1.4 hanging 7 |
| 1.2 constant +0.98 live 1+0.84 right wind 1+1.4 hanging 1 | 1.2 constant +0.98 live 1+0.84 right wind 1+1.4 hanging 7 |
| 1.2 constant +0.98 live 1+0.84 left wind 2+1.4 hanging 1 | 1.2 constant +0.98 live 1+0.84 left wind 2+1.4 hanging 7 |
| 1.2 constant +0.98 live 1+0.84 right wind 2+1.4 hanging 1 | 1.2 constant +0.98 live 1+0.84 right wind 2+1.4 hanging 7 |
| 1.2 constant +0.98 live 2+0.84 left wind 1+1.4 hanging 2 | 1.2 constant +0.98 live 2+0.84 left wind 1+1.4 hanging 8 |
| 1.2 constant +0.98 live 2+0.84 right wind 1+1.4 hanging 2 | 1.2 constant +0.98 live 2+0.84 right wind 1+1.4 hanging 8 |
| 1.2 constant +0.98 live 2+0.84 left wind 2+1.4 hanging 2 | 1.2 constant +0.98 live 2+0.84 left wind 2+1.4 hanging 8 |
| 1.2 constant +0.98 live 2+0.84 right wind 2+1.4 hanging 2 | 1.2 constant +0.98 live 2+0.84 right wind 2+1.4 hanging 8 |
| 1.2 constant +0.98 live 3+0.84 left wind 1+1.4 hanging 3 | 1.2 constant +0.98 live 3+0.84 left wind 1+1.4 hanging 4 |
| 1.2 constant +0.98 live 3+0.84 right wind 1+1.4 hanging 3 | 1.2 constant +0.98 live 3+0.84 right wind 1+1.4 hanging 4 |
| 1.2 constant +0.98 live 3+0.84 left wind 2+1.4 hanging 3 | 1.2 constant +0.98 live 3+0.84 left wind 2+1.4 hanging 4 |
| 1.2 constant +0.98 live 3+0.84 right wind 2+1.4 hanging 3 | 1.2 constant +0.98 live 3+0.84 right wind 2+1.4 hanging 4 |
| 1.2 constant +0.98 live 4+0.84 left wind 1+1.4 hanging 5 | 1.2 constant +0.98 live 4+0.84 left wind 1+1.4 hanging 6 |
| 1.2 constant +0.98 live 4+0.84 right wind 1+1.4 hanging 5 | 1.2 constant +0.98 live 4+0.84 right wind 1+1.4 hanging 6 |
| 1.2 constant +0.98 live 4+0.84 left wind 2+1.4 crane 5 | 1.2 constant +0.98 live 4+0.84 left wind 2+1.4 hanging 6 |
| 1.2 constant +0.98 live 4+0.84 right wind 2+1.4 hanging 5 | 1.2 constant +0.98 live 4+0.84 right wind 2+1.4 hanging 6 |
| 1.0 constant +0.98 live 1+0.84 left wind 1+1.4 hanging 1 | 1.0 constant +0.98 live 1+0.84 left wind 1+1.4 hanging 7 |
| 1.0 constant +0.98 live 1+0.84 right wind 1+1.4 hanging 1 | 1.0 constant +0.98 live 1+0.84 right wind 1+1.4 hanging 7 |
| 1.0 constant +0.98 live 1+0.84 left wind 2+1.4 hanging 1 | 1.0 constant +0.98 live 1+0.84 left wind 2+1.4 hanging 7 |
| 1.0 constant +0.98 live 1+0.84 right wind 2+1.4 hanging 1 | 1.0 constant +0.98 live 1+0.84 right wind 2+1.4 hanging 7 |
| 1.0 constant +0.98 live 2+0.84 left wind 1+1.4 hanging 2 | 1.0 constant +0.98 live 2+0.84 left wind 1+1.4 hanging 8 |
| 1.0 constant +0.98 live 2+0.84 right wind 1+1.4 hanging 2 | 1.0 constant +0.98 live 2+0.84 right wind 1+1.4 hanging 8 |
| 1.0 constant +0.98 live 2+0.84 left wind 2+1.4 hanging 2 | 1.0 constant +0.98 live 2+0.84 left wind 2+1.4 hanging 8 |
| 1.0 constant +0.98 live 2+0.84 right wind 2+1.4 hanging 2 | 1.0 constant +0.98 live 2+0.84 right wind 2+1.4 hanging 8 |
| 1.0 constant +0.98 live 3+0.84 left wind 1+1.4 hanging 3 | 1.0 constant +0.98 live 3+0.84 left wind 1+1.4 hanging 4 |
| 1.0 constant +0.98 live 3+0.84 right wind 1+1.4 hanging 3 | 1.0 constant +0.98 live 3+0.84 right wind 1+1.4 hanging 4 |
| 1.0 constant +0.98 live 3+0.84 left wind 2+1.4 hanging 3 | 1.0 constant +0.98 live 3+0.84 left wind 2+1.4 hanging 4 |
| 1.0 constant +0.98 live 3+0.84 right wind 2+1.4 hanging 3 | 1.0 constant +0.98 live 3+0.84 right wind 2+1.4 hanging 4 |
| 1.0 constant +0.98 live 4+0.84 left wind 1+1.4 hanging 5 | 1.0 constant +0.98 live 4+0.84 left wind 1+1.4 hanging 6 |
| 1.0 constant +0.98 live 4+0.84 right wind 1+1.4 hanging 5 | 1.0 constant +0.98 live 4+0.84 right wind 1+1.4 hanging 6 |
| 1.0 constant +0.98 live 4+0.84 left wind 2+1.4 hanging 5 | 1.0 constant +0.98 live 4+0.84 left wind 2+1.4 hanging 6 |
| 1.0 constant +0.98 live 4+0.84 right wind 2+1.4 hanging 5 | (212)1.0 constant +0.98 live 4+0.84 right wind 2+1.4 hanging 6 |
| 1.2 constant +0.98 live 1+1.4 left wind 1+0.98 hanging 1 | 1.2 constant +0.98 live 1+1.4 left wind 1+0.98 crane 7 |
| 1.2 constant +0.98 live 1+1.4 right wind 1+0.98 crane 1 | 1.2 constant +0.98 live 1+1.4 right wind 1+0.98 crane 7 |
| 1.2 constant +0.98 live 1+1.4 left wind 2+0.98 crane 1 | 1.2 constant +0.98 live 1+1.4 left wind 2+0.98 crane 7 |
| 1.2 constant +0.98 live 1+1.4 right wind 2+0.98 crane 1 | 1.2 constant +0.98 live 1+1.4 right wind 2+0.98 crane 7 |
| 1.2 constant +0.98 live 2+1.4 left wind 1+0.98 crane 2 | 1.2 constant +0.98 live 2+1.4 left wind 1+0.98 crane 8 |
| 1.2 constant +0.98 live 2+1.4 right wind 1+0.98 crane 2 | 1.2 constant +0.98 live 2+1.4 right wind 1+0.98 crane 8 |
| 1.2 constant +0.98 live 2+1.4 left wind 2+0.98 crane 2 | 1.2 constant +0.98 live 2+1.4 left wind 2+0.98 crane 8 |
| 1.2 constant +0.98 live 2+1.4 right wind 2+0.98 crane 2 | 1.2 constant +0.98 live 2+1.4 right wind 2+0.98 crane 8 |
| 1.2 constant +0.98 live 3+1.4 left wind 1+0.98 crane 3 | 1.2 constant +0.98 live 3+1.4 left wind 1+0.98 crane 4 |
| 1.2 constant +0.98 live 3+1.4 right wind 1+0.98 crane 3 | 1.2 constant +0.98 live 3+1.4 right wind 1+0.98 crane 4 |
| 1.2 constant +0.98 live 3+1.4 left wind 2+0.98 crane 3 | 1.2 constant +0.98 live 3+1.4 left wind 2+0.98 crane 4 |
| 1.2 constant +0.98 live 3+1.4 right wind 2+0.98 crane 3 | 1.2 constant +0.98 live 3+1.4 right wind 2+0.98 crane 4 |
| 1.2 constant +0.98 live 4+1.4 left wind 1+0.98 crane 5 | 1.2 constant +0.98 live 4+1.4 left wind 1+0.98 crane 6 |
| 1.2 constant +0.98 live 4+1.4 right wind 1+0.98 crane 5 | 1.2 constant +0.98 live 4+1.4 right wind 1+0.98 crane 6 |
| 1.2 constant +0.98 live 4+1.4 left wind 2+0.98 crane 5 | 1.2 constant +0.98 live 4+1.4 left wind 2+0.98 crane 6 |
| 1.2 constant +0.98 live 4+1.4 right wind 2+0.98 crane 5 | 1.2 constant +0.98 live 4+1.4 right wind 2+0.98 crane 6 |
| 1.0 constant +0.98 live 1+1.4 left wind 1+0.98 hanging 1 | 1.0 constant +0.98 live 1+1.4 left wind 1+0.98 crane 7 |
| 1.0 constant +0.98 live 1+1.4 right wind 1+0.98 hanging 1 | 1.0 constant +0.98 live 1+1.4 right wind 1+0.98 hanging 7 |
| 1.0 constant +0.98 live 1+1.4 left wind 2+0.98 hanging 1 | 1.0 constant +0.98 live 1+1.4 left wind 2+0.98 crane 7 |
| 1.0 constant +0.98 live 1+1.4 right wind 2+0.98 crane 1 | 1.0 constant +0.98 live 1+1.4 right wind 2+0.98 crane 7 |
| 1.0 constant +0.98 live 2+1.4 left wind 1+0.98 crane 2 | 1.0 constant +0.98 live 2+1.4 left wind 1+0.98 crane 8 |
| 1.0 constant +0.98 live 2+1.4 right wind 1+0.98 crane 2 | 1.0 constant +0.98 live 2+1.4 right wind 1+0.98 crane 8 |
| 1.0 constant +0.98 live 2+1.4 left wind 2+0.98 crane 2 | 1.0 constant +0.98 live 2+1.4 left wind 2+0.98 crane 8 |
| 1.0 constant +0.98 live 2+1.4 right wind 2+0.98 crane 2 | 1.0 constant +0.98 live 2+1.4 right wind 2+0.98 crane 8 |
| 1.0 constant +0.98 live 3+1.4 left wind 1+0.98 crane 3 | 1.0 constant +0.98 live 3+1.4 left wind 1+0.98 crane 4 |
| 1.0 constant +0.98 live 3+1.4 right wind 1+0.98 hanging 3 | 1.0 constant +0.98 live 3+1.4 right wind 1+0.98 crane 4 |
| 1.0 constant +0.98 live 3+1.4 left wind 2+0.98 hanging 3 | 1.0 constant +0.98 live 3+1.4 left wind 2+0.98 crane 4 |
| 1.0 constant +0.98 live 3+1.4 right wind 2+0.98 crane 3 | 1.0 constant +0.98 live 3+1.4 right wind 2+0.98 crane 4 |
| 1.0 constant +0.98 live 4+1.4 left wind 1+0.98 crane 5 | 1.0 constant +0.98 live 4+1.4 left wind 1+0.98 crane 6 |
| 1.0 constant +0.98 live 4+1.4 right wind 1+0.98 crane 5 | 1.0 constant +0.98 live 4+1.4 right wind 1+0.98 crane 6 |
| 1.0 constant +0.98 live 4+1.4 left wind 2+0.98 crane 5 | 1.0 constant +0.98 live 4+1.4 left wind 2+0.98 crane 6 |
| 1.0 constant +0.98 live 4+1.4 right wind 2+0.98 crane 5 | 1.0 constant +0.98 live 4+1.4 right wind 2+0.98 crane 6 |
| 1.2 constant +1.4 live 1+0.84 left wind 1+0.98 crane 1 | 1.2 constant +1.4 live 1+0.84 left wind 1+0.98 crane 7 |
| 1.2 constant +1.4 live 1+0.84 right wind 1+0.98 crane 1 | 1.2 constant +1.4 live 1+0.84 right wind 1+0.98 crane 7 |
| 1.2 constant +1.4 live 1+0.84 left wind 2+0.98 crane 1 | 1.2 constant +1.4 live 1+0.84 left wind 2+0.98 crane 7 |
| 1.2 constant +1.4 live 1+0.84 right wind 2+0.98 crane 1 | 1.2 constant +1.4 live 1+0.84 right wind 2+0.98 crane 7 |
| 1.2 constant +1.4 live 2+0.84 left wind 1+0.98 crane 2 | 1.2 constant +1.4 live 2+0.84 left wind 1+0.98 crane 8 |
| 1.2 constant +1.4 live 2+0.84 right wind 1+0.98 crane 2 | 1.2 constant +1.4 live 2+0.84 right wind 1+0.98 crane 8 |
| 1.2 constant +1.4 live 2+0.84 left wind 2+0.98 crane 2 | 1.2 constant +1.4 live 2+0.84 left wind 2+0.98 crane 8 |
| 1.2 constant +1.4 live 2+0.84 right wind 2+0.98 crane 2 | 1.2 constant +1.4 live 2+0.84 right wind 2+0.98 crane 8 |
| 1.2 constant +1.4 live 3+0.84 left wind 1+0.98 crane 3 | 1.2 constant +1.4 live 3+0.84 left wind 1+0.98 crane 4 |
| 1.2 constant +1.4 live 3+0.84 right wind 1+0.98 crane 3 | 1.2 constant +1.4 live 3+0.84 right wind 1+0.98 crane 4 |
| 1.2 constant +1.4 live 3+0.84 left wind 2+0.98 crane 3 | 1.2 constant +1.4 live 3+0.84 left wind 2+0.98 crane 4 |
| 1.2 constant +1.4 live 3+0.84 right wind 2+0.98 crane 3 | 1.2 constant +1.4 live 3+0.84 right wind 2+0.98 crane 4 |
| 1.2 constant +1.4 live 4+0.84 left wind 1+0.98 crane 5 | 1.2 constant +1.4 live 4+0.84 left wind 1+0.98 crane 6 |
| 1.2 constant +1.4 live 4+0.84 right wind 1+0.98 crane 5 | 1.2 constant +1.4 live 4+0.84 right wind 1+0.98 crane 6 |
| 1.2 constant +1.4 live 4+0.84 left wind 2+0.98 crane 5 | 1.2 constant +1.4 live 4+0.84 left wind 2+0.98 crane 6 |
| 1.2 constant +1.4 live 4+0.84 right wind 2+0.98 crane 5 | 1.2 constant +1.4 live 4+0.84 right wind 2+0.98 crane 6 |
| 1.0 constant +1.4 live 1+0.84 left wind 1+0.98 hanging 1 | 1.0 constant +1.4 live 1+0.84 left wind 1+0.98 crane 7 |
| 1.0 constant +1.4 live 1+0.84 right wind 1+0.98 crane 1 | 1.0 constant +1.4 live 1+0.84 right wind 1+0.98 hanging 7 |
| 1.0 constant +1.4 live 1+0.84 left wind 2+0.98 crane 1 | 1.0 constant +1.4 live 1+0.84 left wind 2+0.98 crane 7 |
| 1.0 constant +1.4 live 1+0.84 right wind 2+0.98 crane 1 | 1.0 constant +1.4 live 1+0.84 right wind 2+0.98 crane 7 |
| 1.0 constant +1.4 live 2+0.84 left wind 1+0.98 crane 2 | 1.0 constant +1.4 live 2+0.84 left wind 1+0.98 crane 8 |
| 1.0 constant +1.4 live 2+0.84 right wind 1+0.98 crane 2 | 1.0 constant +1.4 live 2+0.84 right wind 1+0.98 crane 8 |
| 1.0 constant +1.4 live 2+0.84 left wind 2+0.98 crane 2 | 1.0 constant +1.4 live 2+0.84 left wind 2+0.98 crane 8 |
| 1.0 constant +1.4 live 2+0.84 right wind 2+0.98 crane 2 | 1.0 constant +1.4 live 2+0.84 right wind 2+0.98 crane 8 |
| 1.0 constant +1.4 live 3+0.84 left wind 1+0.98 crane 3 | 1.0 constant +1.4 live 3+0.84 left wind 1+0.98 crane 4 |
| 1.0 constant +1.4 live 3+0.84 right wind 1+0.98 crane 3 | 1.0 constant +1.4 live 3+0.84 right wind 1+0.98 crane 4 |
| 1.0 constant +1.4 live 3+0.84 left wind 2+0.98 crane 3 | 1.0 constant +1.4 live 3+0.84 left wind 2+0.98 crane 4 |
| 1.0 constant +1.4 live 3+0.84 right wind 2+0.98 crane 3 | 1.0 constant +1.4 live 3+0.84 right wind 2+0.98 crane 4 |
| 1.0 constant +1.4 live 4+0.84 left wind 1+0.98 crane 5 | 1.0 constant +1.4 live 4+0.84 left wind 1+0.98 crane 6 |
| 1.0 constant +1.4 live 4+0.84 right wind 1+0.98 crane 5 | 1.0 constant +1.4 live 4+0.84 right wind 1+0.98 crane 6 |
| 1.0 constant +1.4 live 4+0.84 left wind 2+0.98 crane 5 | 1.0 constant +1.4 live 4+0.84 left wind 2+0.98 crane 6 |
| 1.0 constant +1.4 live 4+0.84 right wind 2+0.98 crane 5 | 1.0 constant +1.4 live 4+0.84 right wind 2+0.98 crane 6 |
| 1.2 constant +0.6 live 1+0.6 hanging 1+1.3 left earthquake | 1.2 constant +0.6 live 1+0.6 hanging 1+1.3 right earthquake |
| 1.2 constant +0.6 live 1+0.6 hanging 7+1.3 left earthquake | 1.2 constant +0.6 live 1+0.6 hanging 7+1.3 right earthquake |
| 1.2 constant +0.6 live 2+0.6 hanging 2+1.3 left earthquake | 1.2 constant +0.6 live 2+0.6 hanging 2+1.3 right earthquake |
| 1.2 constant +0.6 live 2+0.6 hanging 8+1.3 left earthquake | 1.2 constant +0.6 live 2+0.6 hanging 8+1.3 right earthquake |
| 1.2 constant +0.6 live 3+0.6 hanging 3+1.3 left earthquake | 1.2 constant +0.6 live 3+0.6 hanging 3+1.3 right earthquake |
| 1.2 constant +0.6 live 3+0.6 hanging 4+1.3 left earthquake | 1.2 constant +0.6 live 3+0.6 hanging 4+1.3 right earthquake |
| 1.2 constant +0.6 live 4+0.6 hanging 5+1.3 left earthquake | 1.2 constant +0.6 live 4+0.6 hanging 5+1.3 right earthquake |
| 1.2 heng +0.6 live 4+0.6 hang 6+1.3 left earthquake | 1.2 constant +0.6 live 4+0.6 hanging 6+1.3 right earthquake |
| 1.0 constant +0.5 live 1+0.5 crane 1+1.3 left earthquake | 1.0 constant +0.5 live 1+0.5 crane 1+1.3 right earthquake |
| 1.0 constant +0.5 live 1+0.5 crane 7+1.3 left earthquake | 1.0 constant +0.5 live 1+0.5 crane 7+1.3 right earthquake |
| 1.0 constant +0.5 live 2+0.5 crane 2+1.3 left earthquake | 1.0 constant +0.5 live 2+0.5 crane 2+1.3 right earthquake |
| 1.0 constant +0.5 live 2+0.5 crane 8+1.3 left earthquake | 1.0 constant +0.5 live 2+0.5 crane 8+1.3 right earthquake |
| 1.0 constant +0.5 live 3+0.5 hanging 3+1.3 left earthquake | 1.0 constant +0.5 live 3+0.5 crane 3+1.3 right earthquake |
| 1.0 constant +0.5 live 3+0.5 crane 4+1.3 left earthquake | 1.0 constant +0.5 live 3+0.5 crane 4+1.3 right earthquake |
| 1.0 constant +0.5 live 4+0.5 crane 5+1.3 left earthquake | 1.0 constant +0.5 live 4+0.5 crane 5+1.3 right earthquake |
| 1.0 constant +0.5 live 4+0.5 crane 6+1.3 left earthquake | 1.0 constant +0.5 live 4+0.5 crane 6+1.3 right earthquake |

(2) combined value of beam internal force

| Beam internal force combination |  |
| --- | --- |
| (1)1.35 constant +0.98 active 1 | (2)1.35 constant +0.98 active 2 |
| (3)1.35 constant +0.98 active 3 | (4)1.35 constant +0.98 active 4 |
| (5)1.2 constant +1.4 active 1 | (6)1.2 constant +1.4 active 2 |
| (7)1.2 constant +1.4 active 3 | (8)1.2 constant +1.4 live 4 |
| (9)1.0 constant +1.4 live 1 | (10)1.0 constant +1.4 live 2 |
| (11)1.0 constant +1.4 live 3 | (12)1.0 constant +1.4 live 4 |
| 1.2 constant +1.4 left wind 1 | 1.2 constant +1.4 right wind 1 |
| 1.2 constant +1.4 left wind 2 | 1.2 constant +1.4 right wind 2 |
| 1.0 constant +1.4 left wind 1 | 1.0 constant +1.4 right wind 1 |
| 1.0 constant +1.4 left wind 2 | 1.0 constant +1.4 right wind 2 |
| (21)1.2 constant +1.4 live 1+0.84 left wind 1 | 1.2 constant +1.4 live 1+0.84 right wind 1 |
| 1.2 constant +1.4 live 1+0.84 left wind 2 | 1.2 constant +1.4 live 1+0.84 right wind 2 |
| 1.2 constant +1.4 live 2+0.84 left wind 1 | 1.2 constant +1.4 live 2+0.84 right wind 1 |
| 1.2 constant +1.4 live 2+0.84 left wind 2 | 1.2 constant +1.4 live 2+0.84 right wind 2 |
| 1.2 constant +1.4 live 3+0.84 left wind 1 | 1.2 constant +1.4 live 3+0.84 right wind 1 |
| 1.2 constant +1.4 live 3+0.84 left wind 2 | 1.2 constant +1.4 live 3+0.84 right wind 2 |
| (33)1.2 constant +1.4 live 4+0.84 left wind 1 | 1.2 constant +1.4 live 4+0.84 right wind 1 |
| 1.2 constant +1.4 live 4+0.84 left wind 2 | 1.2 constant +1.4 live 4+0.84 right wind 2 |
| 1.0 constant +1.4 live 1+0.84 left wind 1 | 1.0 constant +1.4 live 1+0.84 right wind 1 |
| 1.0 constant +1.4 live 1+0.84 left wind 2 | 1.0 constant +1.4 live 1+0.84 right wind 2 |
| 1.0 constant +1.4 live 2+0.84 left wind 1 | 1.0 constant +1.4 live 2+0.84 right wind 1 |
| 1.0 constant +1.4 live 2+0.84 left wind 2 | 1.0 constant +1.4 live 2+0.84 right wind 2 |
| 1.0 constant +1.4 live 3+0.84 left wind 1 | 1.0 constant +1.4 live 3+0.84 right wind 1 |
| 1.0 constant +1.4 live 3+0.84 left wind 2 | 1.0 constant +1.4 live 3+0.84 right wind 2 |
| 1.0 constant +1.4 live 4+0.84 left wind 1 | 1.0 constant +1.4 live 4+0.84 right wind 1 |
| 1.0 constant +1.4 live 4+0.84 left wind 2 | 1.0 constant +1.4 live 4+0.84 right wind 2 |
| 1.2 constant +0.98 live 1+1.4 left wind 1 | 1.2 constant +0.98 live 1+1.4 right wind 1 |
| 1.2 constant +0.98 live 1+1.4 left wind 2 | 1.2 constant +0.98 live 1+1.4 right wind 2 |
| 1.2 constant +0.98 live 2+1.4 left wind 1 | 1.2 constant +0.98 live 2+1.4 right wind 1 |
| 1.2 constant +0.98 live 2+1.4 left wind 2 | 1.2 constant +0.98 live 2+1.4 right wind 2 |
| 1.2 constant +0.98 live 3+1.4 left wind 1 | 1.2 constant +0.98 live 3+1.4 right wind 1 |
| 1.2 constant +0.98 live 3+1.4 left wind 2 | 1.2 constant +0.98 live 3+1.4 right wind 2 |
| 1.2 constant +0.98 live 4+1.4 left wind 1 | 1.2 constant +0.98 live 4+1.4 right wind 1 |
| 1.2 constant +0.98 live 4+1.4 left wind 2 | 1.2 constant +0.98 live 4+1.4 right wind 2 |
| 1.0 constant +0.98 live 1+1.4 left wind 1 | 1.0 constant +0.98 live 1+1.4 right wind 1 |
| 1.0 constant +0.98 live 1+1.4 left wind 2 | 1.0 constant +0.98 live 1+1.4 right wind 2 |
| 1.0 constant +0.98 live 2+1.4 left wind 1 | 1.0 constant +0.98 live 2+1.4 right wind 1 |
| 1.0 constant +0.98 live 2+1.4 left wind 2 | 1.0 constant +0.98 live 2+1.4 right wind 2 |
| 1.0 constant +0.98 live 3+1.4 left wind 1 | 1.0 constant +0.98 live 3+1.4 right wind 1 |
| 1.0 constant +0.98 live 3+1.4 left wind 2 | 1.0 constant +0.98 live 3+1.4 right wind 2 |
| 1.0 constant +0.98 live 4+1.4 left wind 1 | 1.0 constant +0.98 live 4+1.4 right wind 1 |
| 1.0 constant +0.98 live 4+1.4 left wind 2 | 1.0 constant +0.98 live 4+1.4 right wind 2 |
| 1.2 constant +0.6 live 1+1.3 left earthquake | 1.2 constant +0.6 live 1+1.3 right earthquake |
| 1.2 constant +0.6 live 2+1.3 left earthquake | 1.2 constant +0.6 live 2+1.3 right earthquake |
| 1.2 constant +0.6 live 3+1.3 left earthquake | 1.2 constant +0.6 live 3+1.3 right earthquake |
| 1.2 constant +0.6 live 4+1.3 left earthquake | 1.2 constant +0.6 live 4+1.3 right earthquake |
| 1.0 constant +0.5 live 1+1.3 left earthquake | 1.0 constant +0.5 live 1+1.3 right earthquake |
| 1.0 constant +0.5 live 2+1.3 left earthquake | 1.0 constant +0.5 live 2+1.3 right earthquake |
| 1.0 constant +0.5 live 3+1.3 left earthquake | 1.0 constant +0.5 live 3+1.3 right earthquake |
| 1.0 constant +0.5 live 4+1.3 left earthquake | 1.0 constant +0.5 live 4+1.3 right earthquake |
| 1.2 constant +1.4 hoisting 1 | 1.2 constant +1.4 crane 2 |
| 1.0 constant +1.4 hoisting 1 | 1.0 constant +1.4 crane 2 |
| 1.2 constant +0.98 live 1+1.4 hanging 1 | 1.2 constant +0.98 live 2+1.4 hanging 2 |
| 1.0 constant +0.98 live 1+1.4 hanging 1 | 1.0 constant +0.98 live 2+1.4 hanging 2 |
| 1.2 constant +1.4 live 1+0.98 hanging 1 | 1.2 constant +1.4 live 2+0.98 crane 2 |
| 1.0 constant +1.4 live 1+0.98 hanging 1 | 1.0 constant +1.4 live 2+0.98 crane 2 |
| 1.2 constant +0.98 live 1+0.84 left wind 1+1.4 hanging 1 | 1.2 constant +0.98 live 1+0.84 right wind 1+1.4 hanging 1 |
| 1.2 constant +0.98 live 1+0.84 left wind 2+1.4 hanging 1 | 1.2 constant +0.98 live 1+0.84 right wind 2+1.4 hanging 1 |
| 1.2 constant +0.98 live 2+0.84 left wind 1+1.4 hanging 2 | 1.2 constant +0.98 live 2+0.84 right wind 1+1.4 hanging 2 |
| 1.2 constant +0.98 live 2+0.84 left wind 2+1.4 hanging 2 | 1.2 constant +0.98 live 2+0.84 right wind 2+1.4 hanging 2 |
| 1.0 constant +0.98 live 1+0.84 left wind 1+1.4 hanging 1 | 1.0 constant +0.98 live 1+0.84 right wind 1+1.4 hanging 1 |
| 1.0 constant +0.98 live 1+0.84 left wind 2+1.4 hanging 1 | 1.0 constant +0.98 live 1+0.84 right wind 2+1.4 hanging 1 |
| 1.0 constant +0.98 live 2+0.84 left wind 1+1.4 hanging 2 | 1.0 constant +0.98 live 2+0.84 right wind 1+1.4 hanging 2 |
| 1.0 constant +0.98 live 2+0.84 left wind 2+1.4 hanging 2 | 1.0 constant +0.98 live 2+0.84 right wind 2+1.4 hanging 2 |
| 1.2 constant +0.98 live 1+1.4 left wind 1+0.98 crane 1 | 1.2 constant +0.98 live 1+1.4 right wind 1+0.98 crane 1 |
| 1.2 constant +0.98 live 1+1.4 left wind 2+0.98 crane 1 | 1.2 constant +0.98 live 1+1.4 right wind 2+0.98 crane 1 |
| 1.2 constant +0.98 live 2+1.4 left wind 1+0.98 crane 2 | 1.2 constant +0.98 live 2+1.4 right wind 1+0.98 crane 2 |
| 1.2 constant +0.98 live 2+1.4 left wind 2+0.98 crane 2 | 1.2 constant +0.98 live 2+1.4 right wind 2+0.98 crane 2 |
| 1.0 constant +0.98 live 1+1.4 left wind 1+0.98 hanging 1 | 1.0 constant +0.98 live 1+1.4 right wind 1+0.98 crane 1 |
| 1.0 constant +0.98 live 1+1.4 left wind 2+0.98 crane 1 | 1.0 constant +0.98 live 1+1.4 right wind 2+0.98 hanging 1 |
| 1.0 constant +0.98 live 2+1.4 left wind 1+0.98 crane 2 | 1.0 constant +0.98 live 2+1.4 right wind 1+0.98 hanging 2 |
| 1.0 constant +0.98 live 2+1.4 left wind 2+0.98 crane 2 | 1.0 constant +0.98 live 2+1.4 right wind 2+0.98 crane 2 |
| 1.2 constant +1.4 live 1+0.84 left wind 1+0.98 crane 1 | 1.2 constant +1.4 live 1+0.84 right wind 1+0.98 crane 1 |
| 1.2 constant +1.4 live 1+0.84 left wind 2+0.98 crane 1 | 1.2 constant +1.4 live 1+0.84 right wind 2+0.98 crane 1 |
| 1.2 constant +1.4 live 2+0.84 left wind 1+0.98 crane 2 | 1.2 constant +1.4 live 2+0.84 right wind 1+0.98 crane 2 |
| 1.2 constant +1.4 live 2+0.84 left wind 2+0.98 crane 2 | 1.2 constant +1.4 live 2+0.84 right wind 2+0.98 crane 2 |
| 1.0 constant +1.4 live 1+0.84 left wind 1+0.98 crane 1 | 1.0 constant +1.4 live 1+0.84 right wind 1+0.98 crane 1 |
| 1.0 constant +1.4 live 1+0.84 left wind 2+0.98 crane 1 | 1.0 constant +1.4 live 1+0.84 right wind 2+0.98 crane 1 |
| 1.0 constant +1.4 live 2+0.84 left wind 1+0.98 crane 2 | 1.0 constant +1.4 live 2+0.84 right wind 1+0.98 crane 2 |
| 1.0 constant +1.4 live 2+0.84 left wind 2+0.98 crane 2 | 1.0 constant +1.4 live 2+0.84 right wind 2+0.98 crane 2 |
| 1.2 constant +0.6 live 1+0.6 hanging 1+1.3 left earthquake | 1.2 constant +0.6 live 1+0.6 hanging 1+1.3 right earthquake |
| 1.2 constant +0.6 live 2+0.6 hanging 2+1.3 left earthquake | 1.2 constant +0.6 live 2+0.6 hanging 2+1.3 right earthquake |
| 1.0 constant +0.5 live 1+0.5 crane 1+1.3 left earthquake | 1.0 constant +0.5 live 1+0.5 crane 1+1.3 right earthquake |
| 1.0 constant +0.5 live 2+0.5 crane 2+1.3 left earthquake | 1.0 constant +0.5 live 2+0.5 crane 2+1.3 right earthquake |

7. Seismic calculation information

1. The earthquake left

Seismic force calculation mass concentration information

| Quality concentration node number | Mass weight (KN) |
| --- | --- |
| 1 | 72.968 |
| 2 | 72.968 |
| 3 | 61.265 |

Bottom shear force acting on horizontal seismic standard value: 12.752

Bottom minimum seismic shear force (article 5.2.5 of seismic code): 3.315

Seismic force adjustment coefficient of each particle: 1.000

Shear weight ratio after seismic force adjustment: 0.062

Period (times reduced coefficient)

| Vibration model | Cycle (s) |
| --- | --- |
| 1 | 0.448 |
| 2 | 0.145 |
| 3 | 0.078 |

2. Right earthquake

Seismic force calculation mass concentration information

| Quality concentration node number | Mass weight (KN) |
| --- | --- |
| 1 | 72.968 |
| 2 | 72.968 |
| 4 | 61.265 |

Bottom shear force acting on horizontal seismic standard value: 12.752

Bottom minimum seismic shear force (article 5.2.5 of seismic code): 3.315

Seismic force adjustment coefficient of each particle: 1.000

Shear weight ratio after seismic force adjustment: 0.062

Period (times reduced coefficient)

| Vibration model | Cycle (s) |
| --- | --- |
| 1 | 0.448 |
| 2 | 0.145 |
| 3 | 0.078 |

Eight. Internal force calculation results

1. Single working condition internal force

Column internal force

| Working condition | unit | I end N (kN) | I end V (kN) | I end M (kN. M) | II end N (kN) | II the V (kN) | II the M (kN. M) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The constant load | 1 | 24.3 | 8.7 | 24.2 | 21.2 | 8.7 | 19.2 |
| 恒荷载 | 2 | 24.3 | 8.7 | 24.2 | 21.2 | 8.7 | 19.2 |
| 恒荷载 | 3 | 21.2 | 8.7 | 19.2 | 19.6 | 8.7 | 40.9 |
| 恒荷载 | 4 | 21.2 | 8.7 | 19.2 | 19.6 | 8.7 | 40.9 |
| Left the wind 1 | 1 | 89.3 | 58.7 | 160.4 | 89.3 | 42.2 | 91.6 |
| 左风1 | 2 | 74.8 | 10.7 | 32.8 | 74.8 | 27.2 | 61.7 |
| 左风1 | 3 | 89.3 | 42.2 | 91.6 | 89.3 | 33.9 | 186.7 |
| 左风1 | 4 | 74.8 | 27.2 | 61.7 | 74.8 | 35.4 | 139.9 |
| 1 the right wind | 1 | 71.4 | 22.9 | 32.7 | 71.4 | 26.6 | 41.8 |
| 右风1 | 2 | 70.2 | 60.3 | 174.3 | 70.2 | 43.8 | 86.1 |
| 右风1 | 3 | 71.4 | 26.6 | 41.8 | 71.4 | 34.8 | 118.5 |
| 右风1 | 4 | 70.2 | 43.8 | 86.1 | 70.2 | 35.6 | 185.4 |
| The left 2 wind | 1 | 44.1 | 52.9 | 158.2 | 44.1 | 36.4 | 65.2 |
| 左风2 | 2 | 75.3 | 2.5 | 17.7 | 75.3 | 14.0 | 46.3 |
| 左风2 | 3 | 44.1 | 36.4 | 65.2 | 44.1 | 28.2 | 146.0 |
| 左风2 | 4 | 75.3 | 14.0 | 46.3 | 75.3 | 22.2 | 91.6 |
| 2 the right wind | 1 | 74.5 | 32.6 | 70.8 | 74.5 | 16.9 | 31.7 |
| 右风2 | 2 | 44.9 | 55.9 | 173.3 | 44.9 | 39.4 | 64.9 |
| 右风2 | 3 | 74.5 | 16.9 | 31.7 | 74.5 | 25.2 | 84.3 |
| 右风2 | 4 | 44.9 | 39.4 | 64.9 | 44.9 | 31.1 | 153.1 |
| Lift 1 left - wheel pressure | 1 | 179.6 | 9.5 | 1.9 | 179.6 | 9.5 | 45.8 |
| 吊 1 左-轮压 | 2 | 58.2 | 9.5 | 31.7 | 58.2 | 9.5 | 16.0 |
| 吊 1 左-轮压 | 3 | 1.8 | 9.5 | 45.0 | 1.8 | 9.5 | 21.1 |
| 吊 1 左-轮压 | 4 | 1.8 | 9.5 | 12.2 | 1.8 | 9.5 | 11.7 |
| Lift 1 right - wheel pressure | 1 | 58.2 | 9.5 | 31.7 | 58.2 | 9.5 | 16.0 |
| 吊 1 右-轮压 | 2 | 179.6 | 9.5 | 1.9 | 179.6 | 9.5 | 45.8 |
| 吊 1 右-轮压 | 3 | 1.8 | 9.5 | 12.2 | 1.8 | 9.5 | 11.7 |
| 吊 1 右-轮压 | 4 | 1.8 | 9.5 | 45.0 | 1.8 | 9.5 | 21.1 |
| Lift 1 left - brake force | 1 | 1.0 | 5.0 | 20.2 | 1.0 | 5.0 | 4.8 |
| 吊 1 左-刹车力 | 2 | 1.0 | 5.0 | 20.2 | 1.0 | 5.0 | 4.8 |
| 吊 1 左-刹车力 | 3 | 1.0 | 5.0 | 4.8 | 1.0 | 0.0 | 8.8 |
| 吊 1 左-刹车力 | 4 | 1.0 | 5.0 | 4.8 | 1.0 | 0.0 | 8.8 |
| Lift 1 right - brake force | 1 | 1.0 | 5.0 | 20.2 | 1.0 | 5.0 | 4.8 |
| 吊 1 右-刹车力 | 2 | 1.0 | 5.0 | 20.2 | 1.0 | 5.0 | 4.8 |
| 吊 1 右-刹车力 | 3 | 1.0 | 5.0 | 4.8 | 1.0 | 0.0 | 8.8 |
| 吊 1 右-刹车力 | 4 | 1.0 | 5.0 | 4.8 | 1.0 | 0.0 | 8.8 |
| The earthquake left | 1 | 1.4 | 6.4 | 25.8 | 1.4 | 6.4 | 6.5 |
| 左地震 | 2 | 1.4 | 6.3 | 25.5 | 1.4 | 6.3 | 6.1 |
| 左地震 | 3 | 1.4 | 2.5 | 6.5 | 1.4 | 2.5 | 12.4 |
| 左地震 | 4 | 1.4 | 2.6 | 6.1 | 1.4 | 2.6 | 12.4 |
| Right earthquake | 1 | 1.4 | 6.3 | 25.5 | 1.4 | 6.3 | 6.1 |
| 右地震 | 2 | 1.4 | 6.4 | 25.8 | 1.4 | 6.4 | 6.5 |
| 右地震 | 3 | 1.4 | 2.6 | 6.1 | 1.4 | 2.6 | 12.4 |
| 右地震 | 4 | 1.4 | 2.5 | 6.5 | 1.4 | 2.5 | 12.4 |

The internal forces of the beam

| Working condition of no. | Unit no. | I end N (kN) | I end V (kN) | I end M (kN. M) | II end N (kN) | II the V (kN) | II the M (kN. M) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The constant load | 1 | 0.0 | 0.0 | 0.0 | 0.4 | 3.6 | 4.2 |
| 恒荷载 | 2 | 10.2 | 15.1 | 45.0 | 8.6 | 0.9 | 19.1 |
| 恒荷载 | 3 | 0.4 | 3.6 | 4.2 | 0.0 | 0.0 | 0.0 |
| 恒荷载 | 4 | 8.6 | 0.9 | 19.1 | 10.2 | 15.1 | 45.0 |
| Left the wind 1 | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 18.3 | 21.2 |
| 左风1 | 2 | 42.6 | 67.2 | 207.9 | 42.6 | 4.5 | 75.7 |
| 左风1 | 3 | 0.0 | 15.3 | 17.6 | 0.0 | 0.0 | 0.0 |
| 左风1 | 4 | 42.7 | 4.1 | 75.7 | 42.7 | 55.6 | 157.6 |
| 1 the right wind | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 15.3 | 17.6 |
| 右风1 | 2 | 41.8 | 52.3 | 136.2 | 41.8 | 7.4 | 66.7 |
| 右风1 | 3 | 0.0 | 7.6 | 8.8 | 0.0 | 0.0 | 0.0 |
| 右风1 | 4 | 42.4 | 1.0 | 66.7 | 42.4 | 58.7 | 194.2 |
| The left 2 wind | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 6.1 | 7.1 |
| 左风2 | 2 | 32.4 | 34.9 | 153.1 | 32.4 | 11.1 | 55.0 |
| 左风2 | 3 | 0.0 | 18.3 | 21.2 | 0.0 | 0.0 | 0.0 |
| 左风2 | 4 | 29.6 | 17.3 | 55.0 | 29.6 | 54.4 | 112.7 |
| 2 the right wind | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 18.3 | 21.2 |
| 右风2 | 2 | 32.5 | 53.3 | 105.5 | 32.5 | 18.4 | 52.5 |
| 右风2 | 3 | 0.0 | 6.1 | 7.1 | 0.0 | 0.0 | 0.0 |
| 右风2 | 4 | 35.5 | 11.6 | 52.5 | 35.5 | 35.4 | 160.1 |
| Lift 1 left - wheel pressure | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 吊 1 左-轮压 | 2 | 9.3 | 2.8 | 21.1 | 9.3 | 2.8 | 3.9 |
| 吊 1 左-轮压 | 3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 吊 1 左-轮压 | 4 | 9.7 | 0.9 | 3.9 | 9.7 | 0.9 | 11.7 |
| Lift 1 right - wheel pressure | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 吊 1 右-轮压 | 2 | 9.7 | 0.9 | 11.7 | 9.7 | 0.9 | 3.9 |
| 吊 1 右-轮压 | 3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 吊 1 右-轮压 | 4 | 9.3 | 2.8 | 3.9 | 9.3 | 2.8 | 21.1 |
| Lift 1 left - brake force | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 吊 1 左-刹车力 | 2 | 0.1 | 1.0 | 8.8 | 0.1 | 1.0 | 0.0 |
| 吊 1 左-刹车力 | 3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 吊 1 左-刹车力 | 4 | 0.1 | 1.0 | 0.0 | 0.1 | 1.0 | 8.8 |
| Lift 1 right - brake force | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 吊 1 右-刹车力 | 2 | 0.1 | 1.0 | 8.8 | 0.1 | 1.0 | 0.0 |
| 吊 1 右-刹车力 | 3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 吊 1 右-刹车力 | 4 | 0.1 | 1.0 | 0.0 | 0.1 | 1.0 | 8.8 |
| The earthquake left | 1 | 1.0 | 0.1 | 0.0 | 1.0 | 0.1 | 0.2 |
| 左地震 | 2 | 0.7 | 1.3 | 12.2 | 0.7 | 1.3 | 0.2 |
| 左地震 | 3 | 1.0 | 0.1 | 0.2 | 1.0 | 0.1 | 0.0 |
| 左地震 | 4 | 0.7 | 1.3 | 0.2 | 0.7 | 1.3 | 12.1 |
| Right earthquake | 1 | 1.0 | 0.1 | 0.0 | 1.0 | 0.1 | 0.2 |
| 右地震 | 2 | 0.7 | 1.3 | 12.1 | 0.7 | 1.3 | 0.2 |
| 右地震 | 3 | 1.0 | 0.1 | 0.2 | 1.0 | 0.1 | 0.0 |
| 右地震 | 4 | 0.7 | 1.3 | 0.2 | 0.7 | 1.3 | 12.2 |

Node displacement

Node displacement under constant load (mm)

| Node number | X to the displacement | Y to the displacement |
| --- | --- | --- |
| 1 | 2.71 | 0.08 |
| 2 | 2.71 | 0.08 |
| 3 | 1.81 | 2.27 |
| 4 | 1.81 | 2.27 |
| 5 | 1.57 | 0.11 |
| 6 | 1.57 | 0.11 |
| 7 | 0.00 | 16.43 |

Node displacement under live load (mm)

| Node number | X to the displacement | Y to the displacement |
| --- | --- | --- |
| 1 | 2.74 | 0.07 |
| 2 | 2.74 | 0.07 |
| 3 | 1.82 | 1.33 |
| 4 | 1.82 | 1.33 |
| 5 | 1.59 | 0.10 |
| 6 | 1.59 | 0.10 |
| 7 | 0.00 | 17.32 |

Lateral (horizontal) displacement of node (mm)

| Working condition | node | Delta x | node | Delta x |
| --- | --- | --- | --- | --- |
| Left the wind 1 | 1 | 19.39 | 2 | 2.28 |
| 左风1 | 3 | 19.03 | 4 | 4.72 |
| 左风1 | 5 | 18.34 | 6 | 5.84 |
| 左风1 | 7 | 12.09 | 8 | 0.00 |
| 左风1 | 9 | 0.00 |  |  |
| 1 the right wind | 1 | 4.21 | 2 | 22.48 |
| 右风1 | 3 | 12.34 | 4 | 25.07 |
| 右风1 | 5 | 13.42 | 6 | 24.43 |
| 右风1 | 7 | 18.93 | 8 | 0.00 |
| 右风1 | 9 | 0.00 |  |  |
| The left 2 wind | 1 | 21.44 | 2 | 5.68 |
| 左风2 | 3 | 26.17 | 4 | 15.76 |
| 左风2 | 5 | 25.94 | 6 | 16.85 |
| 左风2 | 7 | 21.40 | 8 | 0.00 |
| 左风2 | 9 | 0.00 |  |  |
| 2 the right wind | 1 | 10.35 | 2 | 24.28 |
| 右风2 | 3 | 21.10 | 4 | 30.85 |
| 右风2 | 5 | 22.19 | 6 | 30.79 |
| 右风2 | 7 | 26.49 | 8 | 0.00 |
| 右风2 | 9 | 0.00 |  |  |
| Lift 1 left - brake force | 1 | 3.32 | 2 | 3.32 |
| 吊 1 左-刹车力 | 3 | 4.85 | 4 | 4.85 |
| 吊 1 左-刹车力 | 5 | 4.95 | 6 | 4.95 |
| 吊 1 左-刹车力 | 7 | 4.95 | 8 | 0.00 |
| 吊 1 左-刹车力 | 9 | 0.00 |  |  |
| Lift 1 right - brake force | 1 | 3.32 | 2 | 3.32 |
| 吊 1 右-刹车力 | 3 | 4.85 | 4 | 4.85 |
| 吊 1 右-刹车力 | 5 | 4.95 | 6 | 4.95 |
| 吊 1 右-刹车力 | 7 | 4.95 | 8 | 0.00 |
| 吊 1 右-刹车力 | 9 | 0.00 |  |  |
| The earthquake left | 1 | 4.21 | 2 | 4.18 |
| 左地震 | 3 | 6.22 | 4 | 6.21 |
| 左地震 | 5 | 6.34 | 6 | 6.33 |
| 左地震 | 7 | 6.34 | 8 | 0.00 |
| 左地震 | 9 | 0.00 |  |  |
| Right earthquake | 1 | 4.18 | 2 | 4.21 |
| 右地震 | 3 | 6.21 | 4 | 6.22 |
| 右地震 | 5 | 6.33 | 6 | 6.34 |
| 右地震 | 7 | 6.34 | 8 | 0.00 |
| 右地震 | 9 | 0.00 |  |  |

Results of component design

Seismic rating: grade 3

Cross section parameters: B1=250, B2=250, H=400, Tw=6, T1=10, T2=10

Classification of axial compression section: X-axis :b, Y-axis :c

Steel number of component: Q345

Check calculation specification: door gb51022-2015

| \ | I end |  |  | II end |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| combination | M | N | V | M | N | V |
| 1 | 30.69 | 36.94 | 10.96 | 24.14 | 32.69 | 10.96 |
| 2 | 58.53 | 48.81 | 21.03 | 46.61 | 44.57 | 21.03 |
| 3 | 58.06 | 53.10 | 20.65 | 45.21 | 48.85 | 20.65 |
| 4 | 31.15 | 32.66 | 11.34 | 25.54 | 28.41 | 11.34 |
| 5 | 26.22 | 35.03 | 9.34 | 20.50 | 31.26 | 9.34 |
| 6 | 65.99 | 52.00 | 23.72 | 52.60 | 48.23 | 23.72 |
| 7 | 65.32 | 58.12 | 23.19 | 50.60 | 54.34 | 23.19 |
| 8 | 26.88 | 28.92 | 9.88 | 22.50 | 25.15 | 9.88 |
| 9 | 21.38 | 30.17 | 7.61 | 16.67 | 27.02 | 7.61 |
| 10 | 61.15 | 47.13 | 21.98 | 48.77 | 43.99 | 21.98 |
| 11 | 60.49 | 53.25 | 21.45 | 46.77 | 50.11 | 21.45 |
| 12 | 22.04 | 24.05 | 8.14 | 18.67 | 20.91 | 8.14 |
| 13 | 195.58 | 95.80 | 71.72 | 105.28 | 99.57 | 48.62 |
| 14 | 74.78 | 70.69 | 42.51 | 35.50 | 74.46 | 26.79 |
| 15 | 192.51 | 32.49 | 63.71 | 68.29 | 36.26 | 40.61 |
| 16 | 128.10 | 75.04 | 56.01 | 21.31 | 78.81 | 13.29 |
| 17 | 200.42 | 100.67 | 73.46 | 109.12 | 103.81 | 50.36 |
| 18 | 69.94 | 75.56 | 40.77 | 39.34 | 78.70 | 28.53 |
| 19 | 197.34 | 37.36 | 65.44 | 72.12 | 40.50 | 42.34 |
| 20 | 123.26 | 79.90 | 54.27 | 25.15 | 83.05 | 15.03 |
| 21 | 108.54 | 39.97 | 39.93 | 56.48 | 43.74 | 26.07 |
| 22 | 53.67 | 24.91 | 28.60 | 14.61 | 28.68 | 12.98 |
| 23 | 106.70 | 1.99 | 35.13 | 34.28 | 5.76 | 21.27 |
| 24 | 85.66 | 27.51 | 36.70 | 6.10 | 31.29 | 4.88 |
| 25 | 68.77 | 23.00 | 25.56 | 24.38 | 26.78 | 11.70 |
| 26 | 93.44 | 7.94 | 42.98 | 17.49 | 11.71 | 1.40 |
| 27 | 66.93 | 14.98 | 20.75 | 2.18 | 11.21 | 6.89 |
| 28 | 125.43 | 10.55 | 51.08 | 26.00 | 14.32 | 9.50 |
| 29 | 69.44 | 16.89 | 26.09 | 26.38 | 20.66 | 12.23 |
| 30 | 92.78 | 1.82 | 42.44 | 15.49 | 5.59 | 0.86 |
| 31 | 67.59 | 21.10 | 21.28 | 4.18 | 17.32 | 7.42 |
| 32 | 124.77 | 4.43 | 50.54 | 24.00 | 8.20 | 8.96 |
| 33 | 107.88 | 46.09 | 39.40 | 54.48 | 49.86 | 25.54 |
| 34 | 54.33 | 31.02 | 29.14 | 12.61 | 34.79 | 12.44 |
| 35 | 106.04 | 8.10 | 34.59 | 32.28 | 11.87 | 20.73 |
| 36 | 86.33 | 33.63 | 37.24 | 4.10 | 37.40 | 4.34 |
| 37 | 113.38 | 44.84 | 41.67 | 60.31 | 47.98 | 27.81 |
| 38 | 48.83 | 29.77 | 26.87 | 18.45 | 32.92 | 14.71 |
| 39 | 111.54 | 6.85 | 36.86 | 38.12 | 10.00 | 23.00 |
| 40 | 80.83 | 32.38 | 34.97 | 9.93 | 35.52 | 6.61 |
| 41 | 73.61 | 27.87 | 27.30 | 28.21 | 31.02 | 13.44 |
| 42 | 88.60 | 12.81 | 41.24 | 13.65 | 15.95 | 0.34 |
| 43 | 71.77 | 10.11 | 22.49 | 6.02 | 6.97 | 8.63 |
| 44 | 120.60 | 15.41 | 49.34 | 22.16 | 18.56 | 7.76 |
| 45 | 74.27 | 21.76 | 27.83 | 30.21 | 24.90 | 13.97 |
| 46 | 87.94 | 6.69 | 40.71 | 11.65 | 9.83 | 0.87 |
| 47 | 72.43 | 16.23 | 23.02 | 8.02 | 13.09 | 9.16 |
| 48 | 119.93 | 9.30 | 48.81 | 20.16 | 12.44 | 7.23 |
| 49 | 112.72 | 50.96 | 41.14 | 58.31 | 54.10 | 27.28 |
| 50 | 49.50 | 35.89 | 27.40 | 16.45 | 39.03 | 14.18 |
| 51 | 110.87 | 12.97 | 36.33 | 36.12 | 16.11 | 22.47 |
| 52 | 81.49 | 38.50 | 35.50 | 7.94 | 41.64 | 6.08 |
| 53 | 197.54 | 91.72 | 72.47 | 107.04 | 95.50 | 49.37 |
| 54 | 72.82 | 66.61 | 41.76 | 37.26 | 70.39 | 27.54 |
| 55 | 194.47 | 28.41 | 64.45 | 70.05 | 32.19 | 41.35 |
| 56 | 126.13 | 70.96 | 55.26 | 23.08 | 74.73 | 14.04 |
| 57 | 169.70 | 79.85 | 62.41 | 84.57 | 83.62 | 39.31 |
| 58 | 100.66 | 54.74 | 51.82 | 14.79 | 58.51 | 17.48 |
| 59 | 166.63 | 16.54 | 54.39 | 47.58 | 20.31 | 31.29 |
| 60 | 153.97 | 59.08 | 65.32 | 0.61 | 62.85 | 3.98 |
| 61 | 170.17 | 75.57 | 62.78 | 85.97 | 79.34 | 39.68 |
| 62 | 100.19 | 50.46 | 51.45 | 16.19 | 54.23 | 17.85 |
| 63 | 167.09 | 12.26 | 54.76 | 48.98 | 16.03 | 31.66 |
| 64 | 153.51 | 54.80 | 64.95 | 2.01 | 58.57 | 4.35 |
| 65 | 197.08 | 96.00 | 72.09 | 105.64 | 99.78 | 48.99 |
| 66 | 73.28 | 70.89 | 42.13 | 35.86 | 74.67 | 27.17 |
| 67 | 194.01 | 32.69 | 64.08 | 68.65 | 36.47 | 40.98 |
| 68 | 126.60 | 75.24 | 55.63 | 21.68 | 79.01 | 13.67 |
| 69 | 202.38 | 96.59 | 74.20 | 110.88 | 99.74 | 51.10 |
| 70 | 67.98 | 71.48 | 40.03 | 41.10 | 74.62 | 29.27 |
| 71 | 199.31 | 33.28 | 66.19 | 73.88 | 36.43 | 43.09 |
| 72 | 121.30 | 75.83 | 53.53 | 26.91 | 78.97 | 15.77 |
| 73 | 174.54 | 84.71 | 64.14 | 88.41 | 87.86 | 41.04 |
| 74 | 95.82 | 59.60 | 50.09 | 18.63 | 62.75 | 19.21 |
| 75 | 171.47 | 21.40 | 56.13 | 51.42 | 24.55 | 33.03 |
| 76 | 149.14 | 63.95 | 63.59 | 4.45 | 67.09 | 5.71 |
| 77 | 175.01 | 80.43 | 64.51 | 89.81 | 83.58 | 41.41 |
| 78 | 95.35 | 55.32 | 49.72 | 20.03 | 58.47 | 19.58 |
| 79 | 171.93 | 17.12 | 56.50 | 52.82 | 20.27 | 33.40 |
| 80 | 148.67 | 59.67 | 63.22 | 5.84 | 62.81 | 6.08 |
| 81 | 201.92 | 100.87 | 73.83 | 109.48 | 104.02 | 50.73 |
| 82 | 68.44 | 75.76 | 40.40 | 39.70 | 78.91 | 28.90 |
| 83 | 198.84 | 37.56 | 65.82 | 72.49 | 40.71 | 42.72 |
| 84 | 121.76 | 80.11 | 53.90 | 25.51 | 83.25 | 15.40 |
| 85 | 5.75 | 29.89 | 1.58 | 13.54 | 26.11 | 1.58 |
| 86 | 61.02 | 33.52 | 18.17 | 29.87 | 29.75 | 18.17 |
| 87 | 11.30 | 37.16 | 7.74 | 27.30 | 33.39 | 7.74 |
| 88 | 78.06 | 40.80 | 24.33 | 43.63 | 37.02 | 24.33 |
| 89 | 11.01 | 39.78 | 7.51 | 26.44 | 36.01 | 7.51 |
| 90 | 77.78 | 43.42 | 24.10 | 42.77 | 39.64 | 24.10 |
| 91 | 5.46 | 27.27 | 1.80 | 14.40 | 23.49 | 1.80 |
| 92 | 61.30 | 30.90 | 18.40 | 30.73 | 27.13 | 18.40 |
| 93 | 10.38 | 24.60 | 0.08 | 9.88 | 21.46 | 0.08 |
| 94 | 56.38 | 28.24 | 16.51 | 26.21 | 25.10 | 16.51 |
| 95 | 3.82 | 30.66 | 5.05 | 21.35 | 27.52 | 5.05 |
| 96 | 70.58 | 34.30 | 21.64 | 37.68 | 31.16 | 21.64 |
| 97 | 3.58 | 32.85 | 4.86 | 20.63 | 29.70 | 4.86 |
| 98 | 70.35 | 36.48 | 21.45 | 36.96 | 33.34 | 21.45 |
| 99 | 10.15 | 22.42 | 0.11 | 10.60 | 19.28 | 0.11 |
| 100 | 56.62 | 26.06 | 16.70 | 26.93 | 22.91 | 16.70 |
| 101 | 3.40 | 279.30 | 16.76 | 80.42 | 275.53 | 16.76 |
| 102 | 101.73 | 112.07 | 30.77 | 52.12 | 108.30 | 30.77 |
| 103 | 60.09 | 282.02 | 30.77 | 93.75 | 278.25 | 30.77 |
| 104 | 60.09 | 282.02 | 30.77 | 93.75 | 278.25 | 30.77 |
| 105 | 29.02 | 29.21 | 10.41 | 23.02 | 25.44 | 10.41 |
| 106 | 29.02 | 29.21 | 10.41 | 23.02 | 25.44 | 10.41 |
| 107 | 29.02 | 29.21 | 10.41 | 23.02 | 25.44 | 10.41 |
| 108 | 60.09 | 282.02 | 30.77 | 93.75 | 278.25 | 30.77 |
| 109 | 1.44 | 274.43 | 15.03 | 76.59 | 271.29 | 15.03 |
| 110 | 96.89 | 107.20 | 29.03 | 48.28 | 104.06 | 29.03 |
| 111 | 55.25 | 277.16 | 29.03 | 89.92 | 274.01 | 29.03 |
| 112 | 55.25 | 277.16 | 29.03 | 89.92 | 274.01 | 29.03 |
| 113 | 24.19 | 24.34 | 8.67 | 19.19 | 21.20 | 8.67 |
| 114 | 24.19 | 24.34 | 8.67 | 19.19 | 21.20 | 8.67 |
| 115 | 24.19 | 24.34 | 8.67 | 19.19 | 21.20 | 8.67 |
| 116 | 55.25 | 277.16 | 29.03 | 89.92 | 274.01 | 29.03 |
| 117 | 1.43 | 283.38 | 16.02 | 78.66 | 279.60 | 16.02 |
| 118 | 27.06 | 33.29 | 9.66 | 21.26 | 29.51 | 9.66 |
| 119 | 127.60 | 128.02 | 40.08 | 72.82 | 124.25 | 40.08 |
| 120 | 85.96 | 297.98 | 40.08 | 114.46 | 294.21 | 40.08 |
| 121 | 85.50 | 302.26 | 39.71 | 113.06 | 298.49 | 39.71 |
| 122 | 85.50 | 302.26 | 39.71 | 113.06 | 298.49 | 39.71 |
| 123 | 27.52 | 29.01 | 10.04 | 22.66 | 25.23 | 10.04 |
| 124 | 27.52 | 29.01 | 10.04 | 22.66 | 25.23 | 10.04 |
| 125 | 3.40 | 278.51 | 14.28 | 74.82 | 275.36 | 14.28 |
| 126 | 22.22 | 28.42 | 7.93 | 17.42 | 25.27 | 7.93 |
| 127 | 122.76 | 123.15 | 38.35 | 68.99 | 120.01 | 38.35 |
| 128 | 81.12 | 293.11 | 38.35 | 110.62 | 289.97 | 38.35 |
| 129 | 80.66 | 297.39 | 37.98 | 109.23 | 294.25 | 37.98 |
| 130 | 80.66 | 297.39 | 37.98 | 109.23 | 294.25 | 37.98 |
| 131 | 22.69 | 24.14 | 8.30 | 18.82 | 20.99 | 8.30 |
| 132 | 22.69 | 24.14 | 8.30 | 18.82 | 20.99 | 8.30 |
| 133 | 8.28 | 210.10 | 13.79 | 60.68 | 206.32 | 13.79 |
| 134 | 26.22 | 35.03 | 9.34 | 20.50 | 31.26 | 9.34 |
| 135 | 116.88 | 110.00 | 37.97 | 72.97 | 106.23 | 37.97 |
| 136 | 87.73 | 228.97 | 37.97 | 102.12 | 225.20 | 37.97 |
| 137 | 87.07 | 235.09 | 37.44 | 100.12 | 231.32 | 37.44 |
| 138 | 87.07 | 235.09 | 37.44 | 100.12 | 231.32 | 37.44 |
| 139 | 26.88 | 28.92 | 9.88 | 22.50 | 25.15 | 9.88 |
| 140 | 26.88 | 28.92 | 9.88 | 22.50 | 25.15 | 9.88 |
| 141 | 3.44 | 205.23 | 12.06 | 56.85 | 202.09 | 12.06 |
| 142 | 21.38 | 30.17 | 7.61 | 16.67 | 27.02 | 7.61 |
| 143 | 112.04 | 105.13 | 36.23 | 69.13 | 101.99 | 36.23 |
| 144 | 82.89 | 224.10 | 36.23 | 98.28 | 220.96 | 36.23 |
| 145 | 82.23 | 230.22 | 35.70 | 96.28 | 227.08 | 35.70 |
| 146 | 82.23 | 230.22 | 35.70 | 96.28 | 227.08 | 35.70 |
| 147 | 22.04 | 24.05 | 8.14 | 18.67 | 20.91 | 8.14 |
| 148 | 22.04 | 24.05 | 8.14 | 18.67 | 20.91 | 8.14 |
| 149 | 133.33 | 208.37 | 33.26 | 1.68 | 204.60 | 19.40 |
| 150 | 107.70 | 41.72 | 39.61 | 55.72 | 45.49 | 25.75 |
| 151 | 28.89 | 223.44 | 35.28 | 43.55 | 219.66 | 6.30 |
| 152 | 54.51 | 26.65 | 28.92 | 13.85 | 30.43 | 12.66 |
| 153 | 131.48 | 246.36 | 28.45 | 23.88 | 242.58 | 14.59 |
| 154 | 105.86 | 3.73 | 34.81 | 33.52 | 7.51 | 20.95 |
| 155 | 60.88 | 220.83 | 43.38 | 52.06 | 217.06 | 1.80 |
| 156 | 86.50 | 29.26 | 37.02 | 5.34 | 33.03 | 4.56 |
| 157 | 7.16 | 53.02 | 9.19 | 4.16 | 49.24 | 4.67 |
| 158 | 48.80 | 222.97 | 9.19 | 37.48 | 219.20 | 4.67 |
| 159 | 155.05 | 68.08 | 59.34 | 37.71 | 64.31 | 17.76 |
| 160 | 113.42 | 238.04 | 59.34 | 79.35 | 234.27 | 17.76 |
| 161 | 5.32 | 91.00 | 4.39 | 18.04 | 87.23 | 9.47 |
| 162 | 46.96 | 260.96 | 4.39 | 59.68 | 257.19 | 9.47 |
| 163 | 187.05 | 65.48 | 67.44 | 46.22 | 61.70 | 25.86 |
| 164 | 145.41 | 235.43 | 67.44 | 87.86 | 231.66 | 25.86 |
| 165 | 49.26 | 227.25 | 9.57 | 36.08 | 223.48 | 4.29 |
| 166 | 49.26 | 227.25 | 9.57 | 36.08 | 223.48 | 4.29 |
| 167 | 112.95 | 242.32 | 58.97 | 77.95 | 238.55 | 17.39 |
| 168 | 112.95 | 242.32 | 58.97 | 77.95 | 238.55 | 17.39 |
| 169 | 47.42 | 265.24 | 4.76 | 58.28 | 261.47 | 9.10 |
| 170 | 47.42 | 265.24 | 4.76 | 58.28 | 261.47 | 9.10 |
| 171 | 144.94 | 239.71 | 67.07 | 86.46 | 235.94 | 25.49 |
| 172 | 144.94 | 239.71 | 67.07 | 86.46 | 235.94 | 25.49 |
| 173 | 107.24 | 46.00 | 39.24 | 54.32 | 49.77 | 25.38 |
| 174 | 107.24 | 46.00 | 39.24 | 54.32 | 49.77 | 25.38 |
| 175 | 54.98 | 30.93 | 29.29 | 12.45 | 34.71 | 12.29 |
| 176 | 54.98 | 30.93 | 29.29 | 12.45 | 34.71 | 12.29 |
| 177 | 105.39 | 8.01 | 34.43 | 32.13 | 11.79 | 20.57 |
| 178 | 105.39 | 8.01 | 34.43 | 32.13 | 11.79 | 20.57 |
| 179 | 86.97 | 33.54 | 37.40 | 3.94 | 37.31 | 4.18 |
| 180 | 86.97 | 33.54 | 37.40 | 3.94 | 37.31 | 4.18 |
| 181 | 138.17 | 203.50 | 34.99 | 2.16 | 200.36 | 21.13 |
| 182 | 112.54 | 46.59 | 41.35 | 59.56 | 49.73 | 27.49 |
| 183 | 24.05 | 218.57 | 33.54 | 39.71 | 215.42 | 8.04 |
| 184 | 49.68 | 31.52 | 27.19 | 17.69 | 34.66 | 14.39 |
| 185 | 136.32 | 241.49 | 30.19 | 20.04 | 238.34 | 16.33 |
| 186 | 110.70 | 8.60 | 36.54 | 37.36 | 11.75 | 22.68 |
| 187 | 56.04 | 215.96 | 41.64 | 48.22 | 212.82 | 0.06 |
| 188 | 81.67 | 34.13 | 35.29 | 9.18 | 37.27 | 6.29 |
| 189 | 12.00 | 48.15 | 10.93 | 7.99 | 45.01 | 2.93 |
| 190 | 53.64 | 218.11 | 10.93 | 33.64 | 214.96 | 2.93 |
| 191 | 150.22 | 63.21 | 57.61 | 33.87 | 60.07 | 16.03 |
| 192 | 108.58 | 233.17 | 57.61 | 75.51 | 230.03 | 16.03 |
| 193 | 10.15 | 86.13 | 6.12 | 14.20 | 82.99 | 7.74 |
| 194 | 51.79 | 256.09 | 6.12 | 55.84 | 252.95 | 7.74 |
| 195 | 182.21 | 60.61 | 65.71 | 42.38 | 57.46 | 24.13 |
| 196 | 140.57 | 230.56 | 65.71 | 84.02 | 227.42 | 24.13 |
| 197 | 54.10 | 222.39 | 11.30 | 32.24 | 219.24 | 2.56 |
| 198 | 54.10 | 222.39 | 11.30 | 32.24 | 219.24 | 2.56 |
| 199 | 108.11 | 237.45 | 57.24 | 74.11 | 234.31 | 15.66 |
| 200 | 108.11 | 237.45 | 57.24 | 74.11 | 234.31 | 15.66 |
| 201 | 52.26 | 260.37 | 6.49 | 54.44 | 257.23 | 7.37 |
| 202 | 52.26 | 260.37 | 6.49 | 54.44 | 257.23 | 7.37 |
| 203 | 140.11 | 234.84 | 65.34 | 82.62 | 231.70 | 23.76 |
| 204 | 140.11 | 234.84 | 65.34 | 82.62 | 231.70 | 23.76 |
| 205 | 112.08 | 50.87 | 40.98 | 58.16 | 54.01 | 27.12 |
| 206 | 112.08 | 50.87 | 40.98 | 58.16 | 54.01 | 27.12 |
| 207 | 50.14 | 35.80 | 27.56 | 16.29 | 38.95 | 14.02 |
| 208 | 50.14 | 35.80 | 27.56 | 16.29 | 38.95 | 14.02 |
| 209 | 110.23 | 12.88 | 36.17 | 35.96 | 16.03 | 22.31 |
| 210 | 110.23 | 12.88 | 36.17 | 35.96 | 16.03 | 22.31 |
| 211 | 82.13 | 38.41 | 35.66 | 7.78 | 41.55 | 5.92 |
| 212 | 82.13 | 38.41 | 35.66 | 7.78 | 41.55 | 5.92 |
| 213 | 215.48 | 83.34 | 68.02 | 66.86 | 79.57 | 44.92 |
| 214 | 197.54 | 91.72 | 72.47 | 107.04 | 95.50 | 49.37 |
| 215 | 54.88 | 108.45 | 46.21 | 2.92 | 104.68 | 23.09 |
| 216 | 72.82 | 66.61 | 41.76 | 37.26 | 70.39 | 27.54 |
| 217 | 212.41 | 146.65 | 60.01 | 29.87 | 142.88 | 36.91 |
| 218 | 194.47 | 28.41 | 64.45 | 70.05 | 32.19 | 41.35 |
| 219 | 108.20 | 104.10 | 59.71 | 17.10 | 100.33 | 9.59 |
| 220 | 126.13 | 70.96 | 55.26 | 23.08 | 74.73 | 14.04 |
| 221 | 118.81 | 21.84 | 48.15 | 64.21 | 25.62 | 25.05 |
| 222 | 147.96 | 97.12 | 48.15 | 35.06 | 93.35 | 25.05 |
| 223 | 151.55 | 3.27 | 66.07 | 5.57 | 0.51 | 3.23 |
| 224 | 122.40 | 122.24 | 66.07 | 34.72 | 118.46 | 3.23 |
| 225 | 115.74 | 41.46 | 40.14 | 27.21 | 37.69 | 17.04 |
| 226 | 144.89 | 160.43 | 40.14 | 1.93 | 156.66 | 17.04 |
| 227 | 204.87 | 1.08 | 79.57 | 19.76 | 4.85 | 10.27 |
| 228 | 175.72 | 117.89 | 79.57 | 48.90 | 114.12 | 10.27 |
| 229 | 148.42 | 101.41 | 48.53 | 36.46 | 97.63 | 25.43 |
| 230 | 148.42 | 101.41 | 48.53 | 36.46 | 97.63 | 25.43 |
| 231 | 121.94 | 126.52 | 65.70 | 33.32 | 122.74 | 3.60 |
| 232 | 121.94 | 126.52 | 65.70 | 33.32 | 122.74 | 3.60 |
| 233 | 145.35 | 164.72 | 40.51 | 0.53 | 160.94 | 17.41 |
| 234 | 145.35 | 164.72 | 40.51 | 0.53 | 160.94 | 17.41 |
| 235 | 175.25 | 122.17 | 79.20 | 47.51 | 118.40 | 9.90 |
| 236 | 175.25 | 122.17 | 79.20 | 47.51 | 118.40 | 9.90 |
| 237 | 197.08 | 96.00 | 72.09 | 105.64 | 99.78 | 48.99 |
| 238 | 197.08 | 96.00 | 72.09 | 105.64 | 99.78 | 48.99 |
| 239 | 73.28 | 70.89 | 42.13 | 35.86 | 74.67 | 27.17 |
| 240 | 73.28 | 70.89 | 42.13 | 35.86 | 74.67 | 27.17 |
| 241 | 194.01 | 32.69 | 64.08 | 68.65 | 36.47 | 40.98 |
| 242 | 194.01 | 32.69 | 64.08 | 68.65 | 36.47 | 40.98 |
| 243 | 126.60 | 75.24 | 55.63 | 21.68 | 79.01 | 13.67 |
| 244 | 126.60 | 75.24 | 55.63 | 21.68 | 79.01 | 13.67 |
| 245 | 220.32 | 78.47 | 69.75 | 70.70 | 75.33 | 46.65 |
| 246 | 202.38 | 96.59 | 74.20 | 110.88 | 99.74 | 51.10 |
| 247 | 50.04 | 103.58 | 44.47 | 0.92 | 100.44 | 24.83 |
| 248 | 67.98 | 71.48 | 40.03 | 41.10 | 74.62 | 29.27 |
| 249 | 217.25 | 141.78 | 61.74 | 33.70 | 138.64 | 38.64 |
| 250 | 199.31 | 33.28 | 66.19 | 73.88 | 36.43 | 43.09 |
| 251 | 103.36 | 99.24 | 57.98 | 13.27 | 96.09 | 11.32 |
| 252 | 121.30 | 75.83 | 53.53 | 26.91 | 78.97 | 15.77 |
| 253 | 123.65 | 26.71 | 49.89 | 68.04 | 29.86 | 26.79 |
| 254 | 152.80 | 92.26 | 49.89 | 38.90 | 89.11 | 26.79 |
| 255 | 146.71 | 1.60 | 64.34 | 1.74 | 4.75 | 4.96 |
| 256 | 117.56 | 117.37 | 64.34 | 30.88 | 114.22 | 4.96 |
| 257 | 120.58 | 36.60 | 41.88 | 31.05 | 33.45 | 18.78 |
| 258 | 149.72 | 155.57 | 41.88 | 1.90 | 152.42 | 18.78 |
| 259 | 200.03 | 5.95 | 77.84 | 15.92 | 9.09 | 8.54 |
| 260 | 170.88 | 113.02 | 77.84 | 45.07 | 109.88 | 8.54 |
| 261 | 153.26 | 96.54 | 50.26 | 40.30 | 93.39 | 27.16 |
| 262 | 153.26 | 96.54 | 50.26 | 40.30 | 93.39 | 27.16 |
| 263 | 117.10 | 121.65 | 63.97 | 29.48 | 118.50 | 5.33 |
| 264 | 117.10 | 121.65 | 63.97 | 29.48 | 118.50 | 5.33 |
| 265 | 150.19 | 159.85 | 42.25 | 3.30 | 156.70 | 19.15 |
| 266 | 150.19 | 159.85 | 42.25 | 3.30 | 156.70 | 19.15 |
| 267 | 170.42 | 117.30 | 77.47 | 43.67 | 114.16 | 8.17 |
| 268 | 170.42 | 117.30 | 77.47 | 43.67 | 114.16 | 8.17 |
| 269 | 201.92 | 100.87 | 73.83 | 109.48 | 104.02 | 50.73 |
| 270 | 201.92 | 100.87 | 73.83 | 109.48 | 104.02 | 50.73 |
| 271 | 68.44 | 75.76 | 40.40 | 39.70 | 78.91 | 28.90 |
| 272 | 68.44 | 75.76 | 40.40 | 39.70 | 78.91 | 28.90 |
| 273 | 198.84 | 37.56 | 65.82 | 72.49 | 40.71 | 42.72 |
| 274 | 198.84 | 37.56 | 65.82 | 72.49 | 40.71 | 42.72 |
| 275 | 121.76 | 80.11 | 53.90 | 25.51 | 83.25 | 15.40 |
| 276 | 121.76 | 80.11 | 53.90 | 25.51 | 83.25 | 15.40 |
| 277 | 126.48 | 135.09 | 35.49 | 16.30 | 131.32 | 21.63 |
| 278 | 108.54 | 39.97 | 39.93 | 56.48 | 43.74 | 26.07 |
| 279 | 35.73 | 150.16 | 33.05 | 25.57 | 146.38 | 8.53 |
| 280 | 53.67 | 24.91 | 28.60 | 14.61 | 28.68 | 12.98 |
| 281 | 124.64 | 173.08 | 30.68 | 5.90 | 169.30 | 16.82 |
| 282 | 106.70 | 1.99 | 35.13 | 34.28 | 5.76 | 21.27 |
| 283 | 67.72 | 147.55 | 41.15 | 34.08 | 143.78 | 0.43 |
| 284 | 85.66 | 27.51 | 36.70 | 6.10 | 31.29 | 4.88 |
| 285 | 17.88 | 35.00 | 11.31 | 4.01 | 31.23 | 2.55 |
| 286 | 47.03 | 153.97 | 11.31 | 25.14 | 150.19 | 2.55 |
| 287 | 144.33 | 50.06 | 57.23 | 37.86 | 46.29 | 15.65 |
| 288 | 115.19 | 169.03 | 57.23 | 67.00 | 165.26 | 15.65 |
| 289 | 16.04 | 72.98 | 6.50 | 18.18 | 69.21 | 7.36 |
| 290 | 45.19 | 191.95 | 6.50 | 47.33 | 188.18 | 7.36 |
| 291 | 176.32 | 47.46 | 65.33 | 46.37 | 43.68 | 23.75 |
| 292 | 147.18 | 166.43 | 65.33 | 75.51 | 162.65 | 23.75 |
| 293 | 47.69 | 160.08 | 11.84 | 23.14 | 156.31 | 2.02 |
| 294 | 47.69 | 160.08 | 11.84 | 23.14 | 156.31 | 2.02 |
| 295 | 114.52 | 175.15 | 56.70 | 65.00 | 171.38 | 15.12 |
| 296 | 114.52 | 175.15 | 56.70 | 65.00 | 171.38 | 15.12 |
| 297 | 45.85 | 198.07 | 7.03 | 45.33 | 194.30 | 6.83 |
| 298 | 45.85 | 198.07 | 7.03 | 45.33 | 194.30 | 6.83 |
| 299 | 146.51 | 172.54 | 64.80 | 73.51 | 168.77 | 23.22 |
| 300 | 146.51 | 172.54 | 64.80 | 73.51 | 168.77 | 23.22 |
| 301 | 107.88 | 46.09 | 39.40 | 54.48 | 49.86 | 25.54 |
| 302 | 107.88 | 46.09 | 39.40 | 54.48 | 49.86 | 25.54 |
| 303 | 54.33 | 31.02 | 29.14 | 12.61 | 34.79 | 12.44 |
| 304 | 54.33 | 31.02 | 29.14 | 12.61 | 34.79 | 12.44 |
| 305 | 106.04 | 8.10 | 34.59 | 32.28 | 11.87 | 20.73 |
| 306 | 106.04 | 8.10 | 34.59 | 32.28 | 11.87 | 20.73 |
| 307 | 86.33 | 33.63 | 37.24 | 4.10 | 37.40 | 4.34 |
| 308 | 86.33 | 33.63 | 37.24 | 4.10 | 37.40 | 4.34 |
| 309 | 131.32 | 130.22 | 37.22 | 20.13 | 127.08 | 23.36 |
| 310 | 113.38 | 44.84 | 41.67 | 60.31 | 47.98 | 27.81 |
| 311 | 30.90 | 145.29 | 31.32 | 21.73 | 142.15 | 10.26 |
| 312 | 48.83 | 29.77 | 26.87 | 18.45 | 32.92 | 14.71 |
| 313 | 129.48 | 168.21 | 32.41 | 2.06 | 165.06 | 18.55 |
| 314 | 111.54 | 6.85 | 36.86 | 38.12 | 10.00 | 23.00 |
| 315 | 62.89 | 142.68 | 39.42 | 30.25 | 139.54 | 2.16 |
| 316 | 80.83 | 32.38 | 34.97 | 9.93 | 35.52 | 6.61 |
| 317 | 22.72 | 30.13 | 13.04 | 7.85 | 26.99 | 0.82 |
| 318 | 51.87 | 149.10 | 13.04 | 21.30 | 145.96 | 0.82 |
| 319 | 139.50 | 45.20 | 55.49 | 34.02 | 42.05 | 13.91 |
| 320 | 110.35 | 164.16 | 55.49 | 63.17 | 161.02 | 13.91 |
| 321 | 20.88 | 68.11 | 8.24 | 14.35 | 64.97 | 5.62 |
| 322 | 50.02 | 187.08 | 8.24 | 43.49 | 183.94 | 5.62 |
| 323 | 171.49 | 42.59 | 63.59 | 42.53 | 39.44 | 22.01 |
| 324 | 142.34 | 161.56 | 63.59 | 71.68 | 158.41 | 22.01 |
| 325 | 52.53 | 155.21 | 13.58 | 19.30 | 152.07 | 0.28 |
| 326 | 52.53 | 155.21 | 13.58 | 19.30 | 152.07 | 0.28 |
| 327 | 109.69 | 170.28 | 54.96 | 61.17 | 167.14 | 13.38 |
| 328 | 109.69 | 170.28 | 54.96 | 61.17 | 167.14 | 13.38 |
| 329 | 50.69 | 193.20 | 8.77 | 41.50 | 190.06 | 5.09 |
| 330 | 50.69 | 193.20 | 8.77 | 41.50 | 190.06 | 5.09 |
| 331 | 141.68 | 167.67 | 63.06 | 69.68 | 164.53 | 21.48 |
| 332 | 141.68 | 167.67 | 63.06 | 69.68 | 164.53 | 21.48 |
| 333 | 112.72 | 50.96 | 41.14 | 58.31 | 54.10 | 27.28 |
| 334 | 112.72 | 50.96 | 41.14 | 58.31 | 54.10 | 27.28 |
| 335 | 49.50 | 35.89 | 27.40 | 16.45 | 39.03 | 14.18 |
| 336 | 49.50 | 35.89 | 27.40 | 16.45 | 39.03 | 14.18 |
| 337 | 110.87 | 12.97 | 36.33 | 36.12 | 16.11 | 22.47 |
| 338 | 110.87 | 12.97 | 36.33 | 36.12 | 16.11 | 22.47 |
| 339 | 81.49 | 38.50 | 35.50 | 7.94 | 41.64 | 6.08 |
| 340 | 81.49 | 38.50 | 35.50 | 7.94 | 41.64 | 6.08 |
| 341 | 5.75 | 29.89 | 1.58 | 13.54 | 26.11 | 1.58 |
| 342 | 61.02 | 33.52 | 18.17 | 29.87 | 29.75 | 18.17 |
| 343 | 5.75 | 29.89 | 1.58 | 13.54 | 26.11 | 1.58 |
| 344 | 61.02 | 33.52 | 18.17 | 29.87 | 29.75 | 18.17 |
| 345 | 30.31 | 72.09 | 13.46 | 36.91 | 68.31 | 13.46 |
| 346 | 97.07 | 75.72 | 30.05 | 53.24 | 71.95 | 30.05 |
| 347 | 12.46 | 144.92 | 13.46 | 54.76 | 141.15 | 13.46 |
| 348 | 79.23 | 148.56 | 30.05 | 71.09 | 144.79 | 30.05 |
| 349 | 12.18 | 147.54 | 13.23 | 53.90 | 143.77 | 13.23 |
| 350 | 78.94 | 151.18 | 29.83 | 70.23 | 147.41 | 29.83 |
| 351 | 12.18 | 147.54 | 13.23 | 53.90 | 143.77 | 13.23 |
| 352 | 78.94 | 151.18 | 29.83 | 70.23 | 147.41 | 29.83 |
| 353 | 5.46 | 27.27 | 1.80 | 14.40 | 23.49 | 1.80 |
| 354 | 61.30 | 30.90 | 18.40 | 30.73 | 27.13 | 18.40 |
| 355 | 5.46 | 27.27 | 1.80 | 14.40 | 23.49 | 1.80 |
| 356 | 61.30 | 30.90 | 18.40 | 30.73 | 27.13 | 18.40 |
| 357 | 10.38 | 24.60 | 0.08 | 9.88 | 21.46 | 0.08 |
| 358 | 56.38 | 28.24 | 16.51 | 26.21 | 25.10 | 16.51 |
| 359 | 10.38 | 24.60 | 0.08 | 9.88 | 21.46 | 0.08 |
| 360 | 56.38 | 28.24 | 16.51 | 26.21 | 25.10 | 16.51 |
| 361 | 19.66 | 59.77 | 9.82 | 29.36 | 56.62 | 9.82 |
| 362 | 86.43 | 63.41 | 26.42 | 45.69 | 60.26 | 26.42 |
| 363 | 4.79 | 120.47 | 9.82 | 44.23 | 117.32 | 9.82 |
| 364 | 71.55 | 124.10 | 26.42 | 60.56 | 120.96 | 26.42 |
| 365 | 4.55 | 122.65 | 9.63 | 43.52 | 119.51 | 9.63 |
| 366 | 71.32 | 126.29 | 26.22 | 59.84 | 123.15 | 26.22 |
| 367 | 4.55 | 122.65 | 9.63 | 43.52 | 119.51 | 9.63 |
| 368 | 71.32 | 126.29 | 26.22 | 59.84 | 123.15 | 26.22 |
| 369 | 10.15 | 22.42 | 0.11 | 10.60 | 19.28 | 0.11 |
| 370 | 56.62 | 26.06 | 16.70 | 26.93 | 22.91 | 16.70 |
| 371 | 10.15 | 22.42 | 0.11 | 10.60 | 19.28 | 0.11 |
| 372 | 56.62 | 26.06 | 16.70 | 26.93 | 22.91 | 16.70 |

Considering the strength after web buckling, the strength calculation control combination number is 249, M=217.25, N=141.78, M=33.70, N=-138.64

The calculated stress ratio after buckling was equal to 0.736

Shear strength calculation control combination number :228, V=-79.57

The calculated stress ratio of shear strength =0.199

The corresponding combination number of the maximum stress in the plane stability calculation is 249, M=217.25, N=141.78, M=33.70, N=-138.64

The maximum stress calculated by in-plane stability (N/mm*mm) =231.37

The calculated maximum stress ratio in plane stability is 0.759

The corresponding combination number of maximum stress calculated for stability outside the plane :171, M=-144.94, N=239.71, M=-86.46, N=-235.94

The maximum stress ratio calculated out of plane stability =0.901

Door gauge gb51022-2015 allowable thickness ratio of web [H0/TW] =250.00

Flange tolerance width ratio [B/T] =12.38

After buckling, the calculated stress ratio of strength was 0.736 < 1.0

The calculated stress ratio of shear strength =0.199 < 1.0

The calculated maximum stress in plane stability is < f=305.00

The maximum stress ratio calculated for stability outside the plane is < 1.0

Web height/thickness ratio H0/TW=63.33 < [H0/TW]=250.00

Flange width ratio B/T =12.20 < [B/T]=12.38

Compression bar, the internal length - to - thin ratio of plane lambda =56. ≤ [lambda]=180

Pressure bar, plane length of the slender ratio of lambda =84. ≤ [lambda]=180

Component weight (Kg)=285.74

1. Steel column 2     The design results

Section type =16;   Setting Angle =0;   Calculated length: Lx=9.68, Ly=5.00;   Length ratio: lambda x=56.0, lambda y=83.6

Component length =5.00;   Calculate length coefficient: Ux=1.94 Uy=1.00

Seismic rating: grade 3

Cross section parameters: B1=250, B2=250, H=400, Tw=6, T1=10, T2=10

Classification of axial compression section: X-axis :b, Y-axis :c

Steel number of component: Q345

Check calculation specification: door gb51022-2015

| \ | I end |  |  | II end |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| combination | M | N | V | M | N | V |
| 1 | 58.53 | 48.81 | 21.03 | 46.61 | 44.57 | 21.03 |
| 2 | 30.69 | 36.94 | 10.96 | 24.14 | 32.69 | 10.96 |
| 3 | 58.06 | 53.10 | 20.65 | 45.21 | 48.85 | 20.65 |
| 4 | 31.15 | 32.66 | 11.34 | 25.54 | 28.41 | 11.34 |
| 5 | 65.99 | 52.00 | 23.72 | 52.60 | 48.23 | 23.72 |
| 6 | 26.22 | 35.03 | 9.34 | 20.50 | 31.26 | 9.34 |
| 7 | 65.32 | 58.12 | 23.19 | 50.60 | 54.34 | 23.19 |
| 8 | 26.88 | 28.92 | 9.88 | 22.50 | 25.15 | 9.88 |
| 9 | 61.15 | 47.13 | 21.98 | 48.77 | 43.99 | 21.98 |
| 10 | 21.38 | 30.17 | 7.61 | 16.67 | 27.02 | 7.61 |
| 11 | 60.49 | 53.25 | 21.45 | 46.77 | 50.11 | 21.45 |
| 12 | 22.04 | 24.05 | 8.14 | 18.67 | 20.91 | 8.14 |
| 13 | 16.90 | 75.49 | 4.51 | 63.39 | 79.26 | 27.61 |
| 14 | 215.00 | 69.09 | 74.06 | 97.53 | 72.86 | 50.96 |
| 15 | 53.80 | 76.15 | 13.94 | 41.83 | 79.92 | 9.16 |
| 16 | 213.66 | 33.61 | 67.85 | 67.82 | 37.38 | 44.75 |
| 17 | 21.74 | 80.36 | 6.24 | 67.23 | 83.50 | 29.34 |
| 18 | 219.84 | 73.96 | 75.79 | 101.37 | 77.10 | 52.69 |
| 19 | 48.96 | 81.02 | 12.21 | 45.66 | 84.16 | 10.89 |
| 20 | 218.49 | 38.47 | 69.58 | 71.66 | 41.62 | 46.48 |
| 21 | 38.43 | 10.82 | 14.77 | 0.75 | 14.59 | 0.91 |
| 22 | 80.43 | 6.98 | 26.96 | 19.73 | 10.75 | 13.10 |
| 23 | 80.85 | 11.21 | 25.84 | 13.69 | 14.99 | 11.98 |
| 24 | 79.62 | 14.31 | 23.23 | 1.90 | 10.54 | 9.37 |
| 25 | 1.34 | 27.78 | 0.39 | 31.35 | 31.56 | 13.47 |
| 26 | 120.20 | 23.95 | 41.33 | 51.83 | 27.72 | 27.47 |
| 27 | 41.08 | 28.18 | 11.47 | 18.41 | 31.95 | 2.39 |
| 28 | 119.39 | 2.65 | 37.61 | 34.00 | 6.43 | 23.75 |
| 29 | 37.77 | 4.70 | 14.23 | 1.25 | 8.47 | 0.37 |
| 30 | 81.09 | 0.86 | 27.49 | 21.73 | 4.63 | 13.63 |
| 31 | 80.19 | 5.10 | 25.31 | 11.69 | 8.87 | 11.45 |
| 32 | 80.28 | 20.43 | 23.77 | 3.90 | 16.66 | 9.91 |
| 33 | 0.68 | 33.90 | 0.93 | 29.35 | 37.67 | 12.93 |
| 34 | 119.53 | 30.06 | 40.80 | 49.83 | 33.83 | 26.94 |
| 35 | 41.75 | 34.30 | 12.00 | 16.41 | 38.07 | 1.86 |
| 36 | 118.73 | 8.77 | 37.08 | 32.00 | 12.54 | 23.22 |
| 37 | 33.59 | 15.68 | 13.03 | 3.08 | 18.83 | 0.83 |
| 38 | 85.26 | 11.85 | 28.70 | 23.57 | 14.99 | 14.84 |
| 39 | 76.02 | 16.08 | 24.10 | 9.86 | 19.22 | 10.24 |
| 40 | 84.46 | 9.45 | 24.97 | 5.74 | 6.30 | 11.11 |
| 41 | 6.18 | 32.65 | 1.34 | 35.18 | 35.80 | 15.20 |
| 42 | 125.03 | 28.81 | 43.07 | 55.67 | 31.96 | 29.21 |
| 43 | 36.25 | 33.05 | 9.73 | 22.24 | 36.19 | 4.13 |
| 44 | 124.23 | 7.52 | 39.34 | 37.84 | 10.67 | 25.48 |
| 45 | 32.93 | 9.57 | 12.50 | 5.08 | 12.71 | 1.36 |
| 46 | 85.93 | 5.73 | 29.23 | 25.57 | 8.87 | 15.37 |
| 47 | 75.35 | 9.97 | 23.57 | 7.86 | 13.11 | 9.71 |
| 48 | 85.12 | 15.56 | 25.50 | 7.74 | 12.42 | 11.64 |
| 49 | 5.51 | 38.77 | 0.81 | 33.18 | 41.91 | 14.67 |
| 50 | 124.37 | 34.93 | 42.54 | 53.67 | 38.07 | 28.68 |
| 51 | 36.91 | 39.16 | 10.26 | 20.24 | 42.31 | 3.60 |
| 52 | 123.57 | 13.64 | 38.81 | 35.84 | 16.78 | 24.95 |
| 53 | 8.97 | 59.53 | 4.81 | 42.69 | 63.30 | 18.29 |
| 54 | 189.12 | 53.13 | 64.74 | 76.83 | 56.91 | 41.64 |
| 55 | 79.68 | 60.20 | 23.26 | 21.12 | 63.97 | 0.16 |
| 56 | 187.78 | 17.65 | 58.53 | 47.12 | 21.42 | 35.43 |
| 57 | 18.87 | 71.41 | 5.26 | 65.16 | 75.18 | 28.36 |
| 58 | 216.96 | 65.01 | 74.80 | 99.30 | 68.78 | 51.70 |
| 59 | 51.84 | 72.07 | 13.20 | 43.59 | 75.84 | 9.90 |
| 60 | 215.62 | 29.53 | 68.59 | 69.59 | 33.30 | 45.49 |
| 61 | 8.51 | 55.25 | 4.43 | 44.09 | 59.02 | 18.67 |
| 62 | 189.59 | 48.85 | 65.11 | 78.23 | 52.63 | 42.01 |
| 63 | 79.21 | 55.91 | 22.89 | 22.52 | 59.69 | 0.21 |
| 64 | 188.25 | 13.37 | 58.90 | 48.52 | 17.14 | 35.80 |
| 65 | 18.41 | 75.69 | 4.88 | 63.76 | 79.46 | 27.98 |
| 66 | 216.50 | 69.29 | 74.43 | 97.90 | 73.06 | 51.33 |
| 67 | 52.30 | 76.35 | 13.57 | 42.19 | 80.13 | 9.53 |
| 68 | 215.16 | 33.81 | 68.22 | 68.19 | 37.58 | 45.12 |
| 69 | 4.13 | 64.40 | 3.07 | 46.52 | 67.54 | 20.03 |
| 70 | 193.96 | 58.00 | 66.47 | 80.66 | 61.15 | 43.37 |
| 71 | 74.84 | 65.06 | 21.53 | 24.96 | 68.21 | 1.57 |
| 72 | 192.62 | 22.52 | 60.26 | 50.95 | 25.66 | 37.16 |
| 73 | 23.71 | 76.28 | 6.99 | 68.99 | 79.42 | 30.09 |
| 74 | 221.80 | 69.88 | 76.54 | 103.13 | 73.02 | 53.44 |
| 75 | 47.00 | 76.94 | 11.46 | 47.43 | 80.08 | 11.64 |
| 76 | 220.46 | 34.40 | 70.33 | 73.42 | 37.54 | 47.23 |
| 77 | 3.67 | 60.12 | 2.70 | 47.92 | 63.26 | 20.40 |
| 78 | 194.42 | 53.72 | 66.85 | 82.06 | 56.87 | 43.75 |
| 79 | 74.38 | 60.78 | 21.15 | 26.36 | 63.93 | 1.95 |
| 80 | 193.08 | 18.24 | 60.64 | 52.35 | 21.38 | 37.54 |
| 81 | 23.24 | 80.56 | 6.62 | 67.59 | 83.70 | 29.72 |
| 82 | 221.34 | 74.16 | 76.16 | 101.73 | 77.30 | 53.06 |
| 83 | 47.46 | 81.22 | 11.84 | 46.03 | 84.36 | 11.26 |
| 84 | 219.99 | 38.68 | 69.95 | 72.02 | 41.82 | 46.85 |
| 85 | 78.06 | 40.80 | 24.33 | 43.63 | 37.02 | 24.33 |
| 86 | 11.30 | 37.16 | 7.74 | 27.30 | 33.39 | 7.74 |
| 87 | 61.02 | 33.52 | 18.17 | 29.87 | 29.75 | 18.17 |
| 88 | 5.75 | 29.89 | 1.58 | 13.54 | 26.11 | 1.58 |
| 89 | 77.78 | 43.42 | 24.10 | 42.77 | 39.64 | 24.10 |
| 90 | 11.01 | 39.78 | 7.51 | 26.44 | 36.01 | 7.51 |
| 91 | 61.30 | 30.90 | 18.40 | 30.73 | 27.13 | 18.40 |
| 92 | 5.46 | 27.27 | 1.80 | 14.40 | 23.49 | 1.80 |
| 93 | 70.58 | 34.30 | 21.64 | 37.68 | 31.16 | 21.64 |
| 94 | 3.82 | 30.66 | 5.05 | 21.35 | 27.52 | 5.05 |
| 95 | 56.38 | 28.24 | 16.51 | 26.21 | 25.10 | 16.51 |
| 96 | 10.38 | 24.60 | 0.08 | 9.88 | 21.46 | 0.08 |
| 97 | 70.35 | 36.48 | 21.45 | 36.96 | 33.34 | 21.45 |
| 98 | 3.58 | 32.85 | 4.86 | 20.63 | 29.70 | 4.86 |
| 99 | 56.62 | 26.06 | 16.70 | 26.93 | 22.91 | 16.70 |
| 100 | 10.15 | 22.42 | 0.11 | 10.60 | 19.28 | 0.11 |
| 101 | 101.73 | 112.07 | 30.77 | 52.12 | 108.30 | 30.77 |
| 102 | 3.40 | 279.30 | 16.76 | 80.42 | 275.53 | 16.76 |
| 103 | 60.09 | 282.02 | 30.77 | 93.75 | 278.25 | 30.77 |
| 104 | 60.09 | 282.02 | 30.77 | 93.75 | 278.25 | 30.77 |
| 105 | 29.02 | 29.21 | 10.41 | 23.02 | 25.44 | 10.41 |
| 106 | 29.02 | 29.21 | 10.41 | 23.02 | 25.44 | 10.41 |
| 107 | 60.09 | 282.02 | 30.77 | 93.75 | 278.25 | 30.77 |
| 108 | 29.02 | 29.21 | 10.41 | 23.02 | 25.44 | 10.41 |
| 109 | 96.89 | 107.20 | 29.03 | 48.28 | 104.06 | 29.03 |
| 110 | 1.44 | 274.43 | 15.03 | 76.59 | 271.29 | 15.03 |
| 111 | 55.25 | 277.16 | 29.03 | 89.92 | 274.01 | 29.03 |
| 112 | 55.25 | 277.16 | 29.03 | 89.92 | 274.01 | 29.03 |
| 113 | 24.19 | 24.34 | 8.67 | 19.19 | 21.20 | 8.67 |
| 114 | 24.19 | 24.34 | 8.67 | 19.19 | 21.20 | 8.67 |
| 115 | 55.25 | 277.16 | 29.03 | 89.92 | 274.01 | 29.03 |
| 116 | 24.19 | 24.34 | 8.67 | 19.19 | 21.20 | 8.67 |
| 117 | 127.60 | 128.02 | 40.08 | 72.82 | 124.25 | 40.08 |
| 118 | 85.96 | 297.98 | 40.08 | 114.46 | 294.21 | 40.08 |
| 119 | 1.43 | 283.38 | 16.02 | 78.66 | 279.60 | 16.02 |
| 120 | 27.06 | 33.29 | 9.66 | 21.26 | 29.51 | 9.66 |
| 121 | 85.50 | 302.26 | 39.71 | 113.06 | 298.49 | 39.71 |
| 122 | 85.50 | 302.26 | 39.71 | 113.06 | 298.49 | 39.71 |
| 123 | 27.52 | 29.01 | 10.04 | 22.66 | 25.23 | 10.04 |
| 124 | 27.52 | 29.01 | 10.04 | 22.66 | 25.23 | 10.04 |
| 125 | 122.76 | 123.15 | 38.35 | 68.99 | 120.01 | 38.35 |
| 126 | 81.12 | 293.11 | 38.35 | 110.62 | 289.97 | 38.35 |
| 127 | 3.40 | 278.51 | 14.28 | 74.82 | 275.36 | 14.28 |
| 128 | 22.22 | 28.42 | 7.93 | 17.42 | 25.27 | 7.93 |
| 129 | 80.66 | 297.39 | 37.98 | 109.23 | 294.25 | 37.98 |
| 130 | 80.66 | 297.39 | 37.98 | 109.23 | 294.25 | 37.98 |
| 131 | 22.69 | 24.14 | 8.30 | 18.82 | 20.99 | 8.30 |
| 132 | 22.69 | 24.14 | 8.30 | 18.82 | 20.99 | 8.30 |
| 133 | 116.88 | 110.00 | 37.97 | 72.97 | 106.23 | 37.97 |
| 134 | 87.73 | 228.97 | 37.97 | 102.12 | 225.20 | 37.97 |
| 135 | 8.28 | 210.10 | 13.79 | 60.68 | 206.32 | 13.79 |
| 136 | 26.22 | 35.03 | 9.34 | 20.50 | 31.26 | 9.34 |
| 137 | 87.07 | 235.09 | 37.44 | 100.12 | 231.32 | 37.44 |
| 138 | 87.07 | 235.09 | 37.44 | 100.12 | 231.32 | 37.44 |
| 139 | 26.88 | 28.92 | 9.88 | 22.50 | 25.15 | 9.88 |
| 140 | 26.88 | 28.92 | 9.88 | 22.50 | 25.15 | 9.88 |
| 141 | 112.04 | 105.13 | 36.23 | 69.13 | 101.99 | 36.23 |
| 142 | 82.89 | 224.10 | 36.23 | 98.28 | 220.96 | 36.23 |
| 143 | 3.44 | 205.23 | 12.06 | 56.85 | 202.08 | 12.06 |
| 144 | 21.38 | 30.17 | 7.61 | 16.67 | 27.02 | 7.61 |
| 145 | 82.23 | 230.22 | 35.70 | 96.28 | 227.08 | 35.70 |
| 146 | 82.23 | 230.22 | 35.70 | 96.28 | 227.08 | 35.70 |
| 147 | 22.04 | 24.05 | 8.14 | 18.67 | 20.91 | 8.14 |
| 148 | 22.04 | 24.05 | 8.14 | 18.67 | 20.91 | 8.14 |
| 149 | 100.04 | 65.20 | 31.13 | 20.97 | 61.43 | 17.27 |
| 150 | 58.40 | 235.16 | 31.13 | 62.61 | 231.39 | 17.27 |
| 151 | 18.81 | 69.04 | 10.59 | 0.49 | 65.27 | 3.27 |
| 152 | 60.45 | 239.00 | 10.59 | 42.13 | 235.23 | 3.27 |
| 153 | 142.47 | 64.81 | 42.21 | 33.91 | 61.04 | 28.35 |
| 154 | 100.83 | 234.76 | 42.21 | 75.55 | 230.99 | 28.35 |
| 155 | 18.01 | 90.33 | 6.87 | 18.32 | 86.56 | 6.99 |
| 156 | 59.65 | 260.29 | 6.87 | 59.95 | 256.52 | 6.99 |
| 157 | 26.12 | 220.56 | 7.07 | 26.81 | 216.79 | 6.79 |
| 158 | 0.50 | 29.53 | 0.71 | 30.59 | 33.30 | 13.15 |
| 159 | 144.98 | 224.40 | 34.66 | 6.33 | 220.62 | 20.80 |
| 160 | 119.35 | 25.69 | 41.02 | 51.07 | 29.46 | 27.16 |
| 161 | 16.30 | 220.16 | 18.14 | 39.75 | 216.39 | 4.28 |
| 162 | 41.93 | 29.93 | 11.79 | 17.65 | 33.70 | 2.07 |
| 163 | 144.18 | 245.69 | 30.93 | 24.15 | 241.92 | 17.07 |
| 164 | 118.55 | 4.40 | 37.29 | 33.25 | 8.17 | 23.43 |
| 165 | 57.94 | 239.44 | 30.76 | 61.21 | 235.67 | 16.90 |
| 166 | 57.94 | 239.44 | 30.76 | 61.21 | 235.67 | 16.90 |
| 167 | 60.92 | 243.28 | 10.97 | 40.73 | 239.51 | 2.89 |
| 168 | 60.92 | 243.28 | 10.97 | 40.73 | 239.51 | 2.89 |
| 169 | 100.36 | 239.04 | 41.83 | 74.15 | 235.27 | 27.97 |
| 170 | 100.36 | 239.04 | 41.83 | 74.15 | 235.27 | 27.97 |
| 171 | 60.11 | 264.57 | 7.24 | 58.55 | 260.80 | 6.62 |
| 172 | 60.11 | 264.57 | 7.24 | 58.55 | 260.80 | 6.62 |
| 173 | 0.03 | 33.81 | 1.09 | 29.19 | 37.58 | 12.77 |
| 174 | 0.03 | 33.81 | 1.09 | 29.19 | 37.58 | 12.77 |
| 175 | 118.89 | 29.97 | 40.64 | 49.67 | 33.75 | 26.78 |
| 176 | 118.89 | 29.97 | 40.64 | 49.67 | 33.75 | 26.78 |
| 177 | 42.39 | 34.21 | 12.16 | 16.25 | 37.98 | 1.70 |
| 178 | 42.39 | 34.21 | 12.16 | 16.25 | 37.98 | 1.70 |
| 179 | 118.09 | 8.68 | 36.92 | 31.85 | 12.45 | 23.06 |
| 180 | 118.09 | 8.68 | 36.92 | 31.85 | 12.45 | 23.06 |
| 181 | 95.21 | 60.34 | 29.40 | 17.14 | 57.19 | 15.54 |
| 182 | 53.57 | 230.29 | 29.40 | 58.77 | 227.15 | 15.54 |
| 183 | 23.65 | 64.18 | 12.33 | 3.35 | 61.03 | 1.53 |
| 184 | 65.29 | 234.13 | 12.33 | 38.29 | 230.99 | 1.53 |
| 185 | 137.63 | 59.94 | 40.47 | 30.08 | 56.80 | 26.61 |
| 186 | 95.99 | 229.90 | 40.47 | 71.71 | 226.75 | 26.61 |
| 187 | 22.84 | 85.47 | 8.60 | 14.48 | 82.32 | 5.26 |
| 188 | 64.48 | 255.42 | 8.60 | 56.12 | 252.28 | 5.26 |
| 189 | 30.96 | 215.69 | 5.33 | 22.97 | 212.55 | 8.53 |
| 190 | 5.34 | 34.40 | 1.02 | 34.43 | 37.54 | 14.88 |
| 191 | 149.82 | 219.53 | 36.40 | 2.49 | 216.39 | 22.54 |
| 192 | 124.19 | 30.56 | 42.75 | 54.91 | 33.70 | 28.89 |
| 193 | 11.46 | 215.29 | 16.41 | 35.91 | 212.15 | 2.55 |
| 194 | 37.09 | 34.80 | 10.05 | 21.49 | 37.94 | 3.81 |
| 195 | 149.01 | 240.82 | 32.67 | 20.32 | 237.68 | 18.81 |
| 196 | 123.39 | 9.27 | 39.02 | 37.08 | 12.41 | 25.16 |
| 197 | 53.10 | 234.57 | 29.03 | 57.38 | 231.43 | 15.17 |
| 198 | 53.10 | 234.57 | 29.03 | 57.38 | 231.43 | 15.17 |
| 199 | 65.75 | 238.41 | 12.70 | 36.89 | 235.27 | 1.16 |
| 200 | 65.75 | 238.41 | 12.70 | 36.89 | 235.27 | 1.16 |
| 201 | 95.53 | 234.18 | 40.10 | 70.31 | 231.03 | 26.24 |
| 202 | 95.53 | 234.18 | 40.10 | 70.31 | 231.03 | 26.24 |
| 203 | 64.95 | 259.70 | 8.98 | 54.72 | 256.56 | 4.88 |
| 204 | 64.95 | 259.70 | 8.98 | 54.72 | 256.56 | 4.88 |
| 205 | 4.87 | 38.68 | 0.65 | 33.03 | 41.82 | 14.51 |
| 206 | 4.87 | 38.68 | 0.65 | 33.03 | 41.82 | 14.51 |
| 207 | 123.73 | 34.84 | 42.38 | 53.51 | 37.98 | 28.52 |
| 208 | 123.73 | 34.84 | 42.38 | 53.51 | 37.98 | 28.52 |
| 209 | 37.55 | 39.08 | 10.42 | 20.09 | 42.22 | 3.44 |
| 210 | 37.55 | 39.08 | 10.42 | 20.09 | 42.22 | 3.44 |
| 211 | 122.92 | 13.55 | 38.65 | 35.69 | 16.69 | 24.79 |
| 212 | 122.92 | 13.55 | 38.65 | 35.69 | 16.69 | 24.79 |
| 213 | 59.86 | 1.53 | 19.06 | 22.32 | 5.30 | 4.04 |
| 214 | 30.71 | 117.44 | 19.06 | 6.83 | 113.67 | 4.04 |
| 215 | 138.23 | 4.87 | 50.49 | 56.46 | 1.09 | 27.39 |
| 216 | 167.38 | 123.84 | 50.49 | 27.31 | 120.06 | 27.39 |
| 217 | 130.57 | 2.19 | 37.51 | 0.76 | 5.97 | 14.41 |
| 218 | 101.42 | 116.78 | 37.51 | 28.39 | 113.00 | 14.41 |
| 219 | 136.89 | 40.35 | 44.28 | 26.75 | 36.58 | 21.18 |
| 220 | 166.04 | 159.32 | 44.28 | 2.40 | 155.55 | 21.18 |
| 221 | 36.81 | 103.65 | 0.81 | 24.98 | 99.88 | 23.91 |
| 222 | 18.87 | 71.41 | 5.26 | 65.16 | 75.18 | 28.36 |
| 223 | 234.90 | 110.05 | 70.35 | 59.12 | 106.28 | 47.25 |
| 224 | 216.96 | 65.01 | 74.80 | 99.30 | 68.78 | 51.70 |
| 225 | 33.90 | 102.99 | 17.65 | 3.41 | 99.22 | 5.45 |
| 226 | 51.84 | 72.07 | 13.20 | 43.59 | 75.84 | 9.90 |
| 227 | 233.56 | 145.53 | 64.14 | 29.41 | 141.76 | 41.04 |
| 228 | 215.62 | 29.53 | 68.59 | 69.59 | 33.30 | 45.49 |
| 229 | 30.25 | 121.72 | 18.69 | 5.43 | 117.95 | 4.41 |
| 230 | 30.25 | 121.72 | 18.69 | 5.43 | 117.95 | 4.41 |
| 231 | 167.84 | 128.12 | 50.86 | 28.71 | 124.35 | 27.76 |
| 232 | 167.84 | 128.12 | 50.86 | 28.71 | 124.35 | 27.76 |
| 233 | 100.96 | 121.06 | 37.14 | 26.99 | 117.28 | 14.04 |
| 234 | 100.96 | 121.06 | 37.14 | 26.99 | 117.28 | 14.04 |
| 235 | 166.50 | 163.60 | 44.65 | 1.00 | 159.83 | 21.55 |
| 236 | 166.50 | 163.60 | 44.65 | 1.00 | 159.83 | 21.55 |
| 237 | 18.41 | 75.69 | 4.88 | 63.76 | 79.46 | 27.98 |
| 238 | 18.41 | 75.69 | 4.88 | 63.76 | 79.46 | 27.98 |
| 239 | 216.50 | 69.29 | 74.43 | 97.90 | 73.06 | 51.33 |
| 240 | 216.50 | 69.29 | 74.43 | 97.90 | 73.06 | 51.33 |
| 241 | 52.30 | 76.35 | 13.57 | 42.19 | 80.13 | 9.53 |
| 242 | 52.30 | 76.35 | 13.57 | 42.19 | 80.13 | 9.53 |
| 243 | 215.16 | 33.81 | 68.22 | 68.19 | 37.58 | 45.12 |
| 244 | 215.16 | 33.81 | 68.22 | 68.19 | 37.58 | 45.12 |
| 245 | 55.02 | 6.40 | 17.32 | 26.16 | 9.54 | 5.78 |
| 246 | 25.88 | 112.57 | 17.32 | 2.99 | 109.43 | 5.78 |
| 247 | 143.07 | 0.00 | 52.22 | 60.30 | 3.14 | 29.12 |
| 248 | 172.22 | 118.97 | 52.22 | 31.15 | 115.83 | 29.12 |
| 249 | 125.73 | 7.06 | 35.78 | 4.59 | 10.21 | 12.68 |
| 250 | 96.58 | 111.91 | 35.78 | 24.55 | 108.76 | 12.68 |
| 251 | 141.73 | 35.48 | 46.01 | 30.59 | 32.34 | 22.91 |
| 252 | 170.87 | 154.45 | 46.01 | 1.44 | 151.31 | 22.91 |
| 253 | 41.65 | 98.78 | 2.54 | 28.81 | 95.64 | 25.64 |
| 254 | 23.71 | 76.28 | 6.99 | 68.99 | 79.42 | 30.09 |
| 255 | 239.74 | 105.18 | 72.09 | 62.95 | 102.04 | 48.99 |
| 256 | 221.80 | 69.88 | 76.54 | 103.13 | 73.02 | 53.44 |
| 257 | 29.06 | 98.12 | 15.91 | 7.25 | 94.98 | 7.19 |
| 258 | 47.00 | 76.94 | 11.46 | 47.43 | 80.08 | 11.64 |
| 259 | 238.40 | 140.67 | 65.88 | 33.24 | 137.52 | 42.78 |
| 260 | 220.46 | 34.40 | 70.33 | 73.42 | 37.54 | 47.23 |
| 261 | 25.41 | 116.85 | 16.95 | 1.59 | 113.71 | 6.15 |
| 262 | 25.41 | 116.85 | 16.95 | 1.59 | 113.71 | 6.15 |
| 263 | 172.68 | 123.25 | 52.60 | 32.55 | 120.11 | 29.50 |
| 264 | 172.68 | 123.25 | 52.60 | 32.55 | 120.11 | 29.50 |
| 265 | 96.12 | 116.19 | 35.40 | 23.15 | 113.04 | 12.30 |
| 266 | 96.12 | 116.19 | 35.40 | 23.15 | 113.04 | 12.30 |
| 267 | 171.34 | 158.73 | 46.39 | 2.84 | 155.59 | 23.29 |
| 268 | 171.34 | 158.73 | 46.39 | 2.84 | 155.59 | 23.29 |
| 269 | 23.24 | 80.56 | 6.62 | 67.59 | 83.70 | 29.72 |
| 270 | 23.24 | 80.56 | 6.62 | 67.59 | 83.70 | 29.72 |
| 271 | 221.34 | 74.16 | 76.16 | 101.73 | 77.30 | 53.06 |
| 272 | 221.34 | 74.16 | 76.16 | 101.73 | 77.30 | 53.06 |
| 273 | 47.46 | 81.22 | 11.84 | 46.03 | 84.36 | 11.26 |
| 274 | 47.46 | 81.22 | 11.84 | 46.03 | 84.36 | 11.26 |
| 275 | 219.99 | 38.68 | 69.95 | 72.02 | 41.82 | 46.85 |
| 276 | 219.99 | 38.68 | 69.95 | 72.02 | 41.82 | 46.85 |
| 277 | 89.32 | 47.19 | 29.02 | 21.12 | 43.41 | 15.16 |
| 278 | 60.17 | 166.16 | 29.02 | 50.27 | 162.38 | 15.16 |
| 279 | 29.53 | 51.02 | 12.71 | 0.64 | 47.25 | 1.15 |
| 280 | 58.68 | 169.99 | 12.71 | 29.78 | 166.22 | 1.15 |
| 281 | 131.75 | 46.79 | 40.09 | 34.06 | 43.02 | 26.23 |
| 282 | 102.60 | 165.76 | 40.09 | 63.21 | 161.99 | 26.23 |
| 283 | 28.73 | 72.31 | 8.98 | 18.46 | 68.54 | 4.88 |
| 284 | 57.88 | 191.28 | 8.98 | 47.61 | 187.51 | 4.88 |
| 285 | 19.28 | 147.28 | 4.84 | 8.83 | 143.51 | 9.02 |
| 286 | 1.34 | 27.78 | 0.39 | 31.35 | 31.56 | 13.47 |
| 287 | 138.13 | 151.12 | 36.89 | 11.65 | 147.35 | 23.03 |
| 288 | 120.20 | 23.95 | 41.33 | 51.83 | 27.72 | 27.47 |
| 289 | 23.15 | 146.88 | 15.91 | 21.77 | 143.11 | 2.05 |
| 290 | 41.08 | 28.18 | 11.47 | 18.41 | 31.95 | 2.39 |
| 291 | 137.33 | 172.41 | 33.16 | 6.18 | 168.64 | 19.30 |
| 292 | 119.39 | 2.65 | 37.61 | 34.00 | 6.43 | 23.75 |
| 293 | 59.51 | 172.27 | 28.49 | 48.27 | 168.50 | 14.63 |
| 294 | 59.51 | 172.27 | 28.49 | 48.27 | 168.50 | 14.63 |
| 295 | 59.34 | 176.11 | 13.24 | 27.78 | 172.34 | 0.62 |
| 296 | 59.34 | 176.11 | 13.24 | 27.78 | 172.34 | 0.62 |
| 297 | 101.94 | 171.87 | 39.56 | 61.21 | 168.10 | 25.70 |
| 298 | 101.94 | 171.87 | 39.56 | 61.21 | 168.10 | 25.70 |
| 299 | 58.54 | 197.40 | 9.52 | 45.61 | 193.63 | 4.34 |
| 300 | 58.54 | 197.40 | 9.52 | 45.61 | 193.63 | 4.34 |
| 301 | 0.68 | 33.90 | 0.93 | 29.35 | 37.67 | 12.93 |
| 302 | 0.68 | 33.90 | 0.93 | 29.35 | 37.67 | 12.93 |
| 303 | 119.53 | 30.06 | 40.80 | 49.83 | 33.83 | 26.94 |
| 304 | 119.53 | 30.06 | 40.80 | 49.83 | 33.83 | 26.94 |
| 305 | 41.75 | 34.30 | 12.00 | 16.41 | 38.07 | 1.86 |
| 306 | 41.75 | 34.30 | 12.00 | 16.41 | 38.07 | 1.86 |
| 307 | 118.73 | 8.77 | 37.08 | 32.00 | 12.54 | 23.22 |
| 308 | 118.73 | 8.77 | 37.08 | 32.00 | 12.54 | 23.22 |
| 309 | 84.48 | 42.32 | 27.28 | 17.28 | 39.17 | 13.42 |
| 310 | 55.34 | 161.29 | 27.28 | 46.43 | 158.14 | 13.42 |
| 311 | 34.37 | 46.16 | 14.44 | 3.20 | 43.01 | 0.58 |
| 312 | 63.52 | 165.13 | 14.44 | 25.95 | 161.98 | 0.58 |
| 313 | 126.91 | 41.92 | 38.36 | 30.22 | 38.78 | 24.50 |
| 314 | 97.76 | 160.89 | 38.36 | 59.37 | 157.75 | 24.50 |
| 315 | 33.57 | 67.45 | 10.72 | 14.62 | 64.30 | 3.14 |
| 316 | 62.71 | 186.42 | 10.72 | 43.77 | 183.27 | 3.14 |
| 317 | 24.12 | 142.41 | 3.11 | 5.00 | 139.27 | 10.75 |
| 318 | 6.18 | 32.65 | 1.34 | 35.18 | 35.80 | 15.20 |
| 319 | 142.97 | 146.25 | 38.62 | 15.49 | 143.11 | 24.76 |
| 320 | 125.03 | 28.81 | 43.07 | 55.67 | 31.96 | 29.21 |
| 321 | 18.31 | 142.01 | 14.18 | 17.94 | 138.87 | 0.32 |
| 322 | 36.25 | 33.05 | 9.73 | 22.24 | 36.19 | 4.13 |
| 323 | 142.17 | 167.54 | 34.90 | 2.34 | 164.40 | 21.04 |
| 324 | 124.23 | 7.52 | 39.34 | 37.84 | 10.67 | 25.48 |
| 325 | 54.67 | 167.40 | 26.75 | 44.43 | 164.26 | 12.89 |
| 326 | 54.67 | 167.40 | 26.75 | 44.43 | 164.26 | 12.89 |
| 327 | 64.18 | 171.24 | 14.98 | 23.95 | 168.10 | 1.12 |
| 328 | 64.18 | 171.24 | 14.98 | 23.95 | 168.10 | 1.12 |
| 329 | 97.10 | 167.00 | 37.82 | 57.37 | 163.86 | 23.96 |
| 330 | 97.10 | 167.00 | 37.82 | 57.37 | 163.86 | 23.96 |
| 331 | 63.38 | 192.53 | 11.25 | 41.77 | 189.39 | 2.61 |
| 332 | 63.38 | 192.53 | 11.25 | 41.77 | 189.39 | 2.61 |
| 333 | 5.51 | 38.77 | 0.81 | 33.18 | 41.91 | 14.67 |
| 334 | 5.51 | 38.77 | 0.81 | 33.18 | 41.91 | 14.67 |
| 335 | 124.37 | 34.93 | 42.54 | 53.67 | 38.07 | 28.68 |
| 336 | 124.37 | 34.93 | 42.54 | 53.67 | 38.07 | 28.68 |
| 337 | 36.91 | 39.16 | 10.26 | 20.24 | 42.31 | 3.60 |
| 338 | 36.91 | 39.16 | 10.26 | 20.24 | 42.31 | 3.60 |
| 339 | 123.57 | 13.64 | 38.81 | 35.84 | 16.78 | 24.95 |
| 340 | 123.57 | 13.64 | 38.81 | 35.84 | 16.78 | 24.95 |
| 341 | 97.07 | 75.72 | 30.05 | 53.24 | 71.95 | 30.05 |
| 342 | 30.31 | 72.09 | 13.46 | 36.91 | 68.31 | 13.46 |
| 343 | 79.23 | 148.56 | 30.05 | 71.09 | 144.79 | 30.05 |
| 344 | 12.46 | 144.92 | 13.46 | 54.76 | 141.15 | 13.46 |
| 345 | 61.02 | 33.52 | 18.17 | 29.87 | 29.75 | 18.17 |
| 346 | 5.75 | 29.89 | 1.58 | 13.54 | 26.11 | 1.58 |
| 347 | 61.02 | 33.52 | 18.17 | 29.87 | 29.75 | 18.17 |
| 348 | 5.75 | 29.89 | 1.58 | 13.54 | 26.11 | 1.58 |
| 349 | 78.94 | 151.18 | 29.83 | 70.23 | 147.41 | 29.83 |
| 350 | 12.18 | 147.54 | 13.23 | 53.90 | 143.77 | 13.23 |
| 351 | 78.94 | 151.18 | 29.83 | 70.23 | 147.41 | 29.83 |
| 352 | 12.18 | 147.54 | 13.23 | 53.90 | 143.77 | 13.23 |
| 353 | 61.30 | 30.90 | 18.40 | 30.73 | 27.13 | 18.40 |
| 354 | 5.46 | 27.27 | 1.80 | 14.40 | 23.49 | 1.80 |
| 355 | 61.30 | 30.90 | 18.40 | 30.73 | 27.13 | 18.40 |
| 356 | 5.46 | 27.27 | 1.80 | 14.40 | 23.49 | 1.80 |
| 357 | 86.43 | 63.41 | 26.42 | 45.69 | 60.26 | 26.42 |
| 358 | 19.66 | 59.77 | 9.82 | 29.36 | 56.62 | 9.82 |
| 359 | 71.55 | 124.10 | 26.42 | 60.56 | 120.96 | 26.42 |
| 360 | 4.79 | 120.47 | 9.82 | 44.23 | 117.32 | 9.82 |
| 361 | 56.38 | 28.24 | 16.51 | 26.21 | 25.10 | 16.51 |
| 362 | 10.38 | 24.60 | 0.08 | 9.88 | 21.46 | 0.08 |
| 363 | 56.38 | 28.24 | 16.51 | 26.21 | 25.10 | 16.51 |
| 364 | 10.38 | 24.60 | 0.08 | 9.88 | 21.46 | 0.08 |
| 365 | 71.32 | 126.29 | 26.22 | 59.84 | 123.15 | 26.22 |
| 366 | 4.55 | 122.65 | 9.63 | 43.52 | 119.51 | 9.63 |
| 367 | 71.32 | 126.29 | 26.22 | 59.84 | 123.15 | 26.22 |
| 368 | 4.55 | 122.65 | 9.63 | 43.52 | 119.51 | 9.63 |
| 369 | 56.62 | 26.06 | 16.70 | 26.93 | 22.91 | 16.70 |
| 370 | 10.15 | 22.42 | 0.11 | 10.60 | 19.28 | 0.11 |
| 371 | 56.62 | 26.06 | 16.70 | 26.93 | 22.91 | 16.70 |
| 372 | 10.15 | 22.42 | 0.11 | 10.60 | 19.28 | 0.11 |

Considering the strength after web buckling, the strength calculation control combination number :259, M=-238.40, N=140.67, M=-33.24, N=-137.52

After buckling, the strength ratio was calculated as 0.807

Shear strength calculation control combination number: 24, V=9.37

The calculated stress ratio of shear strength =0.192

The combination number corresponding to the maximum stress calculated in plane stability is 259, M=-238.40, N=140.67, M=-33.24, N=-137.52

The maximum stress calculated by in-plane stability (N/mm*mm) =251.15

The maximum in-plane stability stress ratio =0.823

The maximum stress ratio calculated out of plane stability =0.927

Door gauge gb51022-2015 allowable thickness ratio of web [H0/TW] =250.00

Flange tolerance width ratio [B/T] =12.38

After buckling, the calculated stress ratio was 0.807 < 1.0

The calculated stress ratio of shear strength =0.192 < 1.0

The calculated maximum stress in plane stability is < f=305.00

The maximum stress ratio calculated for stability outside the plane is < 1.0

Web height/thickness ratio H0/TW=63.33 < [H0/TW]=250.00

Flange width ratio B/T =12.20 < [B/T]=12.38

Compression bar, the internal length - to - thin ratio of plane lambda =56. ≤ [lambda]=180

Pressure bar, plane length of the slender ratio of lambda =84. ≤ [lambda]=180

Component weight (Kg)=285.74

Steel column 3     The design results

Section type =16;   Setting Angle =0;   Calculated length: Lx=9.68, Ly=2.50;   Length ratio: lambda x=56.0, lambda y=41.8

Component length =2.50;   Calculate length coefficient: Ux=3.87 Uy=1.00

Seismic rating: grade 3

Cross section parameters: B1=250, B2=250, H=400, Tw=6, T1=10, T2=10

Classification of axial compression section: X-axis :b, Y-axis :c

Steel number of component: Q345

Check calculation specification: door gb51022-2015

| \ | I end |  |  | II end |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| combination | M | N | V | M | N | V |
| 1 | 46.61 | 44.57 | 21.03 | 51.55 | 30.57 | 10.96 |
| 2 | 24.14 | 32.69 | 10.96 | 99.17 | 42.45 | 21.03 |
| 3 | 45.21 | 48.85 | 20.65 | 96.84 | 46.73 | 20.65 |
| 4 | 25.54 | 28.41 | 11.34 | 53.88 | 26.29 | 11.34 |
| 5 | 52.60 | 48.23 | 23.72 | 43.86 | 29.38 | 9.34 |
| 6 | 20.50 | 31.26 | 9.34 | 111.90 | 46.34 | 23.72 |
| 7 | 50.60 | 54.34 | 23.19 | 108.57 | 52.46 | 23.19 |
| 8 | 22.50 | 25.15 | 9.88 | 47.20 | 23.26 | 9.88 |
| 9 | 48.77 | 43.99 | 21.98 | 35.69 | 25.45 | 7.61 |
| 10 | 16.67 | 27.02 | 7.61 | 103.72 | 42.42 | 21.98 |
| 11 | 46.77 | 50.11 | 21.45 | 100.39 | 48.53 | 21.45 |
| 12 | 18.67 | 20.91 | 8.14 | 39.02 | 19.34 | 8.14 |
| 13 | 105.28 | 99.57 | 48.62 | 212.39 | 101.46 | 37.07 |
| 14 | 35.50 | 74.46 | 26.79 | 116.92 | 76.35 | 38.34 |
| 15 | 68.28 | 36.26 | 40.61 | 155.37 | 38.15 | 29.06 |
| 16 | 21.31 | 78.81 | 13.29 | 68.98 | 80.69 | 24.84 |
| 17 | 109.12 | 103.81 | 50.36 | 220.57 | 105.38 | 38.81 |
| 18 | 39.34 | 78.70 | 28.53 | 125.09 | 80.27 | 40.08 |
| 19 | 72.12 | 40.50 | 42.34 | 163.54 | 42.07 | 30.79 |
| 20 | 25.15 | 83.05 | 15.03 | 77.16 | 84.62 | 26.58 |
| 21 | 24.38 | 26.78 | 11.70 | 113.00 | 45.63 | 19.14 |
| 22 | 17.49 | 11.71 | 1.40 | 55.71 | 30.56 | 19.91 |
| 23 | 2.18 | 11.21 | 6.89 | 78.78 | 7.64 | 14.34 |
| 24 | 26.00 | 14.32 | 9.50 | 26.95 | 33.17 | 11.81 |
| 25 | 56.48 | 43.74 | 26.07 | 44.96 | 28.66 | 4.77 |
| 26 | 14.61 | 28.68 | 12.98 | 12.32 | 13.60 | 5.53 |
| 27 | 34.28 | 5.76 | 21.27 | 10.75 | 9.32 | 0.04 |
| 28 | 6.10 | 31.29 | 4.88 | 41.08 | 16.20 | 2.57 |
| 29 | 26.38 | 20.66 | 12.23 | 48.30 | 22.55 | 5.30 |
| 30 | 15.49 | 5.59 | 0.86 | 8.99 | 7.48 | 6.07 |
| 31 | 4.18 | 17.32 | 7.42 | 14.08 | 15.44 | 0.49 |
| 32 | 24.00 | 8.20 | 8.96 | 37.75 | 10.09 | 2.03 |
| 33 | 54.48 | 49.86 | 25.54 | 109.67 | 51.74 | 18.61 |
| 34 | 12.61 | 34.79 | 12.44 | 52.38 | 36.68 | 19.37 |
| 35 | 32.28 | 11.87 | 20.73 | 75.45 | 13.76 | 13.80 |
| 36 | 4.10 | 37.40 | 4.34 | 23.62 | 39.29 | 11.27 |
| 37 | 28.21 | 31.02 | 13.43 | 121.17 | 49.56 | 20.88 |
| 38 | 13.65 | 15.95 | 0.34 | 63.89 | 34.49 | 21.64 |
| 39 | 6.02 | 6.97 | 8.63 | 86.96 | 11.57 | 16.07 |
| 40 | 22.16 | 18.56 | 7.76 | 35.13 | 37.10 | 13.54 |
| 41 | 60.31 | 47.98 | 27.81 | 53.14 | 32.59 | 6.50 |
| 42 | 18.45 | 32.92 | 14.71 | 4.15 | 17.52 | 7.27 |
| 43 | 38.12 | 10.00 | 23.00 | 18.92 | 5.40 | 1.70 |
| 44 | 9.93 | 35.52 | 6.61 | 32.91 | 20.13 | 0.83 |
| 45 | 30.21 | 24.90 | 13.97 | 56.47 | 26.47 | 7.04 |
| 46 | 11.65 | 9.83 | 0.87 | 0.81 | 11.41 | 7.80 |
| 47 | 8.02 | 13.09 | 9.16 | 22.25 | 11.51 | 2.23 |
| 48 | 20.16 | 12.44 | 7.23 | 29.58 | 14.01 | 0.30 |
| 49 | 58.31 | 54.10 | 27.28 | 117.84 | 55.67 | 20.35 |
| 50 | 16.45 | 39.03 | 14.18 | 60.56 | 40.60 | 21.11 |
| 51 | 36.12 | 16.11 | 22.47 | 83.63 | 17.68 | 15.54 |
| 52 | 7.94 | 41.64 | 6.08 | 31.80 | 43.21 | 13.01 |
| 53 | 84.57 | 83.62 | 39.31 | 216.02 | 97.38 | 37.82 |
| 54 | 14.79 | 58.51 | 17.48 | 120.55 | 72.27 | 39.09 |
| 55 | 47.58 | 20.31 | 31.29 | 158.99 | 34.07 | 29.80 |
| 56 | 0.61 | 62.85 | 3.98 | 72.61 | 76.62 | 25.59 |
| 57 | 107.04 | 95.50 | 49.37 | 168.40 | 85.50 | 27.76 |
| 58 | 37.26 | 70.39 | 27.54 | 72.92 | 60.39 | 29.03 |
| 59 | 70.05 | 32.19 | 41.35 | 111.37 | 22.19 | 19.74 |
| 60 | 23.08 | 74.73 | 14.04 | 24.99 | 64.74 | 15.53 |
| 61 | 85.97 | 79.34 | 39.68 | 170.73 | 81.22 | 28.13 |
| 62 | 16.19 | 54.23 | 17.85 | 75.25 | 56.11 | 29.40 |
| 63 | 48.98 | 16.03 | 31.66 | 113.70 | 17.91 | 20.11 |
| 64 | 2.01 | 58.57 | 4.35 | 27.32 | 60.46 | 15.90 |
| 65 | 105.64 | 99.78 | 48.99 | 213.69 | 101.66 | 37.44 |
| 66 | 35.86 | 74.67 | 27.17 | 118.22 | 76.55 | 38.72 |
| 67 | 68.65 | 36.47 | 40.98 | 156.66 | 38.35 | 29.43 |
| 68 | 21.68 | 79.01 | 13.67 | 70.28 | 80.90 | 25.22 |
| 69 | 88.41 | 87.86 | 41.04 | 224.19 | 101.31 | 39.55 |
| 70 | 18.63 | 62.75 | 19.21 | 128.72 | 76.20 | 40.82 |
| 71 | 51.42 | 24.55 | 33.03 | 167.17 | 38.00 | 31.54 |
| 72 | 4.44 | 67.09 | 5.71 | 80.79 | 80.54 | 27.32 |
| 73 | 110.88 | 99.74 | 51.10 | 176.57 | 89.43 | 29.49 |
| 74 | 41.10 | 74.62 | 29.27 | 81.10 | 64.32 | 30.76 |
| 75 | 73.88 | 36.43 | 43.09 | 119.54 | 26.12 | 21.48 |
| 76 | 26.91 | 78.97 | 15.77 | 33.16 | 68.66 | 17.26 |
| 77 | 89.81 | 83.58 | 41.41 | 178.90 | 85.15 | 29.86 |
| 78 | 20.03 | 58.47 | 19.58 | 83.43 | 60.04 | 31.13 |
| 79 | 52.81 | 20.27 | 33.40 | 121.88 | 21.84 | 21.85 |
| 80 | 5.84 | 62.81 | 6.08 | 35.49 | 64.38 | 17.63 |
| 81 | 109.48 | 104.02 | 50.73 | 221.86 | 105.59 | 39.18 |
| 82 | 39.70 | 78.91 | 28.90 | 126.39 | 80.48 | 40.45 |
| 83 | 72.49 | 40.71 | 42.72 | 164.84 | 42.28 | 31.17 |
| 84 | 25.51 | 83.25 | 15.40 | 78.45 | 84.82 | 26.95 |
| 85 | 27.30 | 33.39 | 12.85 | 30.73 | 24.23 | 6.69 |
| 86 | 43.63 | 37.02 | 19.45 | 62.90 | 27.87 | 13.29 |
| 87 | 13.54 | 26.11 | 6.69 | 59.89 | 31.50 | 12.85 |
| 88 | 29.87 | 29.75 | 13.29 | 92.06 | 35.14 | 19.45 |
| 89 | 26.44 | 36.01 | 12.63 | 58.46 | 34.12 | 12.63 |
| 90 | 42.77 | 39.64 | 19.22 | 90.63 | 37.76 | 19.22 |
| 91 | 14.40 | 23.49 | 6.92 | 32.16 | 21.61 | 6.92 |
| 92 | 30.73 | 27.13 | 13.52 | 64.33 | 25.25 | 13.52 |
| 93 | 21.35 | 27.52 | 10.17 | 22.93 | 19.89 | 5.03 |
| 94 | 37.68 | 31.16 | 16.76 | 55.10 | 23.53 | 11.63 |
| 95 | 9.88 | 21.46 | 5.03 | 47.22 | 25.95 | 10.17 |
| 96 | 26.21 | 25.10 | 11.63 | 79.39 | 29.59 | 16.76 |
| 97 | 20.63 | 29.70 | 9.98 | 46.03 | 28.13 | 9.98 |
| 98 | 36.96 | 33.34 | 16.57 | 78.20 | 31.77 | 16.57 |
| 99 | 10.60 | 19.28 | 5.22 | 24.11 | 17.70 | 5.22 |
| 100 | 26.93 | 22.91 | 11.82 | 56.29 | 21.34 | 11.82 |
| 101 | 23.02 | 25.44 | 10.41 | 49.05 | 23.55 | 10.41 |
| 102 | 46.58 | 21.52 | 16.76 | 7.23 | 19.64 | 23.77 |
| 103 | 12.64 | 29.35 | 30.77 | 77.66 | 27.46 | 23.77 |
| 104 | 12.64 | 29.35 | 30.77 | 77.66 | 27.46 | 23.77 |
| 105 | 46.58 | 21.52 | 16.76 | 7.23 | 19.64 | 23.77 |
| 106 | 46.58 | 21.52 | 16.76 | 7.23 | 19.64 | 23.77 |
| 107 | 46.58 | 21.52 | 16.76 | 7.23 | 19.64 | 23.77 |
| 108 | 12.64 | 29.35 | 30.77 | 77.66 | 27.46 | 23.77 |
| 109 | 19.19 | 21.20 | 8.67 | 40.87 | 19.63 | 8.67 |
| 110 | 50.42 | 17.28 | 15.03 | 0.94 | 15.71 | 22.03 |
| 111 | 8.81 | 25.11 | 29.03 | 69.49 | 23.54 | 22.03 |
| 112 | 8.81 | 25.11 | 29.03 | 69.49 | 23.54 | 22.03 |
| 113 | 50.42 | 17.28 | 15.03 | 0.94 | 15.71 | 22.03 |
| 114 | 50.42 | 17.28 | 15.03 | 0.94 | 15.71 | 22.03 |
| 115 | 50.42 | 17.28 | 15.03 | 0.94 | 15.71 | 22.03 |
| 116 | 8.81 | 25.11 | 29.03 | 69.49 | 23.54 | 22.03 |
| 117 | 43.73 | 41.39 | 19.73 | 45.42 | 27.63 | 9.66 |
| 118 | 25.87 | 37.48 | 26.08 | 3.61 | 23.72 | 23.02 |
| 119 | 48.34 | 25.60 | 16.02 | 51.23 | 35.59 | 33.08 |
| 120 | 10.88 | 33.43 | 30.02 | 121.66 | 43.42 | 33.08 |
| 121 | 31.95 | 49.58 | 39.71 | 119.33 | 47.70 | 32.71 |
| 122 | 31.95 | 49.58 | 39.71 | 119.33 | 47.70 | 32.71 |
| 123 | 46.94 | 21.32 | 16.39 | 5.94 | 19.44 | 23.39 |
| 124 | 46.94 | 21.32 | 16.39 | 5.94 | 19.44 | 23.39 |
| 125 | 39.89 | 37.15 | 17.99 | 37.24 | 23.70 | 7.93 |
| 126 | 29.71 | 33.24 | 24.35 | 4.57 | 19.79 | 21.29 |
| 127 | 52.18 | 21.36 | 14.28 | 43.06 | 31.67 | 31.35 |
| 128 | 7.04 | 29.19 | 28.29 | 113.48 | 39.49 | 31.35 |
| 129 | 28.11 | 45.35 | 37.98 | 111.15 | 43.77 | 30.97 |
| 130 | 28.11 | 45.35 | 37.98 | 111.15 | 43.77 | 30.97 |
| 131 | 50.78 | 17.08 | 14.66 | 2.23 | 15.51 | 21.66 |
| 132 | 50.78 | 17.08 | 14.66 | 2.23 | 15.51 | 21.66 |
| 133 | 52.60 | 48.23 | 23.72 | 43.86 | 29.38 | 9.34 |
| 134 | 3.88 | 45.49 | 28.17 | 14.60 | 26.64 | 18.69 |
| 135 | 28.22 | 28.52 | 13.79 | 82.63 | 43.61 | 33.07 |
| 136 | 13.24 | 34.00 | 23.60 | 131.93 | 49.08 | 33.07 |
| 137 | 43.34 | 57.08 | 37.44 | 128.60 | 55.20 | 32.54 |
| 138 | 43.34 | 57.08 | 37.44 | 128.60 | 55.20 | 32.54 |
| 139 | 26.22 | 22.41 | 14.33 | 17.93 | 20.52 | 19.23 |
| 140 | 26.22 | 22.41 | 14.33 | 17.93 | 20.52 | 19.23 |
| 141 | 48.77 | 43.99 | 21.98 | 35.69 | 25.45 | 7.61 |
| 142 | 0.05 | 41.25 | 26.43 | 6.42 | 22.71 | 16.96 |
| 143 | 32.05 | 24.28 | 12.06 | 74.46 | 39.68 | 31.33 |
| 144 | 9.40 | 29.76 | 21.86 | 123.75 | 45.16 | 31.33 |
| 145 | 39.50 | 52.84 | 35.70 | 120.42 | 51.27 | 30.80 |
| 146 | 39.50 | 52.84 | 35.70 | 120.42 | 51.27 | 30.80 |
| 147 | 30.05 | 18.17 | 12.59 | 9.75 | 16.60 | 17.49 |
| 148 | 30.05 | 18.17 | 12.59 | 9.75 | 16.60 | 17.49 |
| 149 | 33.25 | 33.61 | 15.69 | 111.44 | 47.38 | 18.82 |
| 150 | 102.85 | 37.53 | 9.34 | 153.25 | 51.29 | 5.47 |
| 151 | 8.62 | 18.55 | 2.60 | 54.16 | 32.31 | 19.59 |
| 152 | 60.98 | 22.46 | 3.76 | 95.97 | 36.22 | 6.23 |
| 153 | 11.06 | 4.37 | 10.88 | 77.23 | 9.39 | 14.02 |
| 154 | 80.66 | 0.46 | 4.53 | 119.04 | 13.30 | 0.66 |
| 155 | 17.13 | 21.16 | 5.50 | 25.40 | 34.92 | 11.49 |
| 156 | 52.47 | 25.07 | 11.86 | 67.21 | 38.83 | 1.87 |
| 157 | 125.32 | 49.40 | 19.40 | 105.63 | 39.41 | 4.59 |
| 158 | 66.10 | 41.58 | 5.40 | 35.21 | 31.59 | 4.59 |
| 159 | 83.45 | 34.34 | 6.30 | 48.35 | 24.35 | 3.83 |
| 160 | 24.23 | 26.51 | 7.70 | 22.08 | 16.52 | 3.83 |
| 161 | 103.13 | 11.42 | 14.59 | 71.42 | 1.43 | 9.40 |
| 162 | 43.90 | 3.59 | 0.59 | 0.99 | 6.40 | 9.40 |
| 163 | 74.94 | 36.95 | 1.80 | 19.59 | 26.95 | 11.93 |
| 164 | 15.72 | 29.12 | 15.80 | 50.84 | 19.13 | 11.93 |
| 165 | 45.03 | 25.42 | 4.29 | 37.54 | 27.31 | 4.22 |
| 166 | 45.03 | 25.42 | 4.29 | 37.54 | 27.31 | 4.22 |
| 167 | 3.16 | 10.35 | 17.39 | 19.75 | 12.24 | 3.46 |
| 168 | 3.16 | 10.35 | 17.39 | 19.75 | 12.24 | 3.46 |
| 169 | 22.83 | 12.56 | 9.10 | 3.32 | 10.68 | 9.03 |
| 170 | 22.83 | 12.56 | 9.10 | 3.32 | 10.68 | 9.03 |
| 171 | 5.35 | 12.96 | 25.49 | 48.51 | 14.85 | 11.56 |
| 172 | 5.35 | 12.96 | 25.49 | 48.51 | 14.85 | 11.56 |
| 173 | 123.92 | 53.68 | 19.03 | 150.92 | 55.57 | 5.09 |
| 174 | 123.92 | 53.68 | 19.03 | 150.92 | 55.57 | 5.09 |
| 175 | 82.05 | 38.62 | 5.93 | 93.64 | 40.50 | 5.86 |
| 176 | 82.05 | 38.62 | 5.93 | 93.64 | 40.50 | 5.86 |
| 177 | 101.73 | 15.70 | 14.22 | 116.71 | 17.58 | 0.29 |
| 178 | 101.73 | 15.70 | 14.22 | 116.71 | 17.58 | 0.29 |
| 179 | 73.54 | 41.23 | 2.17 | 64.88 | 43.11 | 2.24 |
| 180 | 73.54 | 41.23 | 2.17 | 64.88 | 43.11 | 2.24 |
| 181 | 37.09 | 37.85 | 17.43 | 119.62 | 51.30 | 20.56 |
| 182 | 106.69 | 41.77 | 11.07 | 161.43 | 55.22 | 7.20 |
| 183 | 4.78 | 22.79 | 4.33 | 62.33 | 36.24 | 21.32 |
| 184 | 64.82 | 26.70 | 2.02 | 104.14 | 40.15 | 7.97 |
| 185 | 14.89 | 0.13 | 12.62 | 85.40 | 13.32 | 15.75 |
| 186 | 84.49 | 3.78 | 6.26 | 127.21 | 17.23 | 2.39 |
| 187 | 13.29 | 25.39 | 3.77 | 33.57 | 38.84 | 13.22 |
| 188 | 56.31 | 29.31 | 10.12 | 75.38 | 42.76 | 0.13 |
| 189 | 129.16 | 53.64 | 21.13 | 113.81 | 43.34 | 2.86 |
| 190 | 69.94 | 45.82 | 7.13 | 43.38 | 35.51 | 2.86 |
| 191 | 87.29 | 38.58 | 8.04 | 56.52 | 28.27 | 2.10 |
| 192 | 28.07 | 30.75 | 5.97 | 13.90 | 20.45 | 2.10 |
| 193 | 106.96 | 15.66 | 16.33 | 79.59 | 5.35 | 7.67 |
| 194 | 47.74 | 7.83 | 2.32 | 9.16 | 2.47 | 7.67 |
| 195 | 78.78 | 41.18 | 0.06 | 27.76 | 30.88 | 10.20 |
| 196 | 19.56 | 33.36 | 14.07 | 42.66 | 23.05 | 10.20 |
| 197 | 48.87 | 29.66 | 2.56 | 45.71 | 31.23 | 2.49 |
| 198 | 48.87 | 29.66 | 2.56 | 45.71 | 31.23 | 2.49 |
| 199 | 7.00 | 14.59 | 15.66 | 11.57 | 16.17 | 1.72 |
| 200 | 7.00 | 14.59 | 15.66 | 11.57 | 16.17 | 1.72 |
| 201 | 26.67 | 8.33 | 7.37 | 11.50 | 6.75 | 7.29 |
| 202 | 26.67 | 8.33 | 7.37 | 11.50 | 6.75 | 7.29 |
| 203 | 1.51 | 17.20 | 23.76 | 40.33 | 18.77 | 9.82 |
| 204 | 1.51 | 17.20 | 23.76 | 40.33 | 18.77 | 9.82 |
| 205 | 127.76 | 57.92 | 20.76 | 159.10 | 59.50 | 6.83 |
| 206 | 127.76 | 57.92 | 20.76 | 159.10 | 59.50 | 6.83 |
| 207 | 85.89 | 42.86 | 7.67 | 101.81 | 44.43 | 7.59 |
| 208 | 85.89 | 42.86 | 7.67 | 101.81 | 44.43 | 7.59 |
| 209 | 105.56 | 19.94 | 15.95 | 124.88 | 21.51 | 2.02 |
| 210 | 105.56 | 19.94 | 15.95 | 124.88 | 21.51 | 2.02 |
| 211 | 77.38 | 45.46 | 0.44 | 73.05 | 47.04 | 0.51 |
| 212 | 77.38 | 45.46 | 0.44 | 73.05 | 47.04 | 0.51 |
| 213 | 84.57 | 83.62 | 39.31 | 216.02 | 97.38 | 37.82 |
| 214 | 133.29 | 86.36 | 34.86 | 245.29 | 100.12 | 28.47 |
| 215 | 14.79 | 58.51 | 17.48 | 120.55 | 72.27 | 39.09 |
| 216 | 63.51 | 61.25 | 13.03 | 149.81 | 75.01 | 29.74 |
| 217 | 47.58 | 20.31 | 31.29 | 158.99 | 34.07 | 29.80 |
| 218 | 96.30 | 23.05 | 26.84 | 188.26 | 36.81 | 20.45 |
| 219 | 0.61 | 62.85 | 3.98 | 72.61 | 76.62 | 25.59 |
| 220 | 49.33 | 65.59 | 0.47 | 101.88 | 79.36 | 16.24 |
| 221 | 155.76 | 98.23 | 44.92 | 197.66 | 88.24 | 18.41 |
| 222 | 114.31 | 92.76 | 35.12 | 148.37 | 82.77 | 18.41 |
| 223 | 85.98 | 73.12 | 23.09 | 102.19 | 63.13 | 19.68 |
| 224 | 44.53 | 67.65 | 13.29 | 52.89 | 57.65 | 19.68 |
| 225 | 118.77 | 34.92 | 36.91 | 140.64 | 24.93 | 10.39 |
| 226 | 77.31 | 29.45 | 27.10 | 91.34 | 19.46 | 10.39 |
| 227 | 71.80 | 77.47 | 9.59 | 54.25 | 67.48 | 6.18 |
| 228 | 30.34 | 71.99 | 0.21 | 4.96 | 62.00 | 6.18 |
| 229 | 93.24 | 76.60 | 25.43 | 150.70 | 78.48 | 18.78 |
| 230 | 93.24 | 76.60 | 25.43 | 150.70 | 78.48 | 18.78 |
| 231 | 23.46 | 51.49 | 3.60 | 55.22 | 53.37 | 20.05 |
| 232 | 23.46 | 51.49 | 3.60 | 55.22 | 53.37 | 20.05 |
| 233 | 56.24 | 13.29 | 17.41 | 93.67 | 15.17 | 10.76 |
| 234 | 56.24 | 13.29 | 17.41 | 93.67 | 15.17 | 10.76 |
| 235 | 9.27 | 55.83 | 9.90 | 7.29 | 57.72 | 6.55 |
| 236 | 9.27 | 55.83 | 9.90 | 7.29 | 57.72 | 6.55 |
| 237 | 154.36 | 102.51 | 44.55 | 242.96 | 104.40 | 28.09 |
| 238 | 154.36 | 102.51 | 44.55 | 242.96 | 104.40 | 28.09 |
| 239 | 84.58 | 77.40 | 22.72 | 147.48 | 79.29 | 29.37 |
| 240 | 84.58 | 77.40 | 22.72 | 147.48 | 79.29 | 29.37 |
| 241 | 117.37 | 39.20 | 36.53 | 185.93 | 41.09 | 20.08 |
| 242 | 117.37 | 39.20 | 36.53 | 185.93 | 41.09 | 20.08 |
| 243 | 70.40 | 81.75 | 9.22 | 99.55 | 83.64 | 15.87 |
| 244 | 70.40 | 81.75 | 9.22 | 99.55 | 83.64 | 15.87 |
| 245 | 88.41 | 87.86 | 41.04 | 224.19 | 101.31 | 39.55 |
| 246 | 137.13 | 90.60 | 36.59 | 253.46 | 104.05 | 30.20 |
| 247 | 18.63 | 62.75 | 19.21 | 128.72 | 76.20 | 40.82 |
| 248 | 67.35 | 65.49 | 14.76 | 157.99 | 78.94 | 31.47 |
| 249 | 51.42 | 24.55 | 33.03 | 167.17 | 38.00 | 31.54 |
| 250 | 100.14 | 27.29 | 28.58 | 196.44 | 40.74 | 22.19 |
| 251 | 4.44 | 67.09 | 5.71 | 80.79 | 80.54 | 27.32 |
| 252 | 53.17 | 69.83 | 1.26 | 110.05 | 83.28 | 17.97 |
| 253 | 159.60 | 102.47 | 46.65 | 205.84 | 92.17 | 20.14 |
| 254 | 118.14 | 97.00 | 36.85 | 156.54 | 86.69 | 20.14 |
| 255 | 89.82 | 77.36 | 24.83 | 110.36 | 67.06 | 21.41 |
| 256 | 48.36 | 71.89 | 15.02 | 61.07 | 61.58 | 21.41 |
| 257 | 122.61 | 39.16 | 38.64 | 148.81 | 28.86 | 12.13 |
| 258 | 81.15 | 33.69 | 28.84 | 99.51 | 23.38 | 12.13 |
| 259 | 75.63 | 81.71 | 11.32 | 62.43 | 71.40 | 7.91 |
| 260 | 34.18 | 76.23 | 1.52 | 13.13 | 65.93 | 7.91 |
| 261 | 97.07 | 80.84 | 27.16 | 158.87 | 82.41 | 20.51 |
| 262 | 97.07 | 80.84 | 27.16 | 158.87 | 82.41 | 20.51 |
| 263 | 27.29 | 55.73 | 5.33 | 63.40 | 57.30 | 21.78 |
| 264 | 27.29 | 55.73 | 5.33 | 63.40 | 57.30 | 21.78 |
| 265 | 60.08 | 17.53 | 19.15 | 101.85 | 19.10 | 12.50 |
| 266 | 60.08 | 17.53 | 19.15 | 101.85 | 19.10 | 12.50 |
| 267 | 13.11 | 60.07 | 8.17 | 15.46 | 61.65 | 8.28 |
| 268 | 13.11 | 60.07 | 8.17 | 15.46 | 61.65 | 8.28 |
| 269 | 158.20 | 106.75 | 46.28 | 251.13 | 108.33 | 29.83 |
| 270 | 158.20 | 106.75 | 46.28 | 251.13 | 108.33 | 29.83 |
| 271 | 88.42 | 81.64 | 24.45 | 155.66 | 83.22 | 31.10 |
| 272 | 88.42 | 81.64 | 24.45 | 155.66 | 83.22 | 31.10 |
| 273 | 121.21 | 43.44 | 38.27 | 194.10 | 45.02 | 21.82 |
| 274 | 121.21 | 43.44 | 38.27 | 194.10 | 45.02 | 21.82 |
| 275 | 74.24 | 85.99 | 10.95 | 107.72 | 87.56 | 17.60 |
| 276 | 74.24 | 85.99 | 10.95 | 107.72 | 87.56 | 17.60 |
| 277 | 24.38 | 26.78 | 11.70 | 113.00 | 45.63 | 19.14 |
| 278 | 73.10 | 29.51 | 7.25 | 142.27 | 48.37 | 9.79 |
| 279 | 17.49 | 11.71 | 1.40 | 55.71 | 30.56 | 19.91 |
| 280 | 31.23 | 14.45 | 5.84 | 84.98 | 33.30 | 10.56 |
| 281 | 2.18 | 11.21 | 6.89 | 78.78 | 7.64 | 14.34 |
| 282 | 50.90 | 8.47 | 2.44 | 108.05 | 10.38 | 4.99 |
| 283 | 26.00 | 14.32 | 9.50 | 26.95 | 33.17 | 11.81 |
| 284 | 22.72 | 17.06 | 13.95 | 56.22 | 35.91 | 2.46 |
| 285 | 105.20 | 46.48 | 21.63 | 74.23 | 31.40 | 4.58 |
| 286 | 63.74 | 41.01 | 11.82 | 24.93 | 25.92 | 4.58 |
| 287 | 63.33 | 31.42 | 8.53 | 16.95 | 16.33 | 3.82 |
| 288 | 21.87 | 25.94 | 1.27 | 32.35 | 10.86 | 3.82 |
| 289 | 83.00 | 8.50 | 16.82 | 40.02 | 6.59 | 9.39 |
| 290 | 41.55 | 3.02 | 7.01 | 9.28 | 12.06 | 9.39 |
| 291 | 54.82 | 34.02 | 0.43 | 11.81 | 18.94 | 11.92 |
| 292 | 13.36 | 28.55 | 9.37 | 61.11 | 13.46 | 11.92 |
| 293 | 33.64 | 17.92 | 2.02 | 28.27 | 19.81 | 4.05 |
| 294 | 33.64 | 17.92 | 2.02 | 28.27 | 19.81 | 4.05 |
| 295 | 8.23 | 2.86 | 15.12 | 29.02 | 4.74 | 3.28 |
| 296 | 8.23 | 2.86 | 15.12 | 29.02 | 4.74 | 3.28 |
| 297 | 11.45 | 20.06 | 6.83 | 5.95 | 18.18 | 8.86 |
| 298 | 11.45 | 20.06 | 6.83 | 5.95 | 18.18 | 8.86 |
| 299 | 16.74 | 5.46 | 23.22 | 57.78 | 7.35 | 11.38 |
| 300 | 16.74 | 5.46 | 23.22 | 57.78 | 7.35 | 11.38 |
| 301 | 103.20 | 52.60 | 21.09 | 138.94 | 54.48 | 9.26 |
| 302 | 103.20 | 52.60 | 21.09 | 138.94 | 54.48 | 9.26 |
| 303 | 61.33 | 37.53 | 8.00 | 81.65 | 39.42 | 10.03 |
| 304 | 61.33 | 37.53 | 8.00 | 81.65 | 39.42 | 10.03 |
| 305 | 81.00 | 14.61 | 16.29 | 104.72 | 16.50 | 4.45 |
| 306 | 81.00 | 14.61 | 16.29 | 104.72 | 16.50 | 4.45 |
| 307 | 52.82 | 40.14 | 0.10 | 52.89 | 42.02 | 1.92 |
| 308 | 52.82 | 40.14 | 0.10 | 52.89 | 42.02 | 1.92 |
| 309 | 28.21 | 31.02 | 13.43 | 121.17 | 49.56 | 20.88 |
| 310 | 76.93 | 33.75 | 8.99 | 150.44 | 52.29 | 11.53 |
| 311 | 13.65 | 15.95 | 0.34 | 63.89 | 34.49 | 21.64 |
| 312 | 35.07 | 18.69 | 4.11 | 93.16 | 37.23 | 12.29 |
| 313 | 6.02 | 6.97 | 8.63 | 86.96 | 11.57 | 16.07 |
| 314 | 54.74 | 4.23 | 4.18 | 116.22 | 14.31 | 6.72 |
| 315 | 22.16 | 18.56 | 7.76 | 35.13 | 37.10 | 13.54 |
| 316 | 26.56 | 21.30 | 12.21 | 64.39 | 39.84 | 4.19 |
| 317 | 109.03 | 50.72 | 23.36 | 82.41 | 35.33 | 2.84 |
| 318 | 67.58 | 45.24 | 13.56 | 33.11 | 29.85 | 2.84 |
| 319 | 67.17 | 35.66 | 10.26 | 25.12 | 20.26 | 2.08 |
| 320 | 25.71 | 30.18 | 0.46 | 24.18 | 14.78 | 2.08 |
| 321 | 86.84 | 12.74 | 18.55 | 48.19 | 2.66 | 7.65 |
| 322 | 45.38 | 7.26 | 8.75 | 1.11 | 8.14 | 7.65 |
| 323 | 58.66 | 38.26 | 2.16 | 3.64 | 22.87 | 10.18 |
| 324 | 17.20 | 32.79 | 7.64 | 52.94 | 17.39 | 10.18 |
| 325 | 37.48 | 22.16 | 0.28 | 36.44 | 23.73 | 2.31 |
| 326 | 37.48 | 22.16 | 0.28 | 36.44 | 23.73 | 2.31 |
| 327 | 4.39 | 7.10 | 13.38 | 20.84 | 8.67 | 1.55 |
| 328 | 4.39 | 7.10 | 13.38 | 20.84 | 8.67 | 1.55 |
| 329 | 15.28 | 15.82 | 5.09 | 2.22 | 14.25 | 7.12 |
| 330 | 15.28 | 15.82 | 5.09 | 2.22 | 14.25 | 7.12 |
| 331 | 12.90 | 9.70 | 21.48 | 49.61 | 11.27 | 9.65 |
| 332 | 12.90 | 9.70 | 21.48 | 49.61 | 11.27 | 9.65 |
| 333 | 107.03 | 56.84 | 22.83 | 147.11 | 58.41 | 11.00 |
| 334 | 107.03 | 56.84 | 22.83 | 147.11 | 58.41 | 11.00 |
| 335 | 65.17 | 41.77 | 9.73 | 89.83 | 43.34 | 11.76 |
| 336 | 65.17 | 41.77 | 9.73 | 89.83 | 43.34 | 11.76 |
| 337 | 84.84 | 18.85 | 18.02 | 112.89 | 20.42 | 6.19 |
| 338 | 84.84 | 18.85 | 18.02 | 112.89 | 20.42 | 6.19 |
| 339 | 56.66 | 44.38 | 1.63 | 61.06 | 45.95 | 3.66 |
| 340 | 56.66 | 44.38 | 1.63 | 61.06 | 45.95 | 3.66 |
| 341 | 27.30 | 33.39 | 12.85 | 30.73 | 24.23 | 6.69 |
| 342 | 43.63 | 37.02 | 19.45 | 62.90 | 27.87 | 13.29 |
| 343 | 0.33 | 32.29 | 18.58 | 18.07 | 23.14 | 12.42 |
| 344 | 16.66 | 35.93 | 25.17 | 50.24 | 26.77 | 19.01 |
| 345 | 13.43 | 25.02 | 12.42 | 47.23 | 30.41 | 18.58 |
| 346 | 2.90 | 28.66 | 19.01 | 79.40 | 34.05 | 25.17 |
| 347 | 6.24 | 27.21 | 12.42 | 66.89 | 32.59 | 18.58 |
| 348 | 22.57 | 30.84 | 19.01 | 99.06 | 36.23 | 25.17 |
| 349 | 19.14 | 37.10 | 18.35 | 65.46 | 35.21 | 18.35 |
| 350 | 35.47 | 40.74 | 24.95 | 97.64 | 38.85 | 24.95 |
| 351 | 19.14 | 37.10 | 18.35 | 65.46 | 35.21 | 18.35 |
| 352 | 35.47 | 40.74 | 24.95 | 97.64 | 38.85 | 24.95 |
| 353 | 12.57 | 22.40 | 12.65 | 19.50 | 20.52 | 12.65 |
| 354 | 3.76 | 26.04 | 19.24 | 51.67 | 24.15 | 19.24 |
| 355 | 12.57 | 22.40 | 12.65 | 19.50 | 20.52 | 12.65 |
| 356 | 3.76 | 26.04 | 19.24 | 51.67 | 24.15 | 19.24 |
| 357 | 21.35 | 27.52 | 10.17 | 22.93 | 19.89 | 5.03 |
| 358 | 37.68 | 31.16 | 16.76 | 55.10 | 23.53 | 11.63 |
| 359 | 1.13 | 26.61 | 14.94 | 12.37 | 18.98 | 9.81 |
| 360 | 15.20 | 30.25 | 21.53 | 44.55 | 22.61 | 16.40 |
| 361 | 12.59 | 20.55 | 9.81 | 36.67 | 25.04 | 14.94 |
| 362 | 3.74 | 24.19 | 16.40 | 68.84 | 28.67 | 21.53 |
| 363 | 3.80 | 22.37 | 9.81 | 53.06 | 26.86 | 14.94 |
| 364 | 20.13 | 26.01 | 16.40 | 85.23 | 30.50 | 21.53 |
| 365 | 14.55 | 30.61 | 14.75 | 51.87 | 29.04 | 14.75 |
| 366 | 30.88 | 34.25 | 21.34 | 84.04 | 32.68 | 21.34 |
| 367 | 14.55 | 30.61 | 14.75 | 51.87 | 29.04 | 14.75 |
| 368 | 30.88 | 34.25 | 21.34 | 84.04 | 32.68 | 21.34 |
| 369 | 11.88 | 18.36 | 10.00 | 13.56 | 16.79 | 10.00 |
| 370 | 4.45 | 22.00 | 16.59 | 45.73 | 20.43 | 16.59 |
| 371 | 11.88 | 18.36 | 10.00 | 13.56 | 16.79 | 10.00 |
| 372 | 4.45 | 22.00 | 16.59 | 45.73 | 20.43 | 16.59 |

Considering the strength after web buckling, the strength calculation control combination number :246, M=-137.13, N=-90.60, M=253.46, N=104.05

After buckling, the calculated stress ratio was 0.768

Shear strength calculation control combination number: 73, V=51.10

The calculated stress ratio of shear strength =0.128

The corresponding combination number of the maximum stress in the plane stability calculation :136, M=13.24, N=34.00, M=-131.93, N=-49.08

The maximum stress calculated by in-plane stability (N/mm*mm) =131.36

The calculated maximum stress ratio in plane stability =0.431

The combination number corresponding to the maximum stress calculated for stability outside the plane :137, M=43.34, N=57.08, M=-128.60, N=-55.20

The maximum stress ratio calculated out of plane stability =0.354

Door gauge gb51022-2015 allowable thickness ratio of web [H0/TW] =250.00

Flange tolerance width ratio [B/T] =12.38

After buckling, the calculated stress ratio of strength was 0.768 < 1.0

The calculated stress ratio of shear strength =0.128 < 1.0

The calculated maximum stress in plane stability is < f=305.00

The maximum stress ratio calculated for stability outside the plane is < 1.0

Web height/thickness ratio H0/TW=63.33 < [H0/TW]=250.00

Flange width ratio B/T =12.20 < [B/T]=12.38

Compression bar, the internal length - to - thin ratio of plane lambda =56. ≤ [lambda]=180

Pressure bar, plane length of the slender ratio of lambda =42. ≤ [lambda]=180

Component weight (Kg)=142.87

Steel column 4     The design results

Section type =16;   Setting Angle =0;   Calculated length: Lx=9.68, Ly=2.50;   Length ratio: lambda x=56.0, lambda y=41.8

Component length =2.50;   Calculate length coefficient: Ux=3.87 Uy=1.00

Seismic rating: grade 3

Cross section parameters: B1=250, B2=250, H=400, Tw=6, T1=10, T2=10

Classification of axial compression section: X-axis :b, Y-axis :c

Steel number of component: Q345

Check calculation specification: door gb51022-2015

| \ | I end |  |  | II end |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| combination | M | N | V | M | N | V |
| 1 | 24.14 | 32.69 | 10.96 | 99.17 | 42.45 | 21.03 |
| 2 | 46.61 | 44.57 | 21.03 | 51.55 | 30.57 | 10.96 |
| 3 | 45.21 | 48.85 | 20.65 | 96.84 | 46.73 | 20.65 |
| 4 | 25.54 | 28.41 | 11.34 | 53.88 | 26.29 | 11.34 |
| 5 | 20.50 | 31.26 | 9.34 | 111.90 | 46.34 | 23.72 |
| 6 | 52.60 | 48.23 | 23.72 | 43.86 | 29.38 | 9.34 |
| 7 | 50.60 | 54.34 | 23.19 | 108.57 | 52.46 | 23.19 |
| 8 | 22.50 | 25.15 | 9.88 | 47.20 | 23.26 | 9.88 |
| 9 | 16.67 | 27.02 | 7.61 | 103.72 | 42.42 | 21.98 |
| 10 | 48.77 | 43.99 | 21.98 | 35.69 | 25.45 | 7.61 |
| 11 | 46.77 | 50.11 | 21.45 | 100.39 | 48.53 | 21.45 |
| 12 | 18.67 | 20.91 | 8.14 | 39.02 | 19.34 | 8.14 |
| 13 | 63.39 | 79.26 | 27.61 | 146.86 | 81.15 | 39.16 |
| 14 | 97.53 | 72.86 | 50.96 | 210.49 | 74.75 | 39.41 |
| 15 | 41.83 | 79.92 | 9.16 | 79.15 | 81.81 | 20.71 |
| 16 | 67.82 | 37.38 | 44.75 | 165.25 | 39.26 | 33.20 |
| 17 | 67.23 | 83.50 | 29.34 | 155.03 | 85.07 | 40.89 |
| 18 | 101.37 | 77.10 | 52.69 | 218.66 | 78.67 | 41.14 |
| 19 | 45.66 | 84.16 | 10.89 | 87.33 | 85.73 | 22.44 |
| 20 | 71.66 | 41.62 | 46.48 | 173.43 | 43.19 | 34.93 |
| 21 | 31.35 | 31.56 | 13.47 | 5.64 | 16.47 | 6.02 |
| 22 | 51.83 | 27.72 | 27.47 | 43.82 | 12.63 | 6.17 |
| 23 | 18.41 | 31.95 | 2.39 | 34.98 | 16.87 | 5.05 |
| 24 | 34.00 | 6.43 | 23.75 | 16.68 | 8.66 | 2.44 |
| 25 | 0.75 | 14.59 | 0.91 | 73.68 | 33.44 | 20.40 |
| 26 | 19.73 | 10.75 | 13.10 | 111.85 | 29.60 | 20.54 |
| 27 | 13.69 | 14.99 | 11.98 | 33.05 | 33.84 | 9.32 |
| 28 | 1.90 | 10.54 | 9.37 | 84.71 | 8.31 | 16.82 |
| 29 | 1.25 | 8.47 | 0.37 | 8.97 | 10.36 | 6.56 |
| 30 | 21.73 | 4.63 | 13.63 | 47.15 | 6.52 | 6.70 |
| 31 | 11.69 | 8.87 | 11.45 | 31.65 | 10.76 | 4.52 |
| 32 | 3.90 | 16.66 | 9.91 | 20.01 | 14.77 | 2.98 |
| 33 | 29.35 | 37.67 | 12.93 | 70.35 | 39.56 | 19.86 |
| 34 | 49.83 | 33.83 | 26.94 | 108.52 | 35.72 | 20.01 |
| 35 | 16.41 | 38.07 | 1.86 | 29.72 | 39.95 | 8.79 |
| 36 | 32.00 | 12.54 | 23.22 | 81.38 | 14.43 | 16.29 |
| 37 | 35.18 | 35.80 | 15.20 | 13.82 | 20.40 | 7.76 |
| 38 | 55.67 | 31.96 | 29.21 | 51.99 | 16.56 | 7.91 |
| 39 | 22.24 | 36.19 | 4.13 | 26.81 | 20.80 | 3.31 |
| 40 | 37.84 | 10.67 | 25.48 | 24.85 | 4.73 | 4.18 |
| 41 | 3.08 | 18.83 | 0.83 | 81.85 | 37.37 | 22.13 |
| 42 | 23.57 | 14.99 | 14.84 | 120.03 | 33.53 | 22.28 |
| 43 | 9.86 | 19.22 | 10.24 | 41.23 | 37.76 | 11.06 |
| 44 | 5.74 | 6.30 | 11.11 | 92.89 | 12.24 | 18.55 |
| 45 | 5.08 | 12.71 | 1.36 | 17.15 | 14.28 | 8.29 |
| 46 | 25.57 | 8.87 | 15.37 | 55.32 | 10.45 | 8.44 |
| 47 | 7.86 | 13.11 | 9.71 | 23.47 | 14.68 | 2.78 |
| 48 | 7.74 | 12.42 | 11.64 | 28.18 | 10.85 | 4.71 |
| 49 | 33.18 | 41.91 | 14.67 | 78.52 | 43.48 | 21.60 |
| 50 | 53.67 | 38.07 | 28.68 | 116.70 | 39.64 | 21.75 |
| 51 | 20.24 | 42.31 | 3.60 | 37.90 | 43.88 | 10.53 |
| 52 | 35.84 | 16.78 | 24.95 | 89.56 | 18.35 | 18.02 |
| 53 | 65.16 | 75.18 | 28.36 | 102.86 | 65.19 | 29.84 |
| 54 | 99.30 | 68.78 | 51.70 | 166.49 | 58.79 | 30.09 |
| 55 | 43.59 | 75.84 | 9.90 | 35.16 | 65.85 | 11.39 |
| 56 | 69.59 | 33.30 | 45.49 | 121.25 | 23.31 | 23.88 |
| 57 | 42.69 | 63.30 | 18.29 | 150.48 | 77.07 | 39.91 |
| 58 | 76.83 | 56.91 | 41.64 | 214.11 | 70.67 | 40.15 |
| 59 | 21.12 | 63.97 | 0.16 | 82.78 | 77.73 | 21.45 |
| 60 | 47.12 | 21.42 | 35.43 | 168.88 | 35.19 | 33.94 |
| 61 | 44.09 | 59.02 | 18.67 | 105.19 | 60.91 | 30.22 |
| 62 | 78.23 | 52.63 | 42.01 | 168.82 | 54.51 | 30.46 |
| 63 | 22.52 | 59.69 | 0.21 | 37.49 | 61.57 | 11.76 |
| 64 | 48.52 | 17.14 | 35.80 | 123.58 | 19.03 | 24.25 |
| 65 | 63.76 | 79.46 | 27.98 | 148.15 | 81.35 | 39.53 |
| 66 | 97.90 | 73.06 | 51.33 | 211.78 | 74.95 | 39.78 |
| 67 | 42.19 | 80.13 | 9.53 | 80.45 | 82.01 | 21.08 |
| 68 | 68.19 | 37.58 | 45.12 | 166.55 | 39.47 | 33.57 |
| 69 | 68.99 | 79.42 | 30.09 | 111.03 | 69.12 | 31.58 |
| 70 | 103.13 | 73.02 | 53.44 | 174.66 | 62.72 | 31.82 |
| 71 | 47.43 | 80.08 | 11.64 | 43.33 | 69.78 | 13.12 |
| 72 | 73.42 | 37.54 | 47.23 | 129.43 | 27.23 | 25.61 |
| 73 | 46.52 | 67.54 | 20.03 | 158.66 | 80.99 | 41.64 |
| 74 | 80.66 | 61.15 | 43.37 | 222.29 | 74.60 | 41.89 |
| 75 | 24.96 | 68.21 | 1.57 | 90.95 | 81.66 | 23.19 |
| 76 | 50.95 | 25.66 | 37.16 | 177.05 | 39.11 | 35.68 |
| 77 | 47.92 | 63.26 | 20.40 | 113.36 | 64.83 | 31.95 |
| 78 | 82.06 | 56.87 | 43.75 | 176.99 | 58.44 | 32.20 |
| 79 | 26.36 | 63.93 | 1.95 | 45.66 | 65.50 | 13.50 |
| 80 | 52.35 | 21.38 | 37.54 | 131.76 | 22.95 | 25.99 |
| 81 | 67.59 | 83.70 | 29.72 | 156.33 | 85.27 | 41.27 |
| 82 | 101.73 | 77.30 | 53.06 | 219.96 | 78.88 | 41.51 |
| 83 | 46.03 | 84.36 | 11.26 | 88.62 | 85.94 | 22.81 |
| 84 | 72.02 | 41.82 | 46.85 | 174.72 | 43.39 | 35.30 |
| 85 | 29.87 | 29.75 | 13.29 | 92.06 | 35.14 | 19.45 |
| 86 | 13.54 | 26.11 | 6.69 | 59.89 | 31.50 | 12.85 |
| 87 | 43.63 | 37.02 | 19.45 | 62.90 | 27.87 | 13.29 |
| 88 | 27.30 | 33.39 | 12.85 | 30.73 | 24.23 | 6.69 |
| 89 | 42.77 | 39.64 | 19.22 | 90.63 | 37.76 | 19.22 |
| 90 | 26.44 | 36.01 | 12.63 | 58.46 | 34.12 | 12.63 |
| 91 | 30.73 | 27.13 | 13.52 | 64.33 | 25.25 | 13.52 |
| 92 | 14.40 | 23.49 | 6.92 | 32.16 | 21.61 | 6.92 |
| 93 | 26.21 | 25.10 | 11.63 | 79.39 | 29.59 | 16.76 |
| 94 | 9.88 | 21.46 | 5.03 | 47.22 | 25.95 | 10.17 |
| 95 | 37.68 | 31.16 | 16.76 | 55.10 | 23.53 | 11.63 |
| 96 | 21.35 | 27.52 | 10.17 | 22.93 | 19.89 | 5.03 |
| 97 | 36.96 | 33.34 | 16.57 | 78.20 | 31.77 | 16.57 |
| 98 | 20.63 | 29.70 | 9.98 | 46.03 | 28.13 | 9.98 |
| 99 | 26.93 | 22.91 | 11.82 | 56.29 | 21.34 | 11.82 |
| 100 | 10.60 | 19.28 | 5.22 | 24.11 | 17.70 | 5.22 |
| 101 | 46.58 | 21.52 | 16.76 | 7.24 | 19.64 | 23.77 |
| 102 | 23.02 | 25.44 | 10.41 | 49.05 | 23.55 | 10.41 |
| 103 | 12.64 | 29.35 | 30.77 | 77.66 | 27.46 | 23.77 |
| 104 | 12.64 | 29.35 | 30.77 | 77.66 | 27.46 | 23.77 |
| 105 | 46.58 | 21.52 | 16.76 | 7.24 | 19.64 | 23.77 |
| 106 | 46.58 | 21.52 | 16.76 | 7.24 | 19.64 | 23.77 |
| 107 | 12.64 | 29.35 | 30.77 | 77.66 | 27.46 | 23.77 |
| 108 | 46.58 | 21.52 | 16.76 | 7.24 | 19.64 | 23.77 |
| 109 | 50.42 | 17.28 | 15.03 | 0.94 | 15.71 | 22.03 |
| 110 | 19.19 | 21.20 | 8.67 | 40.87 | 19.63 | 8.67 |
| 111 | 8.81 | 25.11 | 29.03 | 69.49 | 23.54 | 22.03 |
| 112 | 8.81 | 25.11 | 29.03 | 69.49 | 23.54 | 22.03 |
| 113 | 50.42 | 17.28 | 15.03 | 0.94 | 15.71 | 22.03 |
| 114 | 50.42 | 17.28 | 15.03 | 0.94 | 15.71 | 22.03 |
| 115 | 8.81 | 25.11 | 29.03 | 69.49 | 23.54 | 22.03 |
| 116 | 50.42 | 17.28 | 15.03 | 0.94 | 15.71 | 22.03 |
| 117 | 48.34 | 25.60 | 16.02 | 51.23 | 35.59 | 33.08 |
| 118 | 10.88 | 33.43 | 30.02 | 121.66 | 43.42 | 33.08 |
| 119 | 43.73 | 41.39 | 19.73 | 45.42 | 27.63 | 9.66 |
| 120 | 25.87 | 37.48 | 26.08 | 3.61 | 23.72 | 23.02 |
| 121 | 31.95 | 49.58 | 39.71 | 119.33 | 47.70 | 32.71 |
| 122 | 31.95 | 49.58 | 39.71 | 119.33 | 47.70 | 32.71 |
| 123 | 46.94 | 21.32 | 16.39 | 5.94 | 19.44 | 23.39 |
| 124 | 46.94 | 21.32 | 16.39 | 5.94 | 19.44 | 23.39 |
| 125 | 52.18 | 21.36 | 14.28 | 43.06 | 31.67 | 31.35 |
| 126 | 7.04 | 29.19 | 28.29 | 113.48 | 39.49 | 31.35 |
| 127 | 39.89 | 37.15 | 17.99 | 37.24 | 23.70 | 7.93 |
| 128 | 29.71 | 33.24 | 24.35 | 4.57 | 19.79 | 21.29 |
| 129 | 28.11 | 45.35 | 37.98 | 111.15 | 43.77 | 30.97 |
| 130 | 28.11 | 45.35 | 37.98 | 111.15 | 43.77 | 30.97 |
| 131 | 50.78 | 17.08 | 14.66 | 2.23 | 15.51 | 21.66 |
| 132 | 50.78 | 17.08 | 14.66 | 2.23 | 15.51 | 21.66 |
| 133 | 28.22 | 28.52 | 13.79 | 82.63 | 43.61 | 33.07 |
| 134 | 13.24 | 34.00 | 23.60 | 131.93 | 49.08 | 33.07 |
| 135 | 52.60 | 48.23 | 23.72 | 43.86 | 29.38 | 9.34 |
| 136 | 3.88 | 45.49 | 28.17 | 14.60 | 26.64 | 18.69 |
| 137 | 43.34 | 57.08 | 37.44 | 128.60 | 55.20 | 32.54 |
| 138 | 43.34 | 57.08 | 37.44 | 128.60 | 55.20 | 32.54 |
| 139 | 26.22 | 22.41 | 14.33 | 17.93 | 20.52 | 19.23 |
| 140 | 26.22 | 22.41 | 14.33 | 17.93 | 20.52 | 19.23 |
| 141 | 32.05 | 24.28 | 12.06 | 74.46 | 39.68 | 31.33 |
| 142 | 9.40 | 29.76 | 21.86 | 123.75 | 45.16 | 31.33 |
| 143 | 48.77 | 43.99 | 21.98 | 35.69 | 25.45 | 7.61 |
| 144 | 0.05 | 41.25 | 26.43 | 6.42 | 22.71 | 16.96 |
| 145 | 39.50 | 52.84 | 35.70 | 120.42 | 51.27 | 30.80 |
| 146 | 39.50 | 52.84 | 35.70 | 120.42 | 51.27 | 30.80 |
| 147 | 30.05 | 18.17 | 12.59 | 9.75 | 16.60 | 17.49 |
| 148 | 30.05 | 18.17 | 12.59 | 9.75 | 16.60 | 17.49 |
| 149 | 100.19 | 37.22 | 6.79 | 66.31 | 27.22 | 3.34 |
| 150 | 40.97 | 29.39 | 7.21 | 4.12 | 19.40 | 3.34 |
| 151 | 120.67 | 33.38 | 20.80 | 104.49 | 23.39 | 3.19 |
| 152 | 61.45 | 25.55 | 6.80 | 34.06 | 15.56 | 3.19 |
| 153 | 87.25 | 37.61 | 4.28 | 25.69 | 27.62 | 14.41 |
| 154 | 28.03 | 29.79 | 18.28 | 44.74 | 19.80 | 14.41 |
| 155 | 102.85 | 12.09 | 17.07 | 77.35 | 2.09 | 6.92 |
| 156 | 43.63 | 4.26 | 3.07 | 6.92 | 5.73 | 6.92 |
| 157 | 8.12 | 21.43 | 3.09 | 72.12 | 35.19 | 20.08 |
| 158 | 77.72 | 25.34 | 3.27 | 113.93 | 39.10 | 6.72 |
| 159 | 28.60 | 17.59 | 17.09 | 110.30 | 31.35 | 20.23 |
| 160 | 98.21 | 21.50 | 10.74 | 152.11 | 35.26 | 6.87 |
| 161 | 4.82 | 21.82 | 7.99 | 31.50 | 35.59 | 9.00 |
| 162 | 64.78 | 25.74 | 14.34 | 73.31 | 39.50 | 4.35 |
| 163 | 10.78 | 3.70 | 13.37 | 83.16 | 10.06 | 16.50 |
| 164 | 80.38 | 0.21 | 7.01 | 124.97 | 13.97 | 3.14 |
| 165 | 19.90 | 13.23 | 16.90 | 1.79 | 15.12 | 2.97 |
| 166 | 19.90 | 13.23 | 16.90 | 1.79 | 15.12 | 2.97 |
| 167 | 40.38 | 9.39 | 2.89 | 36.39 | 11.28 | 2.82 |
| 168 | 40.38 | 9.39 | 2.89 | 36.39 | 11.28 | 2.82 |
| 169 | 6.96 | 13.63 | 27.97 | 42.41 | 15.52 | 14.04 |
| 170 | 6.96 | 13.63 | 27.97 | 42.41 | 15.52 | 14.04 |
| 171 | 22.56 | 11.90 | 6.62 | 9.25 | 10.01 | 6.55 |
| 172 | 22.56 | 11.90 | 6.62 | 9.25 | 10.01 | 6.55 |
| 173 | 98.79 | 41.50 | 6.42 | 111.60 | 43.38 | 6.35 |
| 174 | 98.79 | 41.50 | 6.42 | 111.60 | 43.38 | 6.35 |
| 175 | 119.27 | 37.66 | 20.43 | 149.78 | 39.54 | 6.50 |
| 176 | 119.27 | 37.66 | 20.43 | 149.78 | 39.54 | 6.50 |
| 177 | 85.85 | 41.89 | 4.65 | 70.98 | 43.78 | 4.72 |
| 178 | 85.85 | 41.89 | 4.65 | 70.98 | 43.78 | 4.72 |
| 179 | 101.45 | 16.37 | 16.70 | 122.64 | 18.25 | 2.77 |
| 180 | 101.45 | 16.37 | 16.70 | 122.64 | 18.25 | 2.77 |
| 181 | 104.03 | 41.46 | 8.53 | 74.48 | 31.15 | 1.61 |
| 182 | 44.81 | 33.63 | 5.48 | 4.06 | 23.32 | 1.61 |
| 183 | 124.51 | 37.62 | 22.54 | 112.66 | 27.31 | 1.46 |
| 184 | 65.29 | 29.79 | 8.53 | 42.24 | 19.49 | 1.46 |
| 185 | 91.09 | 41.85 | 2.55 | 33.86 | 31.55 | 12.68 |
| 186 | 31.87 | 34.03 | 16.55 | 36.56 | 23.72 | 12.68 |
| 187 | 106.69 | 16.33 | 18.81 | 85.52 | 6.02 | 5.18 |
| 188 | 47.46 | 8.50 | 4.81 | 15.09 | 1.81 | 5.18 |
| 189 | 11.96 | 25.67 | 4.82 | 80.30 | 39.11 | 21.81 |
| 190 | 81.56 | 29.58 | 1.53 | 122.11 | 43.03 | 8.46 |
| 191 | 32.44 | 21.83 | 18.83 | 118.47 | 35.28 | 21.96 |
| 192 | 102.04 | 25.74 | 12.47 | 160.28 | 39.19 | 8.60 |
| 193 | 0.98 | 26.06 | 6.25 | 39.67 | 39.51 | 10.74 |
| 194 | 68.62 | 29.98 | 12.61 | 81.48 | 43.42 | 2.62 |
| 195 | 14.62 | 0.54 | 15.10 | 91.33 | 13.99 | 18.23 |
| 196 | 84.22 | 4.45 | 8.75 | 133.14 | 17.90 | 4.88 |
| 197 | 23.74 | 17.47 | 15.17 | 6.39 | 19.04 | 1.23 |
| 198 | 23.74 | 17.47 | 15.17 | 6.39 | 19.04 | 1.23 |
| 199 | 44.22 | 13.63 | 1.16 | 44.57 | 15.21 | 1.09 |
| 200 | 44.22 | 13.63 | 1.16 | 44.57 | 15.21 | 1.09 |
| 201 | 10.80 | 17.87 | 26.24 | 34.23 | 19.44 | 12.31 |
| 202 | 10.80 | 17.87 | 26.24 | 34.23 | 19.44 | 12.31 |
| 203 | 26.39 | 7.66 | 4.88 | 17.43 | 6.09 | 4.81 |
| 204 | 26.39 | 7.66 | 4.88 | 17.43 | 6.09 | 4.81 |
| 205 | 102.63 | 45.74 | 8.16 | 119.77 | 47.31 | 8.08 |
| 206 | 102.63 | 45.74 | 8.16 | 119.77 | 47.31 | 8.08 |
| 207 | 123.11 | 41.90 | 22.16 | 157.95 | 43.47 | 8.23 |
| 208 | 123.11 | 41.90 | 22.16 | 157.95 | 43.47 | 8.23 |
| 209 | 89.69 | 46.13 | 2.92 | 79.15 | 47.71 | 2.99 |
| 210 | 89.69 | 46.13 | 2.92 | 79.15 | 47.71 | 2.99 |
| 211 | 105.29 | 20.61 | 18.44 | 130.81 | 22.18 | 4.50 |
| 212 | 105.29 | 20.61 | 18.44 | 130.81 | 22.18 | 4.50 |
| 213 | 113.88 | 77.92 | 23.91 | 132.13 | 67.93 | 20.49 |
| 214 | 72.42 | 72.44 | 14.10 | 82.83 | 62.45 | 20.49 |
| 215 | 148.02 | 71.52 | 47.25 | 195.76 | 61.53 | 20.74 |
| 216 | 106.56 | 66.05 | 37.45 | 146.46 | 56.05 | 20.74 |
| 217 | 92.31 | 78.58 | 5.45 | 64.42 | 68.59 | 2.04 |
| 218 | 50.86 | 73.11 | 4.35 | 15.13 | 63.11 | 2.04 |
| 219 | 118.31 | 36.04 | 41.04 | 150.52 | 26.05 | 14.53 |
| 220 | 76.85 | 30.56 | 31.24 | 101.22 | 20.57 | 14.53 |
| 221 | 42.69 | 63.30 | 18.29 | 150.48 | 77.07 | 39.91 |
| 222 | 91.41 | 66.04 | 13.85 | 179.75 | 79.81 | 30.56 |
| 223 | 76.83 | 56.91 | 41.64 | 214.11 | 70.67 | 40.15 |
| 224 | 125.55 | 59.65 | 37.19 | 243.38 | 73.41 | 30.80 |
| 225 | 21.12 | 63.97 | 0.16 | 82.78 | 77.73 | 21.45 |
| 226 | 69.84 | 66.71 | 4.61 | 112.05 | 80.47 | 12.10 |
| 227 | 47.12 | 21.42 | 35.43 | 168.88 | 35.19 | 33.94 |
| 228 | 95.84 | 24.16 | 30.98 | 198.14 | 37.92 | 24.59 |
| 229 | 51.35 | 56.29 | 4.41 | 85.16 | 58.17 | 20.87 |
| 230 | 51.35 | 56.29 | 4.41 | 85.16 | 58.17 | 20.87 |
| 231 | 85.49 | 49.89 | 27.76 | 148.79 | 51.77 | 21.11 |
| 232 | 85.49 | 49.89 | 27.76 | 148.79 | 51.77 | 21.11 |
| 233 | 29.79 | 56.95 | 14.04 | 17.46 | 58.83 | 2.41 |
| 234 | 29.79 | 56.95 | 14.04 | 17.46 | 58.83 | 2.41 |
| 235 | 55.78 | 14.40 | 21.55 | 103.55 | 16.29 | 14.90 |
| 236 | 55.78 | 14.40 | 21.55 | 103.55 | 16.29 | 14.90 |
| 237 | 112.48 | 82.20 | 23.53 | 177.42 | 84.09 | 30.18 |
| 238 | 112.48 | 82.20 | 23.53 | 177.42 | 84.09 | 30.18 |
| 239 | 146.62 | 75.80 | 46.88 | 241.05 | 77.69 | 30.43 |
| 240 | 146.62 | 75.80 | 46.88 | 241.05 | 77.69 | 30.43 |
| 241 | 90.91 | 82.86 | 5.08 | 109.72 | 84.75 | 11.73 |
| 242 | 90.91 | 82.86 | 5.08 | 109.72 | 84.75 | 11.73 |
| 243 | 116.91 | 40.32 | 40.67 | 195.81 | 42.20 | 24.22 |
| 244 | 116.91 | 40.32 | 40.67 | 195.81 | 42.20 | 24.22 |
| 245 | 117.71 | 82.16 | 25.64 | 140.30 | 71.85 | 22.23 |
| 246 | 76.26 | 76.68 | 15.84 | 91.00 | 66.38 | 22.23 |
| 247 | 151.85 | 75.76 | 48.99 | 203.93 | 65.46 | 22.47 |
| 248 | 110.40 | 70.28 | 39.19 | 154.63 | 59.98 | 22.47 |
| 249 | 96.15 | 82.82 | 7.19 | 72.60 | 72.52 | 3.77 |
| 250 | 54.69 | 77.35 | 2.62 | 23.30 | 67.04 | 3.77 |
| 251 | 122.14 | 40.28 | 42.78 | 158.69 | 29.97 | 16.26 |
| 252 | 80.69 | 34.80 | 32.97 | 109.40 | 24.49 | 16.26 |
| 253 | 46.52 | 67.54 | 20.03 | 158.66 | 80.99 | 41.64 |
| 254 | 95.25 | 70.28 | 15.58 | 187.92 | 83.73 | 32.29 |
| 255 | 80.66 | 61.15 | 43.37 | 222.29 | 74.60 | 41.89 |
| 256 | 129.38 | 63.88 | 38.93 | 251.55 | 77.33 | 32.54 |
| 257 | 24.96 | 68.21 | 1.57 | 90.95 | 81.66 | 23.19 |
| 258 | 73.68 | 70.95 | 2.87 | 120.22 | 84.39 | 13.84 |
| 259 | 50.95 | 25.66 | 37.16 | 177.05 | 39.11 | 35.68 |
| 260 | 99.67 | 28.40 | 32.72 | 206.32 | 41.85 | 26.33 |
| 261 | 55.19 | 60.52 | 6.15 | 93.33 | 62.10 | 22.60 |
| 262 | 55.19 | 60.52 | 6.15 | 93.33 | 62.10 | 22.60 |
| 263 | 89.33 | 54.13 | 29.50 | 156.96 | 55.70 | 22.85 |
| 264 | 89.33 | 54.13 | 29.50 | 156.96 | 55.70 | 22.85 |
| 265 | 33.62 | 61.19 | 12.30 | 25.63 | 62.76 | 4.15 |
| 266 | 33.62 | 61.19 | 12.30 | 25.63 | 62.76 | 4.15 |
| 267 | 59.62 | 18.64 | 23.29 | 111.73 | 20.21 | 16.64 |
| 268 | 59.62 | 18.64 | 23.29 | 111.73 | 20.21 | 16.64 |
| 269 | 116.31 | 86.44 | 25.27 | 185.59 | 88.01 | 31.92 |
| 270 | 116.31 | 86.44 | 25.27 | 185.59 | 88.01 | 31.92 |
| 271 | 150.45 | 80.04 | 48.62 | 249.22 | 81.61 | 32.16 |
| 272 | 150.45 | 80.04 | 48.62 | 249.22 | 81.61 | 32.16 |
| 273 | 94.75 | 87.10 | 6.81 | 117.89 | 88.67 | 13.46 |
| 274 | 94.75 | 87.10 | 6.81 | 117.89 | 88.67 | 13.46 |
| 275 | 120.74 | 44.56 | 42.41 | 203.99 | 46.13 | 25.95 |
| 276 | 120.74 | 44.56 | 42.41 | 203.99 | 46.13 | 25.95 |
| 277 | 80.07 | 34.29 | 9.02 | 34.91 | 19.21 | 3.33 |
| 278 | 38.61 | 28.82 | 0.78 | 14.39 | 13.73 | 3.33 |
| 279 | 100.55 | 30.46 | 23.03 | 73.09 | 15.37 | 3.18 |
| 280 | 59.09 | 24.98 | 13.22 | 23.79 | 9.90 | 3.18 |
| 281 | 67.13 | 34.69 | 2.05 | 5.71 | 19.61 | 14.40 |
| 282 | 25.67 | 29.21 | 11.86 | 55.01 | 14.13 | 14.40 |
| 283 | 82.72 | 9.17 | 19.30 | 45.95 | 5.92 | 6.90 |
| 284 | 41.27 | 3.69 | 9.50 | 3.35 | 11.39 | 6.90 |
| 285 | 0.75 | 14.59 | 0.91 | 73.68 | 33.44 | 20.40 |
| 286 | 47.97 | 17.33 | 5.36 | 102.94 | 36.18 | 11.05 |
| 287 | 19.73 | 10.75 | 13.10 | 111.85 | 29.60 | 20.54 |
| 288 | 68.45 | 13.49 | 8.65 | 141.12 | 32.34 | 11.20 |
| 289 | 13.69 | 14.99 | 11.98 | 33.05 | 33.84 | 9.32 |
| 290 | 35.03 | 17.72 | 16.43 | 62.32 | 36.58 | 0.03 |
| 291 | 1.90 | 10.54 | 9.37 | 84.71 | 8.31 | 16.82 |
| 292 | 50.62 | 7.80 | 4.93 | 113.98 | 11.05 | 7.47 |
| 293 | 8.51 | 5.73 | 14.63 | 11.06 | 7.62 | 2.79 |
| 294 | 8.51 | 5.73 | 14.63 | 11.06 | 7.62 | 2.79 |
| 295 | 29.00 | 1.90 | 0.62 | 27.12 | 3.78 | 2.65 |
| 296 | 29.00 | 1.90 | 0.62 | 27.12 | 3.78 | 2.65 |
| 297 | 4.43 | 6.13 | 25.70 | 51.68 | 8.02 | 13.87 |
| 298 | 4.43 | 6.13 | 25.70 | 51.68 | 8.02 | 13.87 |
| 299 | 11.17 | 19.40 | 4.34 | 0.02 | 17.51 | 6.37 |
| 300 | 11.17 | 19.40 | 4.34 | 0.02 | 17.51 | 6.37 |
| 301 | 78.07 | 40.41 | 8.49 | 99.61 | 42.30 | 10.51 |
| 302 | 78.07 | 40.41 | 8.49 | 99.61 | 42.30 | 10.51 |
| 303 | 98.55 | 36.57 | 22.49 | 137.79 | 38.46 | 10.66 |
| 304 | 98.55 | 36.57 | 22.49 | 137.79 | 38.46 | 10.66 |
| 305 | 65.13 | 40.81 | 2.59 | 58.99 | 42.69 | 0.56 |
| 306 | 65.13 | 40.81 | 2.59 | 58.99 | 42.69 | 0.56 |
| 307 | 80.72 | 15.28 | 18.77 | 110.65 | 17.17 | 6.94 |
| 308 | 80.72 | 15.28 | 18.77 | 110.65 | 17.17 | 6.94 |
| 309 | 83.90 | 38.53 | 10.75 | 43.08 | 23.14 | 1.59 |
| 310 | 42.45 | 33.06 | 0.95 | 6.21 | 17.66 | 1.59 |
| 311 | 104.39 | 34.70 | 24.76 | 81.26 | 19.30 | 1.44 |
| 312 | 62.93 | 29.22 | 14.96 | 31.96 | 13.82 | 1.44 |
| 313 | 70.96 | 38.93 | 0.32 | 2.46 | 23.54 | 12.66 |
| 314 | 29.51 | 33.45 | 10.12 | 46.84 | 18.06 | 12.66 |
| 315 | 86.56 | 13.40 | 21.04 | 54.12 | 1.99 | 5.17 |
| 316 | 45.11 | 7.93 | 11.23 | 4.82 | 7.47 | 5.17 |
| 317 | 3.08 | 18.83 | 0.83 | 81.85 | 37.37 | 22.13 |
| 318 | 51.80 | 21.57 | 3.62 | 111.12 | 40.11 | 12.78 |
| 319 | 23.57 | 14.99 | 14.84 | 120.03 | 33.53 | 22.28 |
| 320 | 72.29 | 17.73 | 10.39 | 149.30 | 36.27 | 12.93 |
| 321 | 9.86 | 19.22 | 10.24 | 41.23 | 37.76 | 11.06 |
| 322 | 38.86 | 21.96 | 14.69 | 70.50 | 40.50 | 1.71 |
| 323 | 5.74 | 6.30 | 11.11 | 92.89 | 12.24 | 18.55 |
| 324 | 54.46 | 3.56 | 6.66 | 122.15 | 14.98 | 9.20 |
| 325 | 12.35 | 9.97 | 12.89 | 2.88 | 11.55 | 1.06 |
| 326 | 12.35 | 9.97 | 12.89 | 2.88 | 11.55 | 1.06 |
| 327 | 32.83 | 6.13 | 1.12 | 35.29 | 7.71 | 0.91 |
| 328 | 32.83 | 6.13 | 1.12 | 35.29 | 7.71 | 0.91 |
| 329 | 0.59 | 10.37 | 23.96 | 43.51 | 11.94 | 12.13 |
| 330 | 0.59 | 10.37 | 23.96 | 43.51 | 11.94 | 12.13 |
| 331 | 15.01 | 15.16 | 2.61 | 8.15 | 13.58 | 4.64 |
| 332 | 15.01 | 15.16 | 2.61 | 8.15 | 13.58 | 4.64 |
| 333 | 81.90 | 44.65 | 10.22 | 107.79 | 46.22 | 12.25 |
| 334 | 81.90 | 44.65 | 10.22 | 107.79 | 46.22 | 12.25 |
| 335 | 102.39 | 40.81 | 24.23 | 145.96 | 42.38 | 12.40 |
| 336 | 102.39 | 40.81 | 24.23 | 145.96 | 42.38 | 12.40 |
| 337 | 68.96 | 45.05 | 0.85 | 67.17 | 46.62 | 1.18 |
| 338 | 68.96 | 45.05 | 0.85 | 67.17 | 46.62 | 1.18 |
| 339 | 84.56 | 19.52 | 20.50 | 118.82 | 21.09 | 8.67 |
| 340 | 84.56 | 19.52 | 20.50 | 118.82 | 21.09 | 8.67 |
| 341 | 2.90 | 28.66 | 19.01 | 79.40 | 34.05 | 25.17 |
| 342 | 13.43 | 25.02 | 12.42 | 47.23 | 30.41 | 18.58 |
| 343 | 22.57 | 30.84 | 19.01 | 99.06 | 36.23 | 25.17 |
| 344 | 6.24 | 27.21 | 12.42 | 66.89 | 32.59 | 18.58 |
| 345 | 43.63 | 37.02 | 19.45 | 62.90 | 27.87 | 13.29 |
| 346 | 27.30 | 33.39 | 12.85 | 30.73 | 24.23 | 6.69 |
| 347 | 16.66 | 35.93 | 25.17 | 50.24 | 26.77 | 19.01 |
| 348 | 0.33 | 32.29 | 18.58 | 18.07 | 23.14 | 12.42 |
| 349 | 35.47 | 40.74 | 24.95 | 97.64 | 38.85 | 24.95 |
| 350 | 19.14 | 37.10 | 18.35 | 65.47 | 35.21 | 18.35 |
| 351 | 35.47 | 40.74 | 24.95 | 97.64 | 38.85 | 24.95 |
| 352 | 19.14 | 37.10 | 18.35 | 65.47 | 35.21 | 18.35 |
| 353 | 3.76 | 26.04 | 19.24 | 51.67 | 24.15 | 19.24 |
| 354 | 12.57 | 22.40 | 12.65 | 19.50 | 20.52 | 12.65 |
| 355 | 3.76 | 26.04 | 19.24 | 51.67 | 24.15 | 19.24 |
| 356 | 12.57 | 22.40 | 12.65 | 19.50 | 20.52 | 12.65 |
| 357 | 3.74 | 24.19 | 16.40 | 68.84 | 28.67 | 21.53 |
| 358 | 12.59 | 20.55 | 9.81 | 36.67 | 25.04 | 14.94 |
| 359 | 20.13 | 26.01 | 16.40 | 85.23 | 30.50 | 21.53 |
| 360 | 3.80 | 22.37 | 9.81 | 53.06 | 26.86 | 14.94 |
| 361 | 37.68 | 31.16 | 16.76 | 55.10 | 23.53 | 11.63 |
| 362 | 21.35 | 27.52 | 10.17 | 22.93 | 19.89 | 5.03 |
| 363 | 15.20 | 30.25 | 21.53 | 44.55 | 22.61 | 16.40 |
| 364 | 1.13 | 26.61 | 14.94 | 12.37 | 18.98 | 9.81 |
| 365 | 30.88 | 34.25 | 21.34 | 84.04 | 32.68 | 21.34 |
| 366 | 14.55 | 30.61 | 14.75 | 51.87 | 29.04 | 14.75 |
| 367 | 30.88 | 34.25 | 21.34 | 84.04 | 32.68 | 21.34 |
| 368 | 14.55 | 30.61 | 14.75 | 51.87 | 29.04 | 14.75 |
| 369 | 4.45 | 22.00 | 16.59 | 45.73 | 20.43 | 16.59 |
| 370 | 11.88 | 18.36 | 10.00 | 13.56 | 16.79 | 10.00 |
| 371 | 4.45 | 22.00 | 16.59 | 45.73 | 20.43 | 16.59 |
| 372 | 11.88 | 18.36 | 10.00 | 13.56 | 16.79 | 10.00 |

Considering the strength after web buckling, the strength calculation control combination number :256, M=129.38, N=-63.88, M=-251.55, N=77.33

After buckling, the calculated stress ratio was 0.771

Shear strength calculation control combination number: 70, V=-53.44

The calculated stress ratio of shear strength =0.134

The combination number corresponding to the maximum stress calculated in plane stability is 134, M=-13.24, N=34.00, M=131.93, N=-49.08

The maximum stress calculated by in-plane stability (N/mm*mm) =131.36

The calculated maximum stress ratio in plane stability =0.431

The combination number corresponding to the maximum stress calculated for stability outside the plane :137, M=-43.34, N=57.08, M=128.60, N=-55.20

The maximum stress ratio calculated out of plane stability =0.354

Door gauge gb51022-2015 allowable thickness ratio of web [H0/TW] =250.00

Flange tolerance width ratio [B/T] =12.38

After buckling, the calculated stress ratio of strength was 0.771 < 1.0

The calculated stress ratio of shear strength =0.134 < 1.0

The calculated maximum stress in plane stability is < f=305.00

The maximum stress ratio calculated for stability outside the plane is < 1.0

Web height/thickness ratio H0/TW=63.33 < [H0/TW]=250.00

Flange width ratio B/T =12.20 < [B/T]=12.38

Compression bar, the internal length - to - thin ratio of plane lambda =56. ≤ [lambda]=180

Pressure bar, plane length of the slender ratio of lambda =42. ≤ [lambda]=180

Component weight (Kg)=142.87

Steel beam 1     The design results

Section type =16;   Setting Angle =0;   Calculated length: Lx=4.62, Ly=3.00

Component length =2.31;   Calculate length coefficient: Ux=2.00 Uy=1.30

Seismic rating: grade 3

Cross section parameters: B1=200, B2=200, H=200, Tw=6, T1=8, T2=8

Classification of axial compression section: X-axis :b, Y-axis :c

Steel number of component: Q345

Check calculation specification: door gb51022-2015

| \ | I end |  |  | II end |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| combination | M | N | V | M | N | V |
| 1 | 0.00 | 0.00 | 0.00 | 5.64 | 0.49 | 4.88 |
| 2 | 0.00 | 0.00 | 0.00 | 10.33 | 0.89 | 8.94 |
| 3 | 0.00 | 0.00 | 0.00 | 10.33 | 0.89 | 8.94 |
| 4 | 0.00 | 0.00 | 0.00 | 5.64 | 0.49 | 4.88 |
| 5 | 0.00 | 0.00 | 0.00 | 5.01 | 0.43 | 4.34 |
| 6 | 0.00 | 0.00 | 0.00 | 11.71 | 1.01 | 10.13 |
| 7 | 0.00 | 0.00 | 0.00 | 11.71 | 1.01 | 10.13 |
| 8 | 0.00 | 0.00 | 0.00 | 5.01 | 0.43 | 4.34 |
| 9 | 0.00 | 0.00 | 0.00 | 4.18 | 0.36 | 3.61 |
| 10 | 0.00 | 0.00 | 0.00 | 10.88 | 0.94 | 9.41 |
| 11 | 0.00 | 0.00 | 0.00 | 10.88 | 0.94 | 9.41 |
| 12 | 0.00 | 0.00 | 0.00 | 4.18 | 0.36 | 3.61 |
| 13 | 0.00 | 0.00 | 0.00 | 24.61 | 0.43 | 21.29 |
| 14 | 0.00 | 0.00 | 0.00 | 19.67 | 0.43 | 17.02 |
| 15 | 0.00 | 0.00 | 0.00 | 4.86 | 0.43 | 4.21 |
| 16 | 0.00 | 0.00 | 0.00 | 24.61 | 0.44 | 21.29 |
| 17 | 0.00 | 0.00 | 0.00 | 25.44 | 0.36 | 22.01 |
| 18 | 0.00 | 0.00 | 0.00 | 20.51 | 0.36 | 17.74 |
| 19 | 0.00 | 0.00 | 0.00 | 5.70 | 0.36 | 4.93 |
| 20 | 0.00 | 0.00 | 0.00 | 25.44 | 0.36 | 22.01 |
| 21 | 0.00 | 0.00 | 0.00 | 12.76 | 0.43 | 11.04 |
| 22 | 0.00 | 0.00 | 0.00 | 9.80 | 0.43 | 8.48 |
| 23 | 0.00 | 0.00 | 0.00 | 0.91 | 0.43 | 0.79 |
| 24 | 0.00 | 0.00 | 0.00 | 12.76 | 0.43 | 11.04 |
| 25 | 0.00 | 0.00 | 0.00 | 6.06 | 1.01 | 5.24 |
| 26 | 0.00 | 0.00 | 0.00 | 3.10 | 1.01 | 2.68 |
| 27 | 0.00 | 0.00 | 0.00 | 5.79 | 1.01 | 5.01 |
| 28 | 0.00 | 0.00 | 0.00 | 6.06 | 1.01 | 5.24 |
| 29 | 0.00 | 0.00 | 0.00 | 6.06 | 1.01 | 5.24 |
| 30 | 0.00 | 0.00 | 0.00 | 3.10 | 1.01 | 2.68 |
| 31 | 0.00 | 0.00 | 0.00 | 5.79 | 1.01 | 5.01 |
| 32 | 0.00 | 0.00 | 0.00 | 6.06 | 1.01 | 5.24 |
| 33 | 0.00 | 0.00 | 0.00 | 12.76 | 0.43 | 11.04 |
| 34 | 0.00 | 0.00 | 0.00 | 9.80 | 0.43 | 8.48 |
| 35 | 0.00 | 0.00 | 0.00 | 0.91 | 0.43 | 0.79 |
| 36 | 0.00 | 0.00 | 0.00 | 12.76 | 0.43 | 11.04 |
| 37 | 0.00 | 0.00 | 0.00 | 13.59 | 0.36 | 11.76 |
| 38 | 0.00 | 0.00 | 0.00 | 10.63 | 0.36 | 9.20 |
| 39 | 0.00 | 0.00 | 0.00 | 1.75 | 0.36 | 1.51 |
| 40 | 0.00 | 0.00 | 0.00 | 13.60 | 0.36 | 11.76 |
| 41 | 0.00 | 0.00 | 0.00 | 6.90 | 0.94 | 5.97 |
| 42 | 0.00 | 0.00 | 0.00 | 3.93 | 0.94 | 3.40 |
| 43 | 0.00 | 0.00 | 0.00 | 4.95 | 0.94 | 4.28 |
| 44 | 0.00 | 0.00 | 0.00 | 6.90 | 0.94 | 5.97 |
| 45 | 0.00 | 0.00 | 0.00 | 6.90 | 0.94 | 5.97 |
| 46 | 0.00 | 0.00 | 0.00 | 3.93 | 0.94 | 3.40 |
| 47 | 0.00 | 0.00 | 0.00 | 4.95 | 0.94 | 4.28 |
| 48 | 0.00 | 0.00 | 0.00 | 6.90 | 0.94 | 5.97 |
| 49 | 0.00 | 0.00 | 0.00 | 13.59 | 0.36 | 11.76 |
| 50 | 0.00 | 0.00 | 0.00 | 10.63 | 0.36 | 9.20 |
| 51 | 0.00 | 0.00 | 0.00 | 1.75 | 0.36 | 1.51 |
| 52 | 0.00 | 0.00 | 0.00 | 13.59 | 0.36 | 11.76 |
| 53 | 0.00 | 0.00 | 0.00 | 24.61 | 0.43 | 21.29 |
| 54 | 0.00 | 0.00 | 0.00 | 19.67 | 0.43 | 17.02 |
| 55 | 0.00 | 0.00 | 0.00 | 4.86 | 0.43 | 4.21 |
| 56 | 0.00 | 0.00 | 0.00 | 24.61 | 0.44 | 21.29 |
| 57 | 0.00 | 0.00 | 0.00 | 19.92 | 0.84 | 17.23 |
| 58 | 0.00 | 0.00 | 0.00 | 14.98 | 0.84 | 12.96 |
| 59 | 0.00 | 0.00 | 0.00 | 0.17 | 0.84 | 0.15 |
| 60 | 0.00 | 0.00 | 0.00 | 19.92 | 0.84 | 17.23 |
| 61 | 0.00 | 0.00 | 0.00 | 19.92 | 0.84 | 17.23 |
| 62 | 0.00 | 0.00 | 0.00 | 14.98 | 0.84 | 12.96 |
| 63 | 0.00 | 0.00 | 0.00 | 0.17 | 0.84 | 0.15 |
| 64 | 0.00 | 0.00 | 0.00 | 19.92 | 0.84 | 17.23 |
| 65 | 0.00 | 0.00 | 0.00 | 24.61 | 0.43 | 21.29 |
| 66 | 0.00 | 0.00 | 0.00 | 19.67 | 0.43 | 17.02 |
| 67 | 0.00 | 0.00 | 0.00 | 4.86 | 0.43 | 4.21 |
| 68 | 0.00 | 0.00 | 0.00 | 24.61 | 0.44 | 21.29 |
| 69 | 0.00 | 0.00 | 0.00 | 25.44 | 0.36 | 22.01 |
| 70 | 0.00 | 0.00 | 0.00 | 20.51 | 0.36 | 17.74 |
| 71 | 0.00 | 0.00 | 0.00 | 5.70 | 0.36 | 4.93 |
| 72 | 0.00 | 0.00 | 0.00 | 25.44 | 0.36 | 22.01 |
| 73 | 0.00 | 0.00 | 0.00 | 20.75 | 0.77 | 17.96 |
| 74 | 0.00 | 0.00 | 0.00 | 15.82 | 0.77 | 13.69 |
| 75 | 0.00 | 0.00 | 0.00 | 1.01 | 0.77 | 0.87 |
| 76 | 0.00 | 0.00 | 0.00 | 20.75 | 0.77 | 17.96 |
| 77 | 0.00 | 0.00 | 0.00 | 20.75 | 0.77 | 17.96 |
| 78 | 0.00 | 0.00 | 0.00 | 15.82 | 0.77 | 13.69 |
| 79 | 0.00 | 0.00 | 0.00 | 1.01 | 0.77 | 0.87 |
| 80 | 0.00 | 0.00 | 0.00 | 20.75 | 0.77 | 17.96 |
| 81 | 0.00 | 0.00 | 0.00 | 25.44 | 0.36 | 22.01 |
| 82 | 0.00 | 0.00 | 0.00 | 20.51 | 0.36 | 17.74 |
| 83 | 0.00 | 0.00 | 0.00 | 5.70 | 0.36 | 4.93 |
| 84 | 0.00 | 0.00 | 0.00 | 25.44 | 0.36 | 22.01 |
| 85 | 0.00 | 1.30 | 0.13 | 5.31 | 0.87 | 4.47 |
| 86 | 0.00 | 1.30 | 0.13 | 4.71 | 1.74 | 4.21 |
| 87 | 0.00 | 1.30 | 0.13 | 8.19 | 0.62 | 6.95 |
| 88 | 0.00 | 1.30 | 0.13 | 7.58 | 1.99 | 6.69 |
| 89 | 0.00 | 1.30 | 0.13 | 8.19 | 0.62 | 6.95 |
| 90 | 0.00 | 1.30 | 0.13 | 7.58 | 1.99 | 6.69 |
| 91 | 0.00 | 1.30 | 0.13 | 5.31 | 0.87 | 4.47 |
| 92 | 0.00 | 1.30 | 0.13 | 4.71 | 1.74 | 4.21 |
| 93 | 0.00 | 1.30 | 0.13 | 4.48 | 0.94 | 3.75 |
| 94 | 0.00 | 1.30 | 0.13 | 3.88 | 1.67 | 3.48 |
| 95 | 0.00 | 1.30 | 0.13 | 6.87 | 0.74 | 5.82 |
| 96 | 0.00 | 1.30 | 0.13 | 6.27 | 1.87 | 5.55 |
| 97 | 0.00 | 1.30 | 0.13 | 6.87 | 0.74 | 5.82 |
| 98 | 0.00 | 1.30 | 0.13 | 6.27 | 1.87 | 5.55 |
| 99 | 0.00 | 1.30 | 0.13 | 4.48 | 0.94 | 3.75 |
| 100 | 0.00 | 1.30 | 0.13 | 3.88 | 1.67 | 3.48 |
| 101 | 0.00 | 0.00 | 0.00 | 5.01 | 0.43 | 4.34 |
| 102 | 0.00 | 0.00 | 0.00 | 5.01 | 0.43 | 4.34 |
| 103 | 0.00 | 0.00 | 0.00 | 4.18 | 0.36 | 3.61 |
| 104 | 0.00 | 0.00 | 0.00 | 4.18 | 0.36 | 3.61 |
| 105 | 0.00 | 0.00 | 0.00 | 5.01 | 0.43 | 4.34 |
| 106 | 0.00 | 0.00 | 0.00 | 9.70 | 0.84 | 8.39 |
| 107 | 0.00 | 0.00 | 0.00 | 4.18 | 0.36 | 3.61 |
| 108 | 0.00 | 0.00 | 0.00 | 8.87 | 0.77 | 7.67 |
| 109 | 0.00 | 0.00 | 0.00 | 5.01 | 0.43 | 4.34 |
| 110 | 0.00 | 0.00 | 0.00 | 11.71 | 1.01 | 10.13 |
| 111 | 0.00 | 0.00 | 0.00 | 4.18 | 0.36 | 3.61 |
| 112 | 0.00 | 0.00 | 0.00 | 10.88 | 0.94 | 9.41 |
| 113 | 0.00 | 0.00 | 0.00 | 12.76 | 0.43 | 11.04 |
| 114 | 0.00 | 0.00 | 0.00 | 9.80 | 0.43 | 8.48 |
| 115 | 0.00 | 0.00 | 0.00 | 0.91 | 0.43 | 0.79 |
| 116 | 0.00 | 0.00 | 0.00 | 12.76 | 0.43 | 11.04 |
| 117 | 0.00 | 0.00 | 0.00 | 8.07 | 0.84 | 6.98 |
| 118 | 0.00 | 0.00 | 0.00 | 5.11 | 0.84 | 4.42 |
| 119 | 0.00 | 0.00 | 0.00 | 3.78 | 0.84 | 3.27 |
| 120 | 0.00 | 0.00 | 0.00 | 8.07 | 0.84 | 6.98 |
| 121 | 0.00 | 0.00 | 0.00 | 13.59 | 0.36 | 11.76 |
| 122 | 0.00 | 0.00 | 0.00 | 10.63 | 0.36 | 9.20 |
| 123 | 0.00 | 0.00 | 0.00 | 1.75 | 0.36 | 1.51 |
| 124 | 0.00 | 0.00 | 0.00 | 13.60 | 0.36 | 11.76 |
| 125 | 0.00 | 0.00 | 0.00 | 8.91 | 0.77 | 7.71 |
| 126 | 0.00 | 0.00 | 0.00 | 5.94 | 0.77 | 5.14 |
| 127 | 0.00 | 0.00 | 0.00 | 2.94 | 0.77 | 2.55 |
| 128 | 0.00 | 0.00 | 0.00 | 8.91 | 0.77 | 7.71 |
| 129 | 0.00 | 0.00 | 0.00 | 24.61 | 0.43 | 21.29 |
| 130 | 0.00 | 0.00 | 0.00 | 19.67 | 0.43 | 17.02 |
| 131 | 0.00 | 0.00 | 0.00 | 4.86 | 0.43 | 4.21 |
| 132 | 0.00 | 0.00 | 0.00 | 24.61 | 0.44 | 21.29 |
| 133 | 0.00 | 0.00 | 0.00 | 19.92 | 0.84 | 17.23 |
| 134 | 0.00 | 0.00 | 0.00 | 14.98 | 0.84 | 12.96 |
| 135 | 0.00 | 0.00 | 0.00 | 0.17 | 0.84 | 0.15 |
| 136 | 0.00 | 0.00 | 0.00 | 19.92 | 0.84 | 17.23 |
| 137 | 0.00 | 0.00 | 0.00 | 25.44 | 0.36 | 22.01 |
| 138 | 0.00 | 0.00 | 0.00 | 20.51 | 0.36 | 17.74 |
| 139 | 0.00 | 0.00 | 0.00 | 5.70 | 0.36 | 4.93 |
| 140 | 0.00 | 0.00 | 0.00 | 25.44 | 0.36 | 22.01 |
| 141 | 0.00 | 0.00 | 0.00 | 20.75 | 0.77 | 17.96 |
| 142 | 0.00 | 0.00 | 0.00 | 15.82 | 0.77 | 13.69 |
| 143 | 0.00 | 0.00 | 0.00 | 1.01 | 0.77 | 0.87 |
| 144 | 0.00 | 0.00 | 0.00 | 20.75 | 0.77 | 17.96 |
| 145 | 0.00 | 0.00 | 0.00 | 12.76 | 0.43 | 11.04 |
| 146 | 0.00 | 0.00 | 0.00 | 9.80 | 0.43 | 8.48 |
| 147 | 0.00 | 0.00 | 0.00 | 0.91 | 0.43 | 0.79 |
| 148 | 0.00 | 0.00 | 0.00 | 12.76 | 0.43 | 11.04 |
| 149 | 0.00 | 0.00 | 0.00 | 6.06 | 1.01 | 5.24 |
| 150 | 0.00 | 0.00 | 0.00 | 3.10 | 1.01 | 2.68 |
| 151 | 0.00 | 0.00 | 0.00 | 5.79 | 1.01 | 5.01 |
| 152 | 0.00 | 0.00 | 0.00 | 6.06 | 1.01 | 5.24 |
| 153 | 0.00 | 0.00 | 0.00 | 13.59 | 0.36 | 11.76 |
| 154 | 0.00 | 0.00 | 0.00 | 10.63 | 0.36 | 9.20 |
| 155 | 0.00 | 0.00 | 0.00 | 1.75 | 0.36 | 1.51 |
| 156 | 0.00 | 0.00 | 0.00 | 13.60 | 0.36 | 11.76 |
| 157 | 0.00 | 0.00 | 0.00 | 6.90 | 0.94 | 5.97 |
| 158 | 0.00 | 0.00 | 0.00 | 3.93 | 0.94 | 3.40 |
| 159 | 0.00 | 0.00 | 0.00 | 4.95 | 0.94 | 4.28 |
| 160 | 0.00 | 0.00 | 0.00 | 6.90 | 0.94 | 5.97 |
| 161 | 0.00 | 1.30 | 0.13 | 5.31 | 0.87 | 4.47 |
| 162 | 0.00 | 1.30 | 0.13 | 4.71 | 1.74 | 4.21 |
| 163 | 0.00 | 1.30 | 0.13 | 8.19 | 0.62 | 6.95 |
| 164 | 0.00 | 1.30 | 0.13 | 7.58 | 1.99 | 6.69 |
| 165 | 0.00 | 1.30 | 0.13 | 4.48 | 0.94 | 3.75 |
| 166 | 0.00 | 1.30 | 0.13 | 3.88 | 1.67 | 3.48 |
| 167 | 0.00 | 1.30 | 0.13 | 6.87 | 0.74 | 5.82 |
| 168 | 0.00 | 1.30 | 0.13 | 6.27 | 1.87 | 5.55 |

The moment envelope of the beam

| The lower part of the beam is under tension |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | 0.00 | 0.71 | 2.83 | 6.36 | 11.31 | 17.67 | 25.44 |
| The upper part of the beam is under tension |  |  |  |  |  |  |  |
| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | 0.00 | 0.33 | 1.30 | 2.93 | 5.21 | 8.13 | 11.71 |

The stress ratio calculated after buckling was 0.269

The calculated stress ratio of shear strength =0.114

The maximum stable stress in plane (N/mm*mm) =78.35

The calculated maximum stress ratio in plane stability =0.257

The maximum stress ratio calculated out of plane stability =0.287

Stress ratio after buckling strength calculation =0.269 < 1.0

The calculated stress ratio of shear strength =0.114 < 1.0

The maximum stable stress in plane < f=305.00

The maximum stress ratio calculated out of plane stability is < 1.0 0.29

Web height/thickness ratio H0/TW=30.67 < [H0/TW]=250.00 (gb51022-2015)

Flange width ratio B/T =12.12 < [B/T] =12.38

(constant + live) beam deflection mm

| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.94 | 0.88 | 0.82 | 0.73 | 0.59 | 0.37 | 0.00 |

Maximum deflection value =0.00 maximum deflection/span =1/100000.

Initial value of slope of inclined beam: 1/10.00

Minimum slope value of deformed inclined beam: 1/10.11

Slope change rate of inclined beam after deformation =0.011 < 1/3

Component weight (Kg)=78.10

Steel beam 2     The design results

Section type =27;   Setting Angle =0;   Calculated length: Lx=18.09, Ly=3.00

Component length =9.04;   Calculate length coefficient: Ux=2.00 Uy=0.33

Seismic rating: grade 3

H: B1=200, B2=200, H1=600, H2=300, T1=6, T2=10, T3=10

Classification of axial compression section: X-axis :b, Y-axis :c

Steel number of component: Q345

Check calculation specification: door gb51022-2015

| \ | I end |  |  | II end |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| combination | M | N | V | M | N | V |
| 1 | 107.17 | 24.31 | 35.51 | 45.22 | 20.92 | 2.09 |
| 2 | 59.52 | 13.41 | 20.15 | 25.41 | 10.91 | 1.09 |
| 3 | 83.93 | 19.26 | 31.86 | 35.31 | 15.53 | 5.50 |
| 4 | 82.76 | 18.46 | 23.80 | 35.31 | 16.31 | 2.31 |
| 5 | 120.28 | 27.28 | 39.76 | 50.67 | 23.60 | 2.36 |
| 6 | 52.21 | 11.71 | 17.83 | 22.38 | 9.30 | 0.93 |
| 7 | 87.08 | 20.07 | 34.55 | 36.52 | 15.89 | 7.22 |
| 8 | 85.41 | 18.92 | 23.03 | 36.52 | 17.01 | 3.93 |
| 9 | 111.27 | 25.23 | 36.75 | 46.85 | 21.87 | 2.19 |
| 10 | 43.20 | 9.66 | 14.82 | 18.55 | 7.57 | 0.76 |
| 11 | 78.07 | 18.02 | 31.54 | 32.70 | 14.17 | 7.05 |
| 12 | 76.40 | 16.87 | 20.02 | 32.70 | 15.28 | 4.11 |
| 13 | 237.00 | 47.42 | 75.97 | 82.99 | 49.33 | 5.22 |
| 14 | 136.59 | 46.18 | 55.13 | 70.49 | 48.09 | 9.34 |
| 15 | 160.23 | 33.14 | 30.86 | 54.10 | 35.05 | 16.53 |
| 16 | 93.59 | 33.18 | 56.53 | 50.52 | 35.09 | 24.66 |
| 17 | 246.01 | 49.46 | 78.99 | 86.81 | 51.05 | 5.39 |
| 18 | 145.60 | 48.23 | 58.14 | 74.31 | 49.82 | 9.52 |
| 19 | 169.24 | 35.19 | 33.87 | 57.92 | 36.78 | 16.36 |
| 20 | 102.60 | 35.23 | 59.54 | 54.34 | 36.82 | 24.84 |
| 21 | 54.36 | 8.53 | 16.66 | 12.88 | 12.21 | 1.39 |
| 22 | 5.89 | 7.79 | 4.16 | 5.38 | 11.47 | 3.87 |
| 23 | 8.29 | 0.03 | 10.40 | 4.45 | 3.65 | 11.66 |
| 24 | 31.69 | 0.01 | 5.00 | 6.60 | 3.67 | 13.06 |
| 25 | 122.43 | 24.10 | 38.60 | 41.18 | 26.51 | 2.82 |
| 26 | 62.18 | 23.36 | 26.09 | 33.68 | 25.77 | 5.30 |
| 27 | 76.36 | 15.54 | 11.53 | 23.85 | 17.95 | 10.23 |
| 28 | 36.38 | 15.56 | 26.93 | 21.70 | 17.97 | 14.49 |
| 29 | 87.56 | 15.74 | 21.87 | 27.03 | 19.92 | 3.47 |
| 30 | 27.31 | 15.00 | 9.36 | 19.53 | 19.18 | 0.99 |
| 31 | 41.49 | 7.18 | 5.20 | 9.70 | 11.36 | 16.52 |
| 32 | 1.51 | 7.20 | 10.20 | 7.55 | 11.38 | 8.20 |
| 33 | 89.22 | 16.89 | 33.39 | 27.03 | 18.80 | 7.69 |
| 34 | 28.98 | 16.15 | 20.89 | 19.53 | 18.06 | 10.16 |
| 35 | 43.16 | 8.33 | 6.32 | 9.70 | 10.24 | 5.36 |
| 36 | 3.18 | 8.35 | 21.72 | 7.55 | 10.26 | 19.35 |
| 37 | 63.37 | 10.58 | 19.67 | 16.71 | 13.94 | 1.57 |
| 38 | 3.12 | 9.84 | 7.17 | 9.21 | 13.20 | 4.04 |
| 39 | 17.30 | 2.01 | 7.39 | 0.63 | 5.37 | 11.48 |
| 40 | 22.68 | 2.04 | 8.01 | 2.78 | 5.40 | 13.23 |
| 41 | 131.44 | 26.15 | 41.61 | 45.00 | 28.24 | 3.00 |
| 42 | 71.19 | 25.41 | 29.10 | 37.50 | 27.50 | 5.47 |
| 43 | 85.37 | 17.58 | 14.54 | 27.67 | 19.68 | 10.05 |
| 44 | 45.39 | 17.61 | 29.94 | 25.52 | 19.70 | 14.66 |
| 45 | 96.57 | 17.79 | 24.88 | 30.85 | 21.65 | 3.30 |
| 46 | 36.32 | 17.05 | 12.37 | 23.35 | 20.91 | 0.82 |
| 47 | 50.50 | 9.22 | 2.19 | 13.52 | 13.08 | 16.35 |
| 48 | 10.52 | 9.25 | 13.21 | 11.37 | 13.10 | 8.37 |
| 49 | 98.23 | 18.94 | 36.40 | 30.85 | 20.53 | 7.86 |
| 50 | 37.99 | 18.20 | 23.90 | 23.35 | 19.79 | 10.33 |
| 51 | 52.17 | 10.37 | 9.33 | 13.52 | 11.97 | 5.19 |
| 52 | 12.19 | 10.40 | 24.73 | 11.37 | 11.99 | 19.53 |
| 53 | 190.65 | 36.91 | 60.79 | 63.57 | 40.06 | 4.29 |
| 54 | 90.24 | 35.68 | 39.95 | 51.07 | 38.82 | 8.42 |
| 55 | 113.87 | 22.64 | 15.67 | 34.69 | 25.78 | 17.46 |
| 56 | 47.24 | 22.67 | 41.34 | 31.10 | 25.82 | 23.74 |
| 57 | 238.30 | 47.81 | 76.14 | 83.38 | 50.07 | 5.29 |
| 58 | 137.89 | 46.57 | 55.30 | 70.88 | 48.83 | 9.42 |
| 59 | 161.52 | 33.54 | 31.03 | 54.49 | 35.80 | 16.46 |
| 60 | 94.89 | 33.57 | 56.69 | 50.91 | 35.83 | 24.74 |
| 61 | 213.89 | 41.96 | 64.43 | 73.48 | 45.45 | 0.89 |
| 62 | 113.48 | 40.72 | 43.59 | 60.98 | 44.22 | 5.01 |
| 63 | 137.11 | 27.68 | 19.32 | 44.59 | 31.18 | 20.86 |
| 64 | 70.48 | 27.72 | 44.98 | 41.01 | 31.22 | 20.33 |
| 65 | 215.05 | 42.76 | 72.50 | 73.48 | 44.67 | 8.70 |
| 66 | 114.64 | 41.53 | 51.66 | 60.98 | 43.44 | 12.82 |
| 67 | 138.28 | 28.49 | 27.38 | 44.59 | 30.40 | 13.05 |
| 68 | 71.65 | 28.53 | 53.05 | 41.01 | 30.44 | 28.14 |
| 69 | 199.66 | 38.95 | 63.80 | 67.40 | 41.78 | 4.46 |
| 70 | 99.25 | 37.72 | 42.96 | 54.90 | 40.55 | 8.59 |
| 71 | 122.88 | 24.68 | 18.69 | 38.51 | 27.51 | 17.28 |
| 72 | 56.25 | 24.72 | 44.35 | 34.93 | 27.55 | 23.91 |
| 73 | 247.31 | 49.85 | 79.15 | 87.21 | 51.80 | 5.47 |
| 74 | 146.90 | 48.62 | 58.31 | 74.70 | 50.56 | 9.59 |
| 75 | 170.53 | 35.58 | 34.04 | 58.32 | 37.52 | 16.28 |
| 76 | 103.90 | 35.62 | 59.70 | 54.74 | 37.56 | 24.91 |
| 77 | 222.90 | 44.00 | 67.44 | 77.30 | 47.18 | 1.06 |
| 78 | 122.49 | 42.77 | 46.60 | 64.80 | 45.95 | 5.19 |
| 79 | 146.12 | 29.73 | 22.33 | 48.41 | 32.91 | 20.69 |
| 80 | 79.49 | 29.77 | 47.99 | 44.83 | 32.94 | 20.51 |
| 81 | 224.06 | 44.81 | 75.51 | 77.30 | 46.40 | 8.87 |
| 82 | 123.65 | 43.57 | 54.67 | 64.80 | 45.16 | 13.00 |
| 83 | 147.29 | 30.53 | 30.39 | 48.41 | 32.13 | 12.88 |
| 84 | 80.65 | 30.57 | 56.06 | 44.83 | 32.16 | 28.31 |
| 85 | 66.64 | 17.83 | 25.61 | 34.55 | 15.17 | 3.36 |
| 86 | 98.22 | 19.65 | 29.10 | 34.55 | 16.98 | 0.14 |
| 87 | 37.47 | 11.16 | 16.21 | 22.42 | 9.04 | 2.74 |
| 88 | 69.04 | 12.98 | 19.70 | 22.42 | 10.85 | 0.75 |
| 89 | 52.41 | 14.74 | 23.38 | 28.49 | 11.86 | 5.44 |
| 90 | 83.99 | 16.56 | 26.87 | 28.49 | 13.68 | 1.95 |
| 91 | 51.70 | 14.25 | 18.44 | 28.49 | 12.34 | 0.66 |
| 92 | 83.27 | 16.07 | 21.93 | 28.49 | 14.16 | 2.83 |
| 93 | 52.90 | 14.72 | 21.05 | 28.75 | 12.49 | 3.09 |
| 94 | 84.48 | 16.53 | 24.54 | 28.75 | 14.31 | 0.40 |
| 95 | 28.59 | 9.16 | 13.21 | 18.64 | 7.39 | 2.58 |
| 96 | 60.17 | 10.97 | 16.71 | 18.64 | 9.20 | 0.91 |
| 97 | 41.04 | 12.14 | 19.19 | 23.69 | 9.74 | 4.83 |
| 98 | 72.62 | 13.96 | 22.68 | 23.69 | 11.56 | 1.33 |
| 99 | 40.45 | 11.73 | 15.07 | 23.69 | 10.14 | 0.84 |
| 100 | 72.02 | 13.55 | 18.56 | 23.69 | 11.95 | 2.65 |
| 101 | 82.67 | 25.95 | 20.62 | 22.94 | 10.36 | 1.04 |
| 102 | 12.25 | 25.17 | 12.84 | 17.51 | 23.26 | 6.26 |
| 103 | 73.66 | 23.90 | 17.61 | 19.11 | 8.63 | 0.86 |
| 104 | 3.24 | 23.12 | 9.83 | 13.69 | 21.53 | 6.09 |
| 105 | 129.03 | 36.45 | 35.81 | 42.35 | 19.63 | 1.96 |
| 106 | 10.95 | 24.78 | 12.67 | 17.12 | 22.52 | 6.18 |
| 107 | 120.02 | 34.41 | 32.80 | 38.53 | 17.90 | 1.79 |
| 108 | 1.94 | 22.73 | 9.66 | 13.30 | 20.79 | 6.01 |
| 109 | 140.31 | 36.85 | 41.55 | 50.67 | 23.60 | 2.36 |
| 110 | 22.94 | 20.74 | 14.17 | 18.58 | 18.33 | 4.59 |
| 111 | 131.30 | 34.81 | 38.54 | 46.85 | 21.87 | 2.19 |
| 112 | 13.93 | 18.69 | 11.16 | 14.76 | 16.60 | 4.41 |
| 113 | 45.61 | 0.64 | 20.61 | 21.20 | 16.18 | 1.79 |
| 114 | 14.64 | 1.38 | 8.10 | 13.70 | 15.44 | 4.27 |
| 115 | 0.46 | 9.21 | 6.46 | 3.87 | 7.62 | 11.26 |
| 116 | 40.44 | 9.18 | 8.94 | 1.72 | 7.64 | 13.46 |
| 117 | 163.68 | 11.03 | 43.75 | 46.44 | 13.29 | 2.43 |
| 118 | 103.44 | 10.29 | 31.24 | 38.94 | 12.55 | 0.04 |
| 119 | 117.62 | 2.47 | 16.68 | 29.10 | 4.73 | 15.48 |
| 120 | 77.64 | 2.49 | 32.08 | 26.95 | 4.75 | 9.24 |
| 121 | 54.62 | 1.40 | 23.62 | 25.03 | 17.91 | 1.96 |
| 122 | 5.63 | 0.66 | 11.11 | 17.53 | 17.17 | 4.44 |
| 123 | 8.55 | 7.16 | 3.45 | 7.69 | 9.35 | 11.09 |
| 124 | 31.43 | 7.14 | 11.95 | 5.54 | 9.37 | 13.63 |
| 125 | 172.69 | 13.08 | 46.76 | 50.26 | 15.02 | 2.26 |
| 126 | 112.45 | 12.34 | 34.25 | 42.76 | 14.28 | 0.22 |
| 127 | 126.63 | 4.51 | 19.69 | 32.92 | 6.46 | 15.31 |
| 128 | 86.65 | 4.54 | 35.09 | 30.78 | 6.48 | 9.41 |
| 129 | 170.62 | 27.33 | 58.99 | 63.57 | 40.06 | 4.29 |
| 130 | 70.21 | 26.10 | 38.15 | 51.07 | 38.82 | 8.42 |
| 131 | 93.84 | 13.06 | 13.88 | 34.69 | 25.78 | 17.46 |
| 132 | 27.21 | 13.10 | 39.55 | 31.10 | 25.82 | 23.74 |
| 133 | 267.56 | 38.78 | 79.80 | 87.18 | 41.04 | 1.64 |
| 134 | 167.15 | 37.54 | 58.95 | 74.68 | 39.80 | 5.76 |
| 135 | 190.79 | 24.50 | 34.68 | 58.29 | 26.77 | 20.11 |
| 136 | 124.15 | 24.54 | 60.35 | 54.71 | 26.80 | 21.08 |
| 137 | 179.63 | 29.38 | 62.00 | 67.40 | 41.78 | 4.46 |
| 138 | 79.22 | 28.14 | 41.16 | 54.90 | 40.55 | 8.59 |
| 139 | 102.85 | 15.11 | 16.89 | 38.51 | 27.51 | 17.28 |
| 140 | 36.22 | 15.14 | 42.56 | 34.93 | 27.55 | 23.91 |
| 141 | 276.57 | 40.82 | 82.81 | 91.00 | 42.76 | 1.81 |
| 142 | 176.16 | 39.59 | 61.96 | 78.50 | 41.53 | 5.94 |
| 143 | 199.80 | 26.55 | 37.69 | 62.11 | 28.49 | 19.94 |
| 144 | 133.16 | 26.59 | 63.36 | 58.53 | 28.53 | 21.25 |
| 145 | 34.33 | 1.04 | 14.87 | 12.88 | 12.21 | 1.39 |
| 146 | 25.92 | 1.78 | 2.36 | 5.38 | 11.47 | 3.87 |
| 147 | 11.74 | 9.61 | 12.20 | 4.45 | 3.65 | 11.66 |
| 148 | 51.72 | 9.58 | 3.20 | 6.60 | 3.67 | 13.06 |
| 149 | 151.69 | 15.07 | 42.25 | 44.98 | 17.48 | 0.83 |
| 150 | 91.45 | 14.33 | 29.75 | 37.48 | 16.74 | 1.64 |
| 151 | 105.63 | 6.51 | 15.18 | 27.64 | 8.92 | 13.88 |
| 152 | 65.65 | 6.53 | 30.58 | 25.49 | 8.94 | 10.83 |
| 153 | 43.34 | 1.00 | 17.88 | 16.71 | 13.94 | 1.57 |
| 154 | 16.91 | 0.26 | 5.37 | 9.21 | 13.20 | 4.04 |
| 155 | 2.73 | 7.56 | 9.19 | 0.63 | 5.37 | 11.48 |
| 156 | 42.71 | 7.54 | 6.21 | 2.78 | 5.40 | 13.23 |
| 157 | 160.70 | 17.12 | 45.26 | 48.80 | 19.21 | 0.66 |
| 158 | 100.46 | 16.38 | 32.76 | 41.30 | 18.47 | 1.82 |
| 159 | 114.64 | 8.55 | 18.19 | 31.47 | 10.64 | 13.71 |
| 160 | 74.66 | 8.57 | 33.59 | 29.32 | 10.67 | 11.01 |
| 161 | 78.90 | 23.70 | 26.71 | 34.55 | 15.17 | 3.36 |
| 162 | 110.48 | 25.51 | 30.20 | 34.55 | 16.98 | 0.14 |
| 163 | 19.55 | 16.69 | 13.97 | 20.10 | 14.57 | 4.98 |
| 164 | 51.12 | 18.51 | 17.46 | 20.10 | 16.38 | 1.49 |
| 165 | 63.12 | 19.60 | 21.96 | 28.75 | 12.49 | 3.09 |
| 166 | 94.70 | 21.42 | 25.45 | 28.75 | 14.31 | 0.40 |
| 167 | 13.66 | 13.76 | 11.35 | 16.70 | 11.99 | 4.44 |
| 168 | 45.23 | 15.58 | 14.84 | 16.70 | 13.81 | 0.95 |

The moment envelope of the beam

| The lower part of the beam is under tension |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | 276.57 | 153.60 | 72.24 | 35.37 | 44.53 | 43.88 | 50.67 |
| The upper part of the beam is under tension |  |  |  |  |  |  |  |
| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | 140.31 | 70.68 | 41.55 | 58.99 | 79.19 | 86.57 | 91.00 |

The calculated stress ratio after buckling was 0.653

The calculated stress ratio of shear strength =0.377

Maximum stable stress in plane (N/mm*mm) =197.33

The calculated maximum stress ratio in plane stability =0.647

The maximum stress ratio calculated out of plane stability =0.805

After buckling, the calculated stress ratio =0.653 < 1.0

The calculated stress ratio of shear strength =0.377 < 1.0

The maximum stable stress in plane < f=305.00

The maximum stress ratio calculated out of plane stability is < 1.0 0.80

Web height/thickness ratio H0/TW=71.67 < [H0/TW]=250.00 (gb51022-2015)

Flange width ratio B/T =9.70 < [B/T] =12.38

(constant + live) beam deflection mm

| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.00 | 6.14 | 13.24 | 20.49 | 26.96 | 31.69 | 33.75 |

Maximum deflection =33.75 maximum deflection/span =1/533.

Initial value of slope of inclined beam: 1/10.00

Minimum slope value of deformed inclined beam: 1/10.51

Slope change rate of inclined beam after deformation =0.049 < 1/3

Component weight (Kg)=467.20

Steel beam 3     The design results

Section type =16;   Setting Angle =0;   Calculated length: Lx=4.62, Ly=3.00

Component length =2.31;   Calculate length coefficient: Ux=2.00 Uy=1.30

Seismic rating: grade 3

Cross section parameters: B1=200, B2=200, H=200, Tw=6, T1=8, T2=8

Classification of axial compression section: X-axis :b, Y-axis :c

Steel number of component: Q345

Check calculation specification: door gb51022-2015

| \ | I end |  |  | II end |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| combination | M | N | V | M | N | V |
| 1 | 10.33 | 0.89 | 8.94 | 0.00 | 0.00 | 0.00 |
| 2 | 5.64 | 0.49 | 4.88 | 0.00 | 0.00 | 0.00 |
| 3 | 10.33 | 0.89 | 8.94 | 0.00 | 0.00 | 0.00 |
| 4 | 5.64 | 0.49 | 4.88 | 0.00 | 0.00 | 0.00 |
| 5 | 11.71 | 1.01 | 10.13 | 0.00 | 0.00 | 0.00 |
| 6 | 5.01 | 0.43 | 4.34 | 0.00 | 0.00 | 0.00 |
| 7 | 11.71 | 1.01 | 10.13 | 0.00 | 0.00 | 0.00 |
| 8 | 5.01 | 0.43 | 4.34 | 0.00 | 0.00 | 0.00 |
| 9 | 10.88 | 0.94 | 9.41 | 0.00 | 0.00 | 0.00 |
| 10 | 4.18 | 0.36 | 3.61 | 0.00 | 0.00 | 0.00 |
| 11 | 10.88 | 0.94 | 9.41 | 0.00 | 0.00 | 0.00 |
| 12 | 4.18 | 0.36 | 3.61 | 0.00 | 0.00 | 0.00 |
| 13 | 19.67 | 0.43 | 17.02 | 0.00 | 0.00 | 0.00 |
| 14 | 7.33 | 0.43 | 6.34 | 0.00 | 0.00 | 0.00 |
| 15 | 24.61 | 0.43 | 21.29 | 0.00 | 0.00 | 0.00 |
| 16 | 4.86 | 0.43 | 4.21 | 0.00 | 0.00 | 0.00 |
| 17 | 20.51 | 0.36 | 17.74 | 0.00 | 0.00 | 0.00 |
| 18 | 8.16 | 0.36 | 7.06 | 0.00 | 0.00 | 0.00 |
| 19 | 25.44 | 0.36 | 22.01 | 0.00 | 0.00 | 0.00 |
| 20 | 5.70 | 0.36 | 4.93 | 0.00 | 0.00 | 0.00 |
| 21 | 3.10 | 1.01 | 2.68 | 0.00 | 0.00 | 0.00 |
| 22 | 4.31 | 1.01 | 3.73 | 0.00 | 0.00 | 0.00 |
| 23 | 6.06 | 1.01 | 5.24 | 0.00 | 0.00 | 0.00 |
| 24 | 5.79 | 1.01 | 5.01 | 0.00 | 0.00 | 0.00 |
| 25 | 9.80 | 0.43 | 8.48 | 0.00 | 0.00 | 0.00 |
| 26 | 2.39 | 0.43 | 2.07 | 0.00 | 0.00 | 0.00 |
| 27 | 12.76 | 0.43 | 11.04 | 0.00 | 0.00 | 0.00 |
| 28 | 0.91 | 0.43 | 0.79 | 0.00 | 0.00 | 0.00 |
| 29 | 3.10 | 1.01 | 2.68 | 0.00 | 0.00 | 0.00 |
| 30 | 4.31 | 1.01 | 3.73 | 0.00 | 0.00 | 0.00 |
| 31 | 6.06 | 1.01 | 5.24 | 0.00 | 0.00 | 0.00 |
| 32 | 5.79 | 1.01 | 5.01 | 0.00 | 0.00 | 0.00 |
| 33 | 9.80 | 0.43 | 8.48 | 0.00 | 0.00 | 0.00 |
| 34 | 2.39 | 0.43 | 2.07 | 0.00 | 0.00 | 0.00 |
| 35 | 12.76 | 0.43 | 11.04 | 0.00 | 0.00 | 0.00 |
| 36 | 0.91 | 0.43 | 0.79 | 0.00 | 0.00 | 0.00 |
| 37 | 3.93 | 0.94 | 3.40 | 0.00 | 0.00 | 0.00 |
| 38 | 3.47 | 0.94 | 3.00 | 0.00 | 0.00 | 0.00 |
| 39 | 6.90 | 0.94 | 5.97 | 0.00 | 0.00 | 0.00 |
| 40 | 4.95 | 0.94 | 4.28 | 0.00 | 0.00 | 0.00 |
| 41 | 10.63 | 0.36 | 9.20 | 0.00 | 0.00 | 0.00 |
| 42 | 3.23 | 0.36 | 2.79 | 0.00 | 0.00 | 0.00 |
| 43 | 13.59 | 0.36 | 11.76 | 0.00 | 0.00 | 0.00 |
| 44 | 1.75 | 0.36 | 1.51 | 0.00 | 0.00 | 0.00 |
| 45 | 3.93 | 0.94 | 3.40 | 0.00 | 0.00 | 0.00 |
| 46 | 3.47 | 0.94 | 3.00 | 0.00 | 0.00 | 0.00 |
| 47 | 6.90 | 0.94 | 5.97 | 0.00 | 0.00 | 0.00 |
| 48 | 4.95 | 0.94 | 4.28 | 0.00 | 0.00 | 0.00 |
| 49 | 10.63 | 0.36 | 9.20 | 0.00 | 0.00 | 0.00 |
| 50 | 3.23 | 0.36 | 2.79 | 0.00 | 0.00 | 0.00 |
| 51 | 13.59 | 0.36 | 11.76 | 0.00 | 0.00 | 0.00 |
| 52 | 1.75 | 0.36 | 1.51 | 0.00 | 0.00 | 0.00 |
| 53 | 14.98 | 0.84 | 12.96 | 0.00 | 0.00 | 0.00 |
| 54 | 2.64 | 0.84 | 2.28 | 0.00 | 0.00 | 0.00 |
| 55 | 19.92 | 0.84 | 17.23 | 0.00 | 0.00 | 0.00 |
| 56 | 0.17 | 0.84 | 0.15 | 0.00 | 0.00 | 0.00 |
| 57 | 19.67 | 0.43 | 17.02 | 0.00 | 0.00 | 0.00 |
| 58 | 7.33 | 0.43 | 6.34 | 0.00 | 0.00 | 0.00 |
| 59 | 24.61 | 0.43 | 21.29 | 0.00 | 0.00 | 0.00 |
| 60 | 4.86 | 0.43 | 4.21 | 0.00 | 0.00 | 0.00 |
| 61 | 14.98 | 0.84 | 12.96 | 0.00 | 0.00 | 0.00 |
| 62 | 2.64 | 0.84 | 2.28 | 0.00 | 0.00 | 0.00 |
| 63 | 19.92 | 0.84 | 17.23 | 0.00 | 0.00 | 0.00 |
| 64 | 0.17 | 0.84 | 0.15 | 0.00 | 0.00 | 0.00 |
| 65 | 19.67 | 0.43 | 17.02 | 0.00 | 0.00 | 0.00 |
| 66 | 7.33 | 0.43 | 6.34 | 0.00 | 0.00 | 0.00 |
| 67 | 24.61 | 0.43 | 21.29 | 0.00 | 0.00 | 0.00 |
| 68 | 4.86 | 0.43 | 4.21 | 0.00 | 0.00 | 0.00 |
| 69 | 15.82 | 0.77 | 13.69 | 0.00 | 0.00 | 0.00 |
| 70 | 3.48 | 0.77 | 3.01 | 0.00 | 0.00 | 0.00 |
| 71 | 20.75 | 0.77 | 17.96 | 0.00 | 0.00 | 0.00 |
| 72 | 1.01 | 0.77 | 0.87 | 0.00 | 0.00 | 0.00 |
| 73 | 20.51 | 0.36 | 17.74 | 0.00 | 0.00 | 0.00 |
| 74 | 8.16 | 0.36 | 7.06 | 0.00 | 0.00 | 0.00 |
| 75 | 25.44 | 0.36 | 22.01 | 0.00 | 0.00 | 0.00 |
| 76 | 5.70 | 0.36 | 4.93 | 0.00 | 0.00 | 0.00 |
| 77 | 15.82 | 0.77 | 13.69 | 0.00 | 0.00 | 0.00 |
| 78 | 3.48 | 0.77 | 3.01 | 0.00 | 0.00 | 0.00 |
| 79 | 20.75 | 0.77 | 17.96 | 0.00 | 0.00 | 0.00 |
| 80 | 1.01 | 0.77 | 0.87 | 0.00 | 0.00 | 0.00 |
| 81 | 20.51 | 0.36 | 17.74 | 0.00 | 0.00 | 0.00 |
| 82 | 8.16 | 0.36 | 7.06 | 0.00 | 0.00 | 0.00 |
| 83 | 25.44 | 0.36 | 22.01 | 0.00 | 0.00 | 0.00 |
| 84 | 5.70 | 0.36 | 4.93 | 0.00 | 0.00 | 0.00 |
| 85 | 7.58 | 1.99 | 6.69 | 0.00 | 1.30 | 0.13 |
| 86 | 8.19 | 0.62 | 6.95 | 0.00 | 1.30 | 0.13 |
| 87 | 4.71 | 1.74 | 4.21 | 0.00 | 1.30 | 0.13 |
| 88 | 5.31 | 0.87 | 4.47 | 0.00 | 1.30 | 0.13 |
| 89 | 7.58 | 1.99 | 6.69 | 0.00 | 1.30 | 0.13 |
| 90 | 8.19 | 0.62 | 6.95 | 0.00 | 1.30 | 0.13 |
| 91 | 4.71 | 1.74 | 4.21 | 0.00 | 1.30 | 0.13 |
| 92 | 5.31 | 0.87 | 4.47 | 0.00 | 1.30 | 0.13 |
| 93 | 6.27 | 1.87 | 5.55 | 0.00 | 1.30 | 0.13 |
| 94 | 6.87 | 0.74 | 5.82 | 0.00 | 1.30 | 0.13 |
| 95 | 3.88 | 1.67 | 3.48 | 0.00 | 1.30 | 0.13 |
| 96 | 4.48 | 0.94 | 3.75 | 0.00 | 1.30 | 0.13 |
| 97 | 6.27 | 1.87 | 5.55 | 0.00 | 1.30 | 0.13 |
| 98 | 6.87 | 0.74 | 5.82 | 0.00 | 1.30 | 0.13 |
| 99 | 3.88 | 1.67 | 3.48 | 0.00 | 1.30 | 0.13 |
| 100 | 4.48 | 0.94 | 3.75 | 0.00 | 1.30 | 0.13 |
| 101 | 5.01 | 0.43 | 4.34 | 0.00 | 0.00 | 0.00 |
| 102 | 5.01 | 0.43 | 4.34 | 0.00 | 0.00 | 0.00 |
| 103 | 4.18 | 0.36 | 3.61 | 0.00 | 0.00 | 0.00 |
| 104 | 4.18 | 0.36 | 3.61 | 0.00 | 0.00 | 0.00 |
| 105 | 9.70 | 0.84 | 8.39 | 0.00 | 0.00 | 0.00 |
| 106 | 5.01 | 0.43 | 4.34 | 0.00 | 0.00 | 0.00 |
| 107 | 8.87 | 0.77 | 7.67 | 0.00 | 0.00 | 0.00 |
| 108 | 4.18 | 0.36 | 3.61 | 0.00 | 0.00 | 0.00 |
| 109 | 11.71 | 1.01 | 10.13 | 0.00 | 0.00 | 0.00 |
| 110 | 5.01 | 0.43 | 4.34 | 0.00 | 0.00 | 0.00 |
| 111 | 10.88 | 0.94 | 9.41 | 0.00 | 0.00 | 0.00 |
| 112 | 4.18 | 0.36 | 3.61 | 0.00 | 0.00 | 0.00 |
| 113 | 5.11 | 0.84 | 4.42 | 0.00 | 0.00 | 0.00 |
| 114 | 2.30 | 0.84 | 1.99 | 0.00 | 0.00 | 0.00 |
| 115 | 8.07 | 0.84 | 6.98 | 0.00 | 0.00 | 0.00 |
| 116 | 3.78 | 0.84 | 3.27 | 0.00 | 0.00 | 0.00 |
| 117 | 9.80 | 0.43 | 8.48 | 0.00 | 0.00 | 0.00 |
| 118 | 2.39 | 0.43 | 2.07 | 0.00 | 0.00 | 0.00 |
| 119 | 12.76 | 0.43 | 11.04 | 0.00 | 0.00 | 0.00 |
| 120 | 0.91 | 0.43 | 0.79 | 0.00 | 0.00 | 0.00 |
| 121 | 5.94 | 0.77 | 5.14 | 0.00 | 0.00 | 0.00 |
| 122 | 1.46 | 0.77 | 1.26 | 0.00 | 0.00 | 0.00 |
| 123 | 8.91 | 0.77 | 7.71 | 0.00 | 0.00 | 0.00 |
| 124 | 2.94 | 0.77 | 2.55 | 0.00 | 0.00 | 0.00 |
| 125 | 10.63 | 0.36 | 9.20 | 0.00 | 0.00 | 0.00 |
| 126 | 3.23 | 0.36 | 2.79 | 0.00 | 0.00 | 0.00 |
| 127 | 13.59 | 0.36 | 11.76 | 0.00 | 0.00 | 0.00 |
| 128 | 1.75 | 0.36 | 1.51 | 0.00 | 0.00 | 0.00 |
| 129 | 14.98 | 0.84 | 12.96 | 0.00 | 0.00 | 0.00 |
| 130 | 2.64 | 0.84 | 2.28 | 0.00 | 0.00 | 0.00 |
| 131 | 19.92 | 0.84 | 17.23 | 0.00 | 0.00 | 0.00 |
| 132 | 0.17 | 0.84 | 0.15 | 0.00 | 0.00 | 0.00 |
| 133 | 19.67 | 0.43 | 17.02 | 0.00 | 0.00 | 0.00 |
| 134 | 7.33 | 0.43 | 6.34 | 0.00 | 0.00 | 0.00 |
| 135 | 24.61 | 0.43 | 21.29 | 0.00 | 0.00 | 0.00 |
| 136 | 4.86 | 0.43 | 4.21 | 0.00 | 0.00 | 0.00 |
| 137 | 15.82 | 0.77 | 13.69 | 0.00 | 0.00 | 0.00 |
| 138 | 3.48 | 0.77 | 3.01 | 0.00 | 0.00 | 0.00 |
| 139 | 20.75 | 0.77 | 17.96 | 0.00 | 0.00 | 0.00 |
| 140 | 1.01 | 0.77 | 0.87 | 0.00 | 0.00 | 0.00 |
| 141 | 20.51 | 0.36 | 17.74 | 0.00 | 0.00 | 0.00 |
| 142 | 8.16 | 0.36 | 7.06 | 0.00 | 0.00 | 0.00 |
| 143 | 25.44 | 0.36 | 22.01 | 0.00 | 0.00 | 0.00 |
| 144 | 5.70 | 0.36 | 4.93 | 0.00 | 0.00 | 0.00 |
| 145 | 3.10 | 1.01 | 2.68 | 0.00 | 0.00 | 0.00 |
| 146 | 4.31 | 1.01 | 3.73 | 0.00 | 0.00 | 0.00 |
| 147 | 6.06 | 1.01 | 5.24 | 0.00 | 0.00 | 0.00 |
| 148 | 5.79 | 1.01 | 5.01 | 0.00 | 0.00 | 0.00 |
| 149 | 9.80 | 0.43 | 8.48 | 0.00 | 0.00 | 0.00 |
| 150 | 2.39 | 0.43 | 2.07 | 0.00 | 0.00 | 0.00 |
| 151 | 12.76 | 0.43 | 11.04 | 0.00 | 0.00 | 0.00 |
| 152 | 0.91 | 0.43 | 0.79 | 0.00 | 0.00 | 0.00 |
| 153 | 3.93 | 0.94 | 3.40 | 0.00 | 0.00 | 0.00 |
| 154 | 3.47 | 0.94 | 3.00 | 0.00 | 0.00 | 0.00 |
| 155 | 6.90 | 0.94 | 5.97 | 0.00 | 0.00 | 0.00 |
| 156 | 4.95 | 0.94 | 4.28 | 0.00 | 0.00 | 0.00 |
| 157 | 10.63 | 0.36 | 9.20 | 0.00 | 0.00 | 0.00 |
| 158 | 3.23 | 0.36 | 2.79 | 0.00 | 0.00 | 0.00 |
| 159 | 13.59 | 0.36 | 11.76 | 0.00 | 0.00 | 0.00 |
| 160 | 1.75 | 0.36 | 1.51 | 0.00 | 0.00 | 0.00 |
| 161 | 7.58 | 1.99 | 6.69 | 0.00 | 1.30 | 0.13 |
| 162 | 8.19 | 0.62 | 6.95 | 0.00 | 1.30 | 0.13 |
| 163 | 4.71 | 1.74 | 4.21 | 0.00 | 1.30 | 0.13 |
| 164 | 5.31 | 0.87 | 4.47 | 0.00 | 1.30 | 0.13 |
| 165 | 6.27 | 1.87 | 5.55 | 0.00 | 1.30 | 0.13 |
| 166 | 6.87 | 0.74 | 5.82 | 0.00 | 1.30 | 0.13 |
| 167 | 3.88 | 1.67 | 3.48 | 0.00 | 1.30 | 0.13 |
| 168 | 4.48 | 0.94 | 3.75 | 0.00 | 1.30 | 0.13 |

The moment envelope of the beam

| The lower part of the beam is under tension |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | 25.44 | 14.24 | 9.11 | 5.13 | 2.28 | 0.57 | 0.00 |
| The upper part of the beam is under tension |  |  |  |  |  |  |  |
| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | 11.71 | 8.13 | 5.21 | 2.93 | 1.30 | 0.33 | 0.00 |

The stress ratio calculated after buckling was 0.269

The calculated stress ratio of shear strength =0.114

The maximum stable stress in plane (N/mm*mm) =78.35

The calculated maximum stress ratio in plane stability =0.257

The maximum stress ratio calculated out of plane stability =0.287

Stress ratio after buckling strength calculation =0.269 < 1.0

The calculated stress ratio of shear strength =0.114 < 1.0

The maximum stable stress in plane < f=305.00

The maximum stress ratio calculated out of plane stability is < 1.0 0.29

Web height/thickness ratio H0/TW=30.67 < [H0/TW]=250.00 (gb51022-2015)

Flange width ratio B/T =12.12 < [B/T] =12.38

(constant + live) beam deflection mm

| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.00 | 0.37 | 0.59 | 0.73 | 0.82 | 0.88 | 0.94 |

Maximum deflection value =0.00 maximum deflection/span =1/100000.

Initial value of slope of inclined beam: 1/10.00

Minimum slope value of deformed inclined beam: 1/10.11

Slope change rate of inclined beam after deformation =0.011 < 1/3

Component weight (Kg)=78.10

7. Steel beam 4     The design results

Section type =27;   Setting Angle =0;   Calculated length: Lx=18.09, Ly=3.00

Component length =9.04;   Calculate length coefficient: Ux=2.00 Uy=0.33

Seismic rating: grade 3

H: B1=200, B2=200, H1=300, H2=600, T1=6, T2=10, T3=10

Classification of axial compression section: X-axis :b, Y-axis :c

Steel number of component: Q345

Check calculation specification: door gb51022-2015

| \ | I end |  |  | II end |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| combination | M | N | V | M | N | V |
| 1 | 25.41 | 10.91 | 1.09 | 59.52 | 13.41 | 20.15 |
| 2 | 45.22 | 20.92 | 2.09 | 107.17 | 24.31 | 35.51 |
| 3 | 35.31 | 15.53 | 5.50 | 83.93 | 19.26 | 31.86 |
| 4 | 35.31 | 16.31 | 2.31 | 82.76 | 18.46 | 23.80 |
| 5 | 22.38 | 9.30 | 0.93 | 52.21 | 11.71 | 17.83 |
| 6 | 50.67 | 23.60 | 2.36 | 120.28 | 27.28 | 39.76 |
| 7 | 36.52 | 15.89 | 7.22 | 87.08 | 20.07 | 34.55 |
| 8 | 36.52 | 17.01 | 3.93 | 85.41 | 18.92 | 23.03 |
| 9 | 18.55 | 7.57 | 0.76 | 43.20 | 9.66 | 14.82 |
| 10 | 46.85 | 21.87 | 2.19 | 111.27 | 25.23 | 36.75 |
| 11 | 32.70 | 14.17 | 7.05 | 78.07 | 18.02 | 31.54 |
| 12 | 32.70 | 15.28 | 4.11 | 76.40 | 16.87 | 20.02 |
| 13 | 82.99 | 49.38 | 4.65 | 166.53 | 47.47 | 59.83 |
| 14 | 70.49 | 48.99 | 0.36 | 217.82 | 47.08 | 64.11 |
| 15 | 54.10 | 31.09 | 23.14 | 103.76 | 29.18 | 58.05 |
| 16 | 50.52 | 39.28 | 17.23 | 170.11 | 37.37 | 31.56 |
| 17 | 86.81 | 51.11 | 4.83 | 175.54 | 49.52 | 62.84 |
| 18 | 74.31 | 50.72 | 0.54 | 226.82 | 49.13 | 67.12 |
| 19 | 57.92 | 32.81 | 23.32 | 112.77 | 31.22 | 61.06 |
| 20 | 54.34 | 41.01 | 17.05 | 179.12 | 39.42 | 34.57 |
| 21 | 41.18 | 26.55 | 2.48 | 80.14 | 24.14 | 28.91 |
| 22 | 33.68 | 26.31 | 0.09 | 110.92 | 23.90 | 31.48 |
| 23 | 23.85 | 15.57 | 13.58 | 42.48 | 13.16 | 27.84 |
| 24 | 21.70 | 20.49 | 10.64 | 82.29 | 18.08 | 11.95 |
| 25 | 12.88 | 12.24 | 1.05 | 12.07 | 8.57 | 6.97 |
| 26 | 5.38 | 12.01 | 1.52 | 42.84 | 8.33 | 9.55 |
| 27 | 4.45 | 1.27 | 12.15 | 25.59 | 2.41 | 5.91 |
| 28 | 6.60 | 6.18 | 12.07 | 14.22 | 2.51 | 9.99 |
| 29 | 27.03 | 19.95 | 3.81 | 45.27 | 15.78 | 12.18 |
| 30 | 19.53 | 19.72 | 6.38 | 76.05 | 15.54 | 14.75 |
| 31 | 9.70 | 8.97 | 7.28 | 7.61 | 4.80 | 11.11 |
| 32 | 7.55 | 13.89 | 16.94 | 47.42 | 9.71 | 4.78 |
| 33 | 27.03 | 18.84 | 7.35 | 46.94 | 16.93 | 23.70 |
| 34 | 19.53 | 18.60 | 4.77 | 77.71 | 16.69 | 26.28 |
| 35 | 9.70 | 7.86 | 18.44 | 9.28 | 5.95 | 22.64 |
| 36 | 7.55 | 12.78 | 5.78 | 49.09 | 10.87 | 6.74 |
| 37 | 45.00 | 28.27 | 2.66 | 89.15 | 26.18 | 31.92 |
| 38 | 37.50 | 28.04 | 0.08 | 119.93 | 25.95 | 34.49 |
| 39 | 27.67 | 17.29 | 13.75 | 51.49 | 15.20 | 30.85 |
| 40 | 25.52 | 22.21 | 10.47 | 91.30 | 20.12 | 14.96 |
| 41 | 16.71 | 13.97 | 1.23 | 21.08 | 10.61 | 9.98 |
| 42 | 9.21 | 13.74 | 1.35 | 51.85 | 10.38 | 12.56 |
| 43 | 0.63 | 2.99 | 12.32 | 16.58 | 0.37 | 8.92 |
| 44 | 2.78 | 7.91 | 11.90 | 23.23 | 4.55 | 6.98 |
| 45 | 30.85 | 21.68 | 3.64 | 54.28 | 17.82 | 15.19 |
| 46 | 23.35 | 21.44 | 6.21 | 85.06 | 17.58 | 17.76 |
| 47 | 13.52 | 10.70 | 7.46 | 16.62 | 6.84 | 14.12 |
| 48 | 11.37 | 15.62 | 16.76 | 56.43 | 11.76 | 1.77 |
| 49 | 30.85 | 20.56 | 7.52 | 55.95 | 18.97 | 26.71 |
| 50 | 23.35 | 20.33 | 4.95 | 86.72 | 18.74 | 29.29 |
| 51 | 13.52 | 9.59 | 18.61 | 18.29 | 7.99 | 25.65 |
| 52 | 11.37 | 14.50 | 5.61 | 58.10 | 12.91 | 9.75 |
| 53 | 83.38 | 50.12 | 4.73 | 167.82 | 47.86 | 59.99 |
| 54 | 70.88 | 49.73 | 0.44 | 219.11 | 47.47 | 64.28 |
| 55 | 54.49 | 31.83 | 23.22 | 105.06 | 29.57 | 58.21 |
| 56 | 50.91 | 40.02 | 17.15 | 171.41 | 37.76 | 31.72 |
| 57 | 63.57 | 40.11 | 3.73 | 120.17 | 36.97 | 44.64 |
| 58 | 51.07 | 39.72 | 0.56 | 171.46 | 36.57 | 48.93 |
| 59 | 34.69 | 21.82 | 22.22 | 57.41 | 18.67 | 42.86 |
| 60 | 31.10 | 30.01 | 18.15 | 123.76 | 26.87 | 16.37 |
| 61 | 73.48 | 45.51 | 0.32 | 143.41 | 42.01 | 48.28 |
| 62 | 60.98 | 45.12 | 3.97 | 194.70 | 41.62 | 52.57 |
| 63 | 44.59 | 27.21 | 18.81 | 80.65 | 23.71 | 46.50 |
| 64 | 41.01 | 35.41 | 21.56 | 147.00 | 31.91 | 20.01 |
| 65 | 73.48 | 44.73 | 8.13 | 144.58 | 42.82 | 56.35 |
| 66 | 60.98 | 44.34 | 3.84 | 195.87 | 42.43 | 60.64 |
| 67 | 44.59 | 26.43 | 26.62 | 81.81 | 24.52 | 54.57 |
| 68 | 41.01 | 34.63 | 13.75 | 148.16 | 32.72 | 28.08 |
| 69 | 87.21 | 51.85 | 4.90 | 176.83 | 49.91 | 63.00 |
| 70 | 74.70 | 51.46 | 0.61 | 228.12 | 49.52 | 67.29 |
| 71 | 58.32 | 33.55 | 23.39 | 114.07 | 31.61 | 61.22 |
| 72 | 54.74 | 41.75 | 16.98 | 180.42 | 39.81 | 34.73 |
| 73 | 67.40 | 41.84 | 3.90 | 129.18 | 39.01 | 47.65 |
| 74 | 54.90 | 41.45 | 0.39 | 180.47 | 38.62 | 51.94 |
| 75 | 38.51 | 23.54 | 22.39 | 66.42 | 20.71 | 45.87 |
| 76 | 34.93 | 31.74 | 17.98 | 132.77 | 28.91 | 19.38 |
| 77 | 77.30 | 47.24 | 0.49 | 152.42 | 44.06 | 51.29 |
| 78 | 64.80 | 46.84 | 3.79 | 203.71 | 43.67 | 55.58 |
| 79 | 48.41 | 28.94 | 18.99 | 89.66 | 25.76 | 49.51 |
| 80 | 44.83 | 37.13 | 21.38 | 156.01 | 33.96 | 23.02 |
| 81 | 77.30 | 46.45 | 8.30 | 153.59 | 44.86 | 59.36 |
| 82 | 64.80 | 46.06 | 4.01 | 204.88 | 44.47 | 63.65 |
| 83 | 48.41 | 28.16 | 26.79 | 90.82 | 26.57 | 57.58 |
| 84 | 44.83 | 36.35 | 13.58 | 157.17 | 34.76 | 31.09 |
| 85 | 22.42 | 10.85 | 0.75 | 69.04 | 12.98 | 19.70 |
| 86 | 22.42 | 9.04 | 2.74 | 37.47 | 11.16 | 16.21 |
| 87 | 34.55 | 16.98 | 0.14 | 98.22 | 19.65 | 29.10 |
| 88 | 34.55 | 15.17 | 3.36 | 66.64 | 17.83 | 25.61 |
| 89 | 28.49 | 13.68 | 1.95 | 83.99 | 16.56 | 26.87 |
| 90 | 28.49 | 11.86 | 5.44 | 52.41 | 14.74 | 23.38 |
| 91 | 28.49 | 14.16 | 2.83 | 83.27 | 16.07 | 21.93 |
| 92 | 28.49 | 12.34 | 0.66 | 51.70 | 14.25 | 18.44 |
| 93 | 18.64 | 9.20 | 0.91 | 60.17 | 10.97 | 16.71 |
| 94 | 18.64 | 7.39 | 2.58 | 28.59 | 9.16 | 13.21 |
| 95 | 28.75 | 14.31 | 0.40 | 84.48 | 16.53 | 24.54 |
| 96 | 28.75 | 12.49 | 3.09 | 52.90 | 14.72 | 21.05 |
| 97 | 23.69 | 11.56 | 1.33 | 72.62 | 13.96 | 22.68 |
| 98 | 23.69 | 9.74 | 4.83 | 41.04 | 12.14 | 19.19 |
| 99 | 23.69 | 11.95 | 2.65 | 72.02 | 13.55 | 18.56 |
| 100 | 23.69 | 10.14 | 0.84 | 40.45 | 11.73 | 15.07 |
| 101 | 17.51 | 23.26 | 6.26 | 12.25 | 25.17 | 12.84 |
| 102 | 22.94 | 10.36 | 1.04 | 82.67 | 25.95 | 20.62 |
| 103 | 13.69 | 21.53 | 6.09 | 3.24 | 23.12 | 9.83 |
| 104 | 19.11 | 8.63 | 0.86 | 73.66 | 23.90 | 17.61 |
| 105 | 17.12 | 22.52 | 6.18 | 10.95 | 24.78 | 12.67 |
| 106 | 42.35 | 19.63 | 1.96 | 129.03 | 36.45 | 35.81 |
| 107 | 13.30 | 20.79 | 6.01 | 1.94 | 22.73 | 9.66 |
| 108 | 38.53 | 17.90 | 1.79 | 120.02 | 34.41 | 32.80 |
| 109 | 18.58 | 18.33 | 4.59 | 22.94 | 20.74 | 14.17 |
| 110 | 50.67 | 23.60 | 2.36 | 140.31 | 36.85 | 41.55 |
| 111 | 14.76 | 16.60 | 4.41 | 13.93 | 18.69 | 11.16 |
| 112 | 46.85 | 21.87 | 2.19 | 131.30 | 34.81 | 38.54 |
| 113 | 46.44 | 13.33 | 2.77 | 121.40 | 11.07 | 34.06 |
| 114 | 38.94 | 13.09 | 5.34 | 152.17 | 10.83 | 36.63 |
| 115 | 29.10 | 2.35 | 8.32 | 83.74 | 0.09 | 32.99 |
| 116 | 26.95 | 7.27 | 15.90 | 123.55 | 5.01 | 17.10 |
| 117 | 21.20 | 16.22 | 1.45 | 3.32 | 0.61 | 10.92 |
| 118 | 13.70 | 15.98 | 1.12 | 34.10 | 0.84 | 13.49 |
| 119 | 3.87 | 5.24 | 12.54 | 34.34 | 11.59 | 9.85 |
| 120 | 1.72 | 10.16 | 11.68 | 5.47 | 6.67 | 6.04 |
| 121 | 50.26 | 15.05 | 2.60 | 130.41 | 13.11 | 37.07 |
| 122 | 42.76 | 14.82 | 5.17 | 161.18 | 12.88 | 39.64 |
| 123 | 32.92 | 4.08 | 8.50 | 92.75 | 2.13 | 36.00 |
| 124 | 30.78 | 8.99 | 15.73 | 132.56 | 7.05 | 20.11 |
| 125 | 25.03 | 17.94 | 1.62 | 12.33 | 1.43 | 13.93 |
| 126 | 17.53 | 17.71 | 0.95 | 43.11 | 1.20 | 16.50 |
| 127 | 7.69 | 6.96 | 12.72 | 25.33 | 9.54 | 12.86 |
| 128 | 5.54 | 11.88 | 11.50 | 14.48 | 4.63 | 3.03 |
| 129 | 87.18 | 41.09 | 1.07 | 197.09 | 38.83 | 63.65 |
| 130 | 74.68 | 40.70 | 3.22 | 248.38 | 38.44 | 67.93 |
| 131 | 58.29 | 22.80 | 19.56 | 134.32 | 20.54 | 61.87 |
| 132 | 54.71 | 30.99 | 20.81 | 200.67 | 28.73 | 35.38 |
| 133 | 63.57 | 40.11 | 3.73 | 100.14 | 27.39 | 42.84 |
| 134 | 51.07 | 39.72 | 0.56 | 151.43 | 27.00 | 47.13 |
| 135 | 34.69 | 21.82 | 22.22 | 37.38 | 9.09 | 41.07 |
| 136 | 31.10 | 30.01 | 18.15 | 103.73 | 17.29 | 14.58 |
| 137 | 91.00 | 42.82 | 1.24 | 206.10 | 40.88 | 66.66 |
| 138 | 78.50 | 42.43 | 3.04 | 257.39 | 40.49 | 70.94 |
| 139 | 62.11 | 24.52 | 19.73 | 143.33 | 22.58 | 64.88 |
| 140 | 58.53 | 32.72 | 20.63 | 209.68 | 30.78 | 38.39 |
| 141 | 67.40 | 41.84 | 3.90 | 109.15 | 29.43 | 45.85 |
| 142 | 54.90 | 41.45 | 0.39 | 160.44 | 29.04 | 50.14 |
| 143 | 38.51 | 23.54 | 22.39 | 46.39 | 11.14 | 44.08 |
| 144 | 34.93 | 31.74 | 17.98 | 112.74 | 19.33 | 17.59 |
| 145 | 44.98 | 17.52 | 1.17 | 109.41 | 15.11 | 32.56 |
| 146 | 37.48 | 17.28 | 3.75 | 140.18 | 14.87 | 35.14 |
| 147 | 27.64 | 6.54 | 9.92 | 71.75 | 4.13 | 31.50 |
| 148 | 25.49 | 11.46 | 14.30 | 111.56 | 9.04 | 15.60 |
| 149 | 12.88 | 12.24 | 1.05 | 7.96 | 1.01 | 5.18 |
| 150 | 5.38 | 12.01 | 1.52 | 22.81 | 1.24 | 7.75 |
| 151 | 4.45 | 1.27 | 12.15 | 45.62 | 11.99 | 4.11 |
| 152 | 6.60 | 6.18 | 12.07 | 5.81 | 7.07 | 11.78 |
| 153 | 48.80 | 19.24 | 1.00 | 118.42 | 17.15 | 35.57 |
| 154 | 41.30 | 19.01 | 3.57 | 149.19 | 16.91 | 38.15 |
| 155 | 31.47 | 8.26 | 10.09 | 80.76 | 6.17 | 34.51 |
| 156 | 29.32 | 13.18 | 14.13 | 120.57 | 11.09 | 18.61 |
| 157 | 16.71 | 13.97 | 1.23 | 1.05 | 1.04 | 8.19 |
| 158 | 9.21 | 13.74 | 1.35 | 31.82 | 0.80 | 10.76 |
| 159 | 0.63 | 2.99 | 12.32 | 36.61 | 9.94 | 7.12 |
| 160 | 2.78 | 7.91 | 11.90 | 3.20 | 5.03 | 8.77 |
| 161 | 20.10 | 16.38 | 1.49 | 51.12 | 18.51 | 17.46 |
| 162 | 20.10 | 14.57 | 4.98 | 19.55 | 16.69 | 13.97 |
| 163 | 34.55 | 16.98 | 0.14 | 110.48 | 25.51 | 30.20 |
| 164 | 34.55 | 15.17 | 3.36 | 78.90 | 23.70 | 26.71 |
| 165 | 16.70 | 13.81 | 0.95 | 45.23 | 15.58 | 14.84 |
| 166 | 16.70 | 11.99 | 4.44 | 13.66 | 13.76 | 11.35 |
| 167 | 28.75 | 14.31 | 0.40 | 94.70 | 21.42 | 25.45 |
| 168 | 28.75 | 12.49 | 3.09 | 63.12 | 19.60 | 21.96 |

The moment envelope of the beam

| The lower part of the beam is under tension |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | 50.67 | 43.88 | 44.53 | 35.37 | 80.41 | 150.19 | 257.39 |
| The upper part of the beam is under tension |  |  |  |  |  |  |  |
| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | 91.00 | 87.82 | 77.54 | 50.27 | 41.55 | 70.68 | 140.31 |

The stress ratio calculated after buckling was 0.608

The calculated stress ratio of shear strength =0.323

Maximum stable stress in plane (N/mm*mm) =184.24

Calculated maximum stress ratio in plane stability =0.604

The maximum stress ratio calculated out of plane stability =0.760

After buckling, the calculated stress ratio =0.608 < 1.0

The calculated stress ratio of shear strength =0.323 < 1.0

The maximum stable stress in plane < f=305.00

The maximum stress ratio calculated out of plane stability is < 1.0 0.76

Web height/thickness ratio H0/TW=71.67 < [H0/TW]=250.00 (gb51022-2015)

Flange width ratio B/T =9.70 < [B/T] =12.38

(constant + live) beam deflection mm

| Cross section | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 33.75 | 31.69 | 26.96 | 20.49 | 13.24 | 6.14 | 0.00 |

Maximum deflection =33.75 maximum deflection/span =1/533.

Initial value of slope of inclined beam: 1/10.00

Minimum slope value of deformed inclined beam: 1/10.51

Slope change rate of inclined beam after deformation =0.049 < 1/3

Component weight (Kg)=467.20

Maximum horizontal (x-direction) displacement of cylinder roof under wind load:

Node (6), horizontal displacement dx=24.431(mm) =H /307.

The maximum horizontal (x-direction) displacement of the cylinder roof under seismic load:

The horizontal displacement of node (6) dx=6.340(mm) =H /1183.

Maximum deflection of beam (constant + active) :

Beam (2), flexural span ratio = 1/533.

Maximum horizontal displacement of column top under wind load: H/307< allowable value of column top displacement: H/60

Maximum horizontal displacement of column top under earthquake action: H/1183< allowable displacement of column top: H/60

The maximum (constant + active) flexural span ratio of beam: 1/533< the allowable flexural span ratio of beam: 1/180

Total weight (Kg) of all steel columns =857.

Total weight (Kg) of all steel beams =1091.

Weight of steel beam and steel column (Kg)=1948.

PK11 calculation is complete

Schematic diagram of load and calculation results

1. Schematic diagram of load

FIG. 11-1 schematic diagram of dead load

FIG. 11-2 schematic diagram of live load

FIG. 11-3 schematic diagram of left wind 1

FIG. 11-4 schematic diagram of right wind 1

FIG. 11-5 schematic diagram of left wind 2

FIG. 11-6 schematic diagram of right wind 2

FIG. 11-7 schematic diagram of crane load

2. Try to

FIG. 11-8 constant load bending moment diagram

FIG. 11-9 diagram of constant load shear force

FIG. 11-10 constant load axial force diagram

FIG. 11-11 live load bending moment diagram

Fig.11-12 diagram of live load shear force

Figure 11-13 live load axle force diagram

FIG. 11-14 FIG. 1 bending moment of left wind

FIG. 11-15 FIG. 1 bending moment of right wind

FIG. 11-16 FIG. 2 bending moment of left wind

FIG. 11-17 FIG. 2 bending moment of right wind

FIG. 11-18 left seismic bending moment diagram

FIG. 11-19 right seismic bending moment diagram

FIG. 11-20 envelope diagram of bending moment

FIG. 11-21 shear envelope diagram

FIG. 11-22 axial force envelope diagram

3. The displacement map

FIG. 11-23 constant load displacement diagram

Figure 11-24 live load displacement diagram

FIG. 11-25 left wind 1 displacement diagram

FIG. 11-26 displacement diagram of right wind 1

FIG. 11-27 displacement diagram of left wind 2

FIG. 11-28 displacement diagram of right wind 2

Figure 11-29 left seismic displacement diagram

Figure 11-30 right seismic displacement diagram

Figure 11-31 constant addition displacement diagram

FIG. 11-32 diagram of crane brake displacement